3GPP TSG-SA WG1 Meeting #102 	S1-231458

(Revision of S1-231404)

22 – 26 May, 2023, Berlin, Germany

Source:	OPPO

pCR Title:	pCR on removal of comparison operators in the KPI table

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	xuweijie@oppo.com

Abstract: This document provides a Text Proposal on removal of some of the comparision operators in the KPI table in TR22840.

****************Start of Change *************

5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] The 5G system shall be able to support means to discover and locate Ambient-IoT devices in a certain geographical area, e.g. at cell level.

[PR 5.7.6-002] The 5G system shall be able to provide communication with Ambient-IoT devices with the following KPIs :

Table 5.7.6-1:  KPI Requirements for Airport Terminal/Shipping Port Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airport Terminal/ Shipping Port | 1sec | 99% | NA | NA | 256 bits (UL) (Note 1) | FFS | FFS | 1-10km2 (Note 2) | NA | NA | NA | NA | NA |
| Note 1: 128 bits for the Electronic Product Code (EPC) of the tracked object and additional 128 bits assumed for control / other data (e.g., location-related). Note 2: As an example, Newark Airport size ~8km2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: Table format would be updated to align with the KPI format agreed on in the study.

5.8.6	Potential New Requirements needed to support the use case

[PR.5.8.6-001] The 5G system shall be able to assist an Ambient IoT device with discovery and communication with 5GS entities that can provide location related information.

[PR 5.8.6-002] Based on operator policy, the 5G system shall be able to support authorization of UEs communicating with an Ambient IoT device.

[PR.5.8.6-003] The 5G system shall be able to support means to support RAN entities and authorized UEs to communicate with Ambient IoT devices and transfer related information to other 5G system entities (e.g., core network) / servers.

[PR.5.8.6-004] The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device.

NOTE 1:	This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs.

[PR 5.8.6-005] The 5G system shall be able to support a UE to authenticate an Ambient IoT device.

[PR 5.8.6-006] The 5G system shall be able to support Ambient IoT devices with the following KPIs:

Table 5.8.6-1:  KPI Requirements for “Finding Remote Lost Item” Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote lost item finding (Indoor) | 5s | 99% (Note 1 ) | NA | NA | 256 bits (Note 2 ) | <750 devices/100m2 (Note 3) | 10m | NA | NA | NA | NA | NA | NA |
| Remote lost item finding (Outdoor) | 5s | 99% (Note 1) | NA | NA | 256 bits (Note 2) | <750 devices/100m2 (Note 3) | 100m | NA | NA | NA | NA | NA | NA |
| Note 1: Service can be potentially provided by both multiple UEs and RAN entities. Note 2: 64bits corresponds to 20 digits in decimal number. 20 digits is assumed for the length of tag ID. Additional 192bits were assumed for control and other data (e.g., location information, IP address of server). Note 3: 100m2 space can be packed with about 250 large size baggage (with H=0.7m, W=0.5m). Assuming 3 layers of baggage stacked gives = 250x3 = 750 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: Table format would be updated to align with the KPI format agreed on in the study.

5.15.6	Potential New Requirements needed to support the use case

[PR.5.15.6-001] The 5G system shall support communication for Ambient IoT devices.

Editor’s Note:  	It is FFS enhancement of specific 5GS efficiency required to support this use case.

[PR 5.15.6-002] The 5G System shall provide the network connection to address the KPIs for the use of Ambient IoT devices for preventive care in smart laundry, see table 5.15.6-1.

Table 5.15.6-1: Potential key performance requirements for the use of Ambient IoT devices for smart laundry

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for smart laundry | 10 s~15s | NA | NA | <100bit/s | Typically < 100 bytes | 20 / 100m2 | NA | several m2 up to 1000 m2 | NA | NA | NA | NA | NA |

5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-001] The 5G system shall be able to support a mechanism to authenticate and authorize Ambient IoT devices.

[PR 5.16.6-002] The 5G system shall optimize mobility management support for non-stationary Ambient IoT devices that are unable to initiate communication towards the network.

[PR 5.16.6-003] The 5G System shall allow an operator to manage (e.g. provision, authenticate, authorise, etc.) Ambient IoT devices that have limited or no power source.

[PR 5.16.6-004] The 5G System shall be able to provide suitable and secure means to report to an authorized third-party the location of Ambient IoT devices.

[PR 5.16.6-005] The 5G System shall provide the network connection to address the KPIs for the use of Ambient IoT devices for automated supply distribution, see table 5.16.6-1.

Table 5.16.6-1: Potential key performance requirements for the use of Ambient IoT devices for automated supply distribution

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for automated supply distribution | 10 s~15s | NA | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km2 | NA | 600 000 m2 | NA | NA | NA | NA | [3] m (Indoor, 90% confidence level and in horizontal) |

5.18.6	Potential new requirements needed to support the use case

[PR 5.18.6-1] Based on operator policy, the 5G system shall provide means for an authorised third party to poll a group of multiple ambient IoT devices.

[PR 5.18.6.-2] The 5G system shall be able to provide ambient IoT service with following KPIs

Table 5.18.6-1: Potential key performance requirements for the use of Ambient IoT devices for food supply chain

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for food supply chain | 1 minute | NA | NA | <0.12 bit/s (Note 1) | Typically, < 100 bits (Note 2) | 1.5 Million devices/ km2 (Note 3) | NA | 30,000m2 | NA | NA | NA | NA | NA |
| NOTE 1: Based on sending 1 message of 100 bits once in 15 minutes NOTE 2: If more sensors are used, like humidity or ethylene level, then longer message is required. NOTE 3: This is the highest density inside the distribution center and is based on 50,000 RTIs inside a 30,000m2 distribution center facility. See Figure 5.18.1-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.19.6	Potential New Requirements needed to support the use case

[PR 5.19.6-001] The 5G system shall support Fault-Tolerant Reliable Ambient IoT communication.

Editor’s Note: Requirement PR5.2.6-001 is for FFS.

[PR 5.19.6-001] The 5G system shall support on-demand access to/from Ambient IoT device.

[PR 5.19.6-002] The 5G system shall meet the following KPI table:

Table 5.19.6-1: Potential key performance requirements for Forest Fire Monitoring using Ambient IoT devices

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Forest Fire Monitor | 10sec | NA | 99.9% with fault tolerance. | NA | NA | FFS | [150-FFS] meters | FFS | NA | FFS | NA | NA | NA |

Editor’s Note: Values for Device Density, Service Area and Transfer Interval are FFS.

5.20.6	Potential New Requirements needed to support the use case

[PR.5.20.6-001] The 5G system shall support communication with Ambient IoT device with 3rd party application server.

[PR.5.20.6-002] The 5G system shall be able to authenticate an Ambient IoT device.

[PR.5.20.6-003] The 5G system shall be able to provide Ambient IoT service with following KPIs:

Table 5.20.6-1: Ambient IoT KPI for Smart Agriculture

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart Agriculture | 1 s~5s | 99.9% | NA | <1 kbit/s | <1000 bits | 1 per m2 | 30-100m | 500-70000 m2 per greenhouse | static | 1 hour | NA | NA | NA |

NOTE: There is no requirement for positioning of the Ambient IoT devices for this use case.

5.22.6	Potential New Requirements needed to support the use case

[PR 5.22.6-1] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the device communication power consumption) for Ambient IoT devices, while meeting the communication performance requirements.

[PR 5.22.6-2] The 5G system shall provide a mechanism for a 3rd party application to write user data to and to read user data from an Ambient IoT device.

[PR 5.22.6-3] The 5G system shall be able to collect charging information for a large group of closely located Ambient IoT devices  in an efficient way.

NOTE: for example, the efficiency could be reduced total number of charging data related to a group of Ambient IoT devices, the reduction is compared with already specified 3GPP technologies.

[PR 5.22.6-4] The 5G system shall provide the network connection with the following KPIs for the use of Ambient IoT devices for smart dairy farms, see table 5.22.6-1.

Table 5.22.6-1: Potential key performance requirements for the use of Ambient IoT devices for smart grazing dairy farming

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart dairy farm | 1 s~2s (note 1) | NA | NA | <500 bit/s | Typically, [< 100 bytes] (note 2) | <5200 devices / km2 (note 5) | FFS | 430000 m2 (note 6) | NA | 15 min (note 4) | NA | NA | NA |
| NOTE 1: Latency is not critical. NOTE 2: Electronic Product Code standard [5], this size is the payload size. NOTE 3: The livestock health management application monitors dairy cow body temperature many times daily, typically two consecutive transfers of the application data have an interval of 15 minutes. NOTE 4: Calculated from 80 dairy cows assuming regular values for parameters (e.g. daily intake, pre-grazing yield) previously mentioned, the paddock size comes to 1.54 hectares [47] (about 124m by 124m). NOTE 5: For a relatively large-sized industrialized smart dairy farm, the surface area of pasture for grazing is typically 430000 m2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.23.6	Potential New Requirements needed to support the use case

[PR.5.23.6-1] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the device communication power consumption) for Ambient IoT devices, while meeting the communication performance requirements.

[PR 5.23.6-2] The 5G system shall provide a mechanism for a 3rd party application to write user data to and to read user data from an Ambient IoT device.

[PR 5.23.6-3] The 5G system shall be able to collect charging information for a large group of closely located Ambient IoT devices  in an efficient way.

NOTE: for example, the efficiency could be reduced total number of charging data related to a group of Ambient IoT devices, the reduction is compared with already specified 3GPP technologies

[PR.5.23.6-4] The 5G system shall provide the network connection to address the following KPIs for the use of Ambient IoT devices on smart pig farms.

Table 5.23.6-1: Potential key performance requirements for the use of Ambient IoT devices industrialized smart pig farming

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart livestock farming (pig barns) | 10 s~15s (Note 1) | NA | NA | <500 bit/s | Typically, [< 100 bytes] (note 2) | 850 000 devices / km2 (note 4) | 250 m Indoor | 6000 m2 (note 5) | NA | 15 minutes to half an hour (note 3) | NA | NA | NA |
| NOTE 1: Latency is not critical for this use case. NOTE 2: Electronic Product Code standard [5], this size is the payload size. NOTE 3: The livestock health management application monitors pigs’ temperature on a half-hourly basis, sometimes even down to once per 15 minutes [53]. NOTE 4: Stocking density of 1.2 m2/pig is considered high and 2.4 m2/pig considered low [54]. In [55] the stockinggrazing density range from 0.82 m2/pig and 2.46 m2/pig is studied and 1.23 m2/pig proofs suitable stocking density for growing pigs. The device density of 850000 is from a real farm and translates to stocking density of 1.17 m2/pig. NOTE 5: For a relatively large-sized industrialized smart pig farm, the surface area of a barn is typically 6000 m2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.4	Potential Key Performance Requirements

[PR 6.1-001] The 5G system shall be able to provide Ambient IoT service with the following KPIs:

Table 6.1.4-1: KPIs for Flower Auction scenario

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Container logistics in a flower auction | [10 s] | [99,99%] (note 1) | NA | [5] kbit/s (note 3) | [96] bits (note 2) | [< 1,3] Million/km2 (note 4) | [35] m Indoors | [1 700 000] m2 (note 5) | NA | NA | NA | NA | NA |
| NOTE 1: Chance of communication service unavailability needs to be significantly lower than chance of errors in logistics handling NOTE 2: Only an identifier for the tag is sent (Electronic Product Code (EPC) lengths used for identification is 96 bits). NOTE 3: This value is calculated as the instant data rate for transmitting 96 bits within 100 ms time period. The need for data transmission is infrequent. NOTE 4: Based on closely packing containers. NOTE 5: Size of the flower auction location in Aalsmeer. |  |  |  |  |  |  |  |  |  |  |  |  |  |

*******************End of Change*****************