3GPP TSG SA WG 1 Meeting #102 	S1-231492

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231419, S1-231077)

Source:	Deutsche Telekom AG, Nokia, Ericsson

pCR Title:	Pseudo-CR on Consolidated KPIs of FS_Sensing

Draft Spec:	3GPP TR 23.837 v.1.0.0

Agenda item:	7.1

Document for:	Approval

Contact:	Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: Based on DP S1-231075, this pCR proposes to agree consolidated KPIs section of the FS_Sensing study.

1. Introduction

DP S1-231075 proposes a way to consolidate the KPIs available in FS_Sensing study. Based on it section 7.2 needs to be updated.

2. Reason for Change

Part of the study work is to consolidate the proposed KPIs. Such consolidation is needed to clarify further the main aspects of the needed 5G wireless sensing service.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.827 v.1.0.0.

* * * First Change * * * *

## 7.2	Consolidated potential KPIs of sensing results

Table 7.2-1	Performance requirements of sensing results for integrated sensing and communication scenarios

| Scenario | Sensing service level | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 (use cases 5.1; 5.13 – level1) | Object to be detected indoor: Human, object to be detected outdoor: UAV | 95 | ≤10 | ≤10 | N/A | N/A | 10 NOTE 2 | 10 NOTE 3 | <1000 | < 1 | < 5 | < 2 | intruder detection in smart home, UAV intrusion detection |
|  | 2 (use cases 5.13 – level2, 5.6) | Object to be detected outdoor: Human, UAV | 95 | ≤5 | ≤5 | N/A | N/A | 10 NOTE 2 | 10 NOTE 3 | [≤1000] | [≤1] | ≤5 | ≤5 | UAV flight route intrusion detection, intruder detection in surroundings of smart home |
|  | 3 (use cases 5.2, 5.7, 5.10, 5.11, 5.12, 5.23) | Factory (100m2), crossroad, highway, railway NOTE 4 Object to be detected: Animal, Human, UAV, Vehicle | 95 | ≤1 | N/A | 1 NOTE 5 | N/A | <1 NOTE 5 NOTE 8 | 1m/s x 1m/s NOTE 9 | ≤100 ≤1000 NOTE 6 NOTE 10 | ≥0.05 ÷ ≤ 0.1 ≤ 1 NOTE 11 | ≤2 | ≤2 | pedestrian/animal intrusion detection on a highway/railway, sensing at crossroads with/without obstacle, UAV flight trajectory tracing UAV collision avoidance, AMR collision avoidance in smart factories |
|  | 4 (use cases 5.20, 5.22, 5.25, 5.27, 5.32) | Public area safety, factory NOTE 7 Object to be detected: Animal, Human, UAV, AGV, Vehicle | 95 | 0.5 | 0.5 | 0.1 Pedestrian: ≤1.5 Vehicle: ≤15 | N/A Pedestrian: ≤1.5 | 0.5m | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 ≤ 1 | 1 | 5 | Parking Space Determination, UAVs/vehicles/pedestrians detection near Smart Grid equipment, immersive experience based on sensing, public safety search and rescue or apprehend, integrated sensing and positioning in factory hall |
|  | 5 (use cases 5.28) | ADAS -TBD Object to be detected: Vehicle | [95] | ~ [0.1]-[1.3] ; for short range radar ~[0.02]-[2.6] | TBD | [±0.03] -[±0.12] m/s | N/A | [0.4] | [0.1] – [0.6] | [50] | [0.05] -[0.2] | [1-10] % | [<1] % | ADAS |
| Environment monitoring | 1 (use case 5.14) | Object to be detected outdoor: Human | 95 | [≤2] | N/A | N/A | N/A | [1] | [1] | 1000 NOTE 10 | [≤0.2] | 5 | 5 | traffic management |
|  | 2 (use cases 5.3 and 5.5.) | Rainfall monitoring and flooding NOTE 14 Object to be detected: Rain | 95 | ≤10 | [≤0.2] NOTE 15 | N/A | N/A | N/A | N/A | 1 min | 1<10min, application configurable | < 0.1 | < 3 | rainfall monitoring, flooding monitoring |
| Motion monitoring | 1 (use cases 5.15, 5.24, 5.29) | Indoor human motion -sleep monitoring NOTE 12, sports monitoring NOTE 13, gesture recognition Motion on human, gesture | 95 | N/A | N/A | 0.1 valid for gesture recognition | 0.1 valid for gesture recognition | N/A | N/A | 60s | 60, ≤0.1 for gesture recognition | 5 | 5 | sleep monitoring, sports monitoring, gesture recognition |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: To detect the UAV existance (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. NOTE 4: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. NOTE 5: The KPI values for UAVs are sourced from [25] and [40] and for factories are sourced from [47]. NOTE 6: The value 100 ms is sourced from [28] and is valid for sensing at crossroads. NOTE 7: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. The size of the park of Smart Grid depends on the real environment. NOTE 8: To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 9: To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 10: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1s. NOTE 11: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 12: Additional KPI on motion rate accuracy of 2 times/min (0.033 Hz). NOTE 13: Additional KPI on motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) NOTE 14: Rainfall estimation accuracy is1 mm/h[39] and describes the closeness of the measured rainfall estimation to its true rainfall value. NOTE 15: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note:  The values  in brackets are FFS