3GPP TSG SA WG 1 Meeting #102 	S1-231526

Berlin, Germany, 22-26 May 2023	(revision of S1-231507)

(revision of S1-231048)

Source:	China Unicom, Charter Communications, ZTE Corporation and CATT

pCR Title:	Updated Consolidation on general and charging reqs

Draft Spec:	3GPP TR 22.851 V1.1.0

Agenda item:	7.4

Document for:	Approval

Contact:	Qun wei

Weiqun5@chinaunicom.cn

Abstract: Consolidation of requirements on general and charging aspects from the use cases on Network Sharing.

1. Introduction

New definitions were introduced and requirement categories was discussed since last meeting. The consolidation of general aspects of this pCR includes the following parts:

- General aspects // basic capability

[PR 5.1.6-001], [PR 5.1.6-002]

-General aspects // Service accessibility

[PR 5.2.6-004], [PR 5.5.6-001], [PR 5.7.6-001]

-Charging // or target for 22.261 Chapter 9

[PR 5.2.6-001]

2. Reason for Change

To consolidate requirements. The large consolidation document was discussed at the last meeting, which has been divided into parts to make the entire process more effective.

3. Conclusions

None.

4. Proposal

It is proposed to approve the following changes to 3GPP TR 22.851 V1.1.0.

* * * Copy of 22.851 v110 for reference * * * *

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Indirect Network Sharing: describes the communication between the shared access NG-RAN and the Participating NG-RAN Operator’s core network being routed through the Hosting NG-RAN Operator’s core network.

Hosted Service: a service containing the operator's own application(s) and/or trusted third-party application(s) in the Service Hosting Environment, which can be accessed by the user.

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

Hosting NG-RAN Operator: the operator that has operational control of a shared NG-RAN.

NOTE 1: Hosting NG-RAN Operator can also be a Hosting RAN Operator. See 3GPP TS 22.101 [2].

Participating NG-RAN Operator: authorized operator that is sharing NG-RAN resources provided by a Hosting NG-RAN Operator.

NOTE 2: Participating NG-RAN Operator can also be participating operator. See 3GPP TS 22.101 [2].

Shared NG-RAN: NG-RAN that is shared among a number of operators.

* * * First Change * * * *

7.2	Consolidated Potential Requirements

7.2.1	Introduction

The following consolidated requirements are specified. Different aspects related network sharing with indirect connection between the hosting and the participating operators are specially taken into account. See Tables below.

7.2.2	General aspects

Table 7.2.2-1 General requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2.2-001 | The 5G system shall be able to support Indirect Network Sharing between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks, by means of the connection being routed through the Hosting NG-RAN Operator’s core network. | [PR 5.1.6-001] | Align with the definitions. |
| CPR 7.2.2-002 | In case of Indirect Network Sharing, the 5G system shall be able to support means for a Participating NG-RAN Operator to provide its information on operator/network name to UEs, e.g., for display to the user. | [PR 5.1.6-002] | Align with the definitions. Updated wording |
| CPR 7.2.2-003 | In case of Indirect Network Sharing, UEs shall be able to access their subscribed PLMN services when accessing a Shared NG-RAN, where the subscribed PLMN is one of the Participating NG-RAN Operators. | [PR 5.2.6-004] | Align with the definitions. Updated wording |
| CPR 7.2.2-004 | Subject to the agreement between hosting operators and the participating operator, the 5G system shall support a mechanism to enable a UE to obtain its subscribed services, including Hosted Services, of participating operator via a Shared NG-RAN using Indirect Network Sharing. NOTE: the requirement assumes no impact to UE. | [PR 5.5.6-001] [PR 5.7.6-001] | Align with the definitions. PRs merged. |

Editor’s Note: Consolidated requirements for further aspects are FFS.

* * * Second Change * * * *

7.2.X	Charging aspects

Table 7.2.X-1 Charging requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.2.X-001 | The 5G core network shall be able to support collection of charging information associated with a UE accessing a Shared NG-RAN using Indirect Network Sharing. | [PR 5.2.6-001] The 5G core network shall be able to support collection of charging information associated with a UE accessing a shared network using indirect network sharing. | Align with the definitions. shared network ->Shared NG-RAN |

* * * End of Changes * * * *