3GPP TSG SA WG 1 Meeting #102 	S1-231569

Berlin, Germany, 22 - 26 May 2023

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom, CATT

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.865 – for discussion

Draft Spec:	3GPP TR 22.865 1.0.0

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in S1-231086.

1. Introduction

The “Consolidated Requirements” of TR 22.865 are currently not included in version 1.0.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in  S1-231382.

The document  was updated into S1-231383 to merge the documents S1-231072 & S1-231224.

Then, S1-231383 was revised and split into 3 documents:

- S1-231568 – potential CPR agreeable at this meeting

- S1-231569 - potential CPR for further discussion at this meeting (present document)

- S1-231570 - potential CPR for next meeting

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v1.0.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # |  | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- | --- |
| CPR 6.2-7 | A 5G system with satellite access shall be able to inform a trusted application server or 3rd party application whether Store & Forward Satellite operation is applied for communication with a UE. |  | [PR 5.2.6-001] | x |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-6 | Subject to the operator’s policy, a 5G system shall be able to support two UEs using different satellite access paths to communicate with each other without going through the ground network. (e.g. one UE using a direct satellite connection path, and one UE using a connection via a terrestrial relay with satellite access) | [PR 5.10.6-002] | x |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.5	General and other aspects for satellite access

The potential requirements corresponding to the support of enhancements of general and other aspects of satellite access are listed in the table below

Table 6.5-1 – Consolidated Requirements for general and other aspects of satellite access

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-2 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F operation when satellite connectivity is intermittently/temporarily unavailable. NOTE: The authorizing entity could be an operator, a user/subscriber (UE owner) or a third party (e.g., a business customer having service level agreement with the operator and interacting with the 5G network via an application server). | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] | X |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the communication between UEs using satellite access without going through the ground network, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a terrestrial relay. | [PR 5.6.6-001] [PR 5.10.6-001] [PR 5.1.6-006] [PR 5.2.6-005] | X |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, without the user traffic going through the ground network. | [PR 5.6.6-002] [PR 5.7.6-002] | X |