3GPP TSG SA WG 1 Meeting #102 	S1-231620

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231189, S1-231600)

Source:	Qualcomm

pCR Title:	Pseudo-CR on TR updates and clean-up

Draft Spec:	3GPP TR 22.841

Agenda item:	7.9

Document for:	Approval

Contact:	Francesco Pica
fpica <at> qti.qualcomm.com

Abstract: This document includes a pCR proposal, toward TR 22.841, to make small updates to the overview section and other clean-up.

* * * First Change * * * *

# 4	Overview

This TR captures a set of use cases and potential service requirements related to 5G system support of traffic steering, splitting and switching of UE’s user data (pertaining to the same data session), across two 3GPP access networks. Different scenarios are covered: same PLMN, two PLMNs, or between a PLMN and an NPN, solely considering a single PLMN subscription.

The following figure shows two general examples: (a) single PLMN using two terrestrial RAN; (b) traffic across two PLMNs, using terrestrial and satellite access RAN, respectively.

Figure 4.1: Examples of Intra-PLMN and Inter-PLMN scenarios

Use cases cover various example combinations of 3GPP access networks using same or different RAT, including terrestrial NR plus NR or NR plus E-UTRA (e.g. using a combined EPC and 5GC), mix of terrestrial plus satellite NR, as well as dual NR satellite access (e.g. using same or different NTN orbits, e.g., GEO/MEO/LEO).

There are also use cases covering connectivity across PLMN and NPN, with one or multiple UEs accessing local NPN services. In case of inter-PLMN/NPN scenarios, a proper business agreement is assumed to be in place between the two network operators (no impact on normal inter-PLMN roaming), and UE’s user data transferred over the two networks is anchored in the HPLMN core network. There are also use cases covering connectivity across PLMN and NPN, with one or multiple UEs accessing local NPN services.

Traffic configuration policies, assumed to be under HPLMN control can consider different criteria, rules or conditions/restrictions.

Regarding potential new requirements, one general consideration is that they assume 5GS functionalities that do not introduce RAN impacts.

* * * Next Change * * * *

### 5.2.5	Existing features partly or fully covering the use case functionality

None identified.

* * * End of Changes * * * *