3GPP TSG SA WG 1 Meeting #102 	S1-231704

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231569)

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom, CATT

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.865 – for discussion

Draft Spec:	3GPP TR 22.865 1.0.0

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in S1-231086.

1. Introduction

The “Consolidated Requirements” of TR 22.865 are currently not included in version 1.0.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in  S1-231382.

The document  was updated into S1-231383 to merge the documents S1-231072 & S1-231224.

Then, S1-231383 was revised and split into 3 documents:

- S1-231568 – potential CPR agreeable at this meeting

- S1-231569 - potential CPR for further discussion at this meeting ( which was revised to 1704 the present document)

- S1-231570 - potential CPR for next meeting

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v1.0.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # |  | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- | --- |
| CPR 6.2-7 | A 5G system with satellite access shall be able to inform an authorised 3rd party whether Store & Forward Satellite operation is applied for communication with a UE and to provide information associated (e.g. estimated delivery time to the UE). |  | [PR 5.2.6-001] [PR 5.2.6-005] | To be aligned (merge) with the new PR New PR coming from S1-231579 [PR 5.2.6-005] The 5G system with satellite access shall be able to provide to a trusted third party application the information about the store and forward operation applied to a UE (e.g. estimated delivery time to the UE). Missing similar mention of the estimated delivery time for MO use case (in CPR 6.2-6 – to check CPR 6.2-2 too) |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-6 | Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support service continuity for a remote UE when the UE communication path moves between a direct network connection via 5G terrestrial access network and an indirect network connection via a relay UE (using satellite access). | [PR 5.10.6-002] | Related revised PR still under discussion in S1-231724 |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.5	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below

Table 6.5-1 – Consolidated Requirements for other aspects of satellite access

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-2 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F operation. | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] |  |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the communication between UEs using satellite access without going through the ground network, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a terrestrial relay. | [PR 5.6.6-001] [PR 5.10.6-001] [PR 5.1.6-006] [PR 5.2.6-005] | X [PR 5.10.6-001] Subject to regulatory requirements, the 5G system with satellite access shall support mechanisms to authorize a remote UE to use UE-Satellite-UE communication via a relay UE (using satellite access). |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, without the user traffic going through the ground network. | [PR 5.6.6-002] [PR 5.7.6-002] | X |