3GPP TSG-SA1 Meeting #103	S1-232117

Goteborg, Sweden, 21 – 25 Aug., 2023	  (Revision of S1-231456)

Title:	Ambient IoT functional CPR

Agenda Item:	7.2

Source:	OPPO (Rapporteur)

Contact:	Yang Xu 	xuyang@oppo.com

Weijie Xu xuweijie@oppo.com

Abstract:  This is the CPRs in S1-231456 which were converged in SA1#102, except the text in yellow highlight for further clarification. It is proposed to bring it back and get approved.

****************Start of Change***************

7	Consolidated potential requirements and KPIs

7.1	Consolidated potential requirements

Mapping table is used per each subclause for consolidated potential requirements.

7.1.1	Communication aspects of Ambient IoT devices

Table 7.1.1 – [Communication aspects of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-1 | The 5G system shall be able to support mechanisms to communicate with an ambient IoT device. The communication shall be able to support direct communication mode, indirect communication mode and device to device mode. Note 1: for example, the communication between 5G network/UE and Ambient IoT device can include periodic sensor reporting or network-initiated inventory. Note 2: This requirement applies to 5G base station/UE with the capability to communicate with an Ambient IoT device. | PR 5.2.6-001 PR.5.6.6-001 PR 5.11.6-001 PR.5.12.6-002 PR.5.13.6-001 PR.5.1.6-001 PR.5.4.6-001 PR.5.2.6-002 PR.5.12.6-002 PR.5.8.6-003 PR.5.27-004 PR.5.13.6-004 PR.5.3.6-001 PR 5.22.6-1, PR5.23.6-1, PR.5.24.6-1 PR.5.26.6-001 | New wordingl: - adding communication mode - adding note 2 to address “capability” |
| CPR 7.1.1-2 | The 5G system shall be able to support UE or 5G network to communicate with a group of Ambient IoT devices simultaneously. |  |  |
| CPR 7.1.1-3 | The 5G system shall support energy efficient communication for an Ambient IoT device while meeting the communication performance requirements. | PR 5.3.6-001, PR 5.13.6-004, PR 5.22.6-001, |  |
| CPR 7.1.1-4 | The 5G network shall support a mechanism to authorize a UE to communicate with an Ambient IoT device. | PR 5.8.6-002 PR.5.12.6-001 PR.5.14.6-001 PR.5.21.6-003 PR.5.21.6-001 |  |

7.1.2	Positioning/location of Ambient IoT devices

Table 7.1.2 – [Positioning/location of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall support positioning mechanisms for Ambient IoT devices (e.g locating Ambient IoT device using absolute or relative positioning methods. NOTE: the relative is to allow 5G system to measure the distance between a specific AIoT device and a UE/Base station. | PR 5.9.6-002 PR 5.12.6-004 PR 5.2.6-004 | New wording: merge CPR 7.1.2-a1/b1/2 by adding “e,g,” and a note for clarification. Daniel: strike out “or locating a UE using Ambient IoT devices” |

7.1.3	Management of Ambient IoT devices

Table 7.1.3  [Management of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-1 | The 5G system shall support a mechanism to: - disable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently able to transmit RF signals - enable the capability to transmit RF signals for one or more Ambient IoT device that is / are currently not able to transmit RF signals | P.R.5.1.6-005 PR 5.16.6-003 PR 5.17.6-001 |  |

7.1.4	Collected information and network capability exposure

Table 7.1.4  [Collected information and network capability exposure] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.4-1 | Subject to user consent, operator policy and 3rd party request, the 5G system shall be able to get information from Ambient IoT devices (e.g. sensor data) and provide it to a trusted 3rd party. | PR.5.1.6-002 PR.5.12.6-003 PR 5.13.6-005 PR.5.14.6-003 PR 5.16.6-004 PR 5.18.6-1 PR.5.21.6-004 PR 5.2.6-003 |  |
| CPR 7.1.4-2 | Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices to the trusted 3rd party. Note: A third party can request information on e.g. the Ambient IoT device identity and position. |  |  |
| CPR 7.1.4-5 | The 5G system shall enable an authorized 3rd party to instruct the 5G network to trigger a group of Ambient IoT devices in an specific area and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value). | PR 5.3.6-003 PR 5.19.6-001 PR.5.27-003 |  |

7.1.5	Charging

Table 7.1.5  [Charging] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

7.1.6	Security and privacy

Table 7.1.6  [Security and privacy] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.6-2 | The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device. | P.R.5.1.6-003 PR 5.3.6-002 PR.5.6-002 P.R.5.13.6-002 PR.5.21.6-005 PR.5.8.6-004 P.R.5.1.6-004 PR 5.8.6-005 PR. 5.12.6-006 P.R.5.13.6-003 PR. 5.14.6-005 PR 5.16.6-001 PR.5.20.6-002 |  |
| CPR 7.1.6-4 | The 5G network shall provide a suitable means for the operator to authorize communication between a UE and an Ambient IoT device. | PR. 5.12.6-005 PR. 5.14.6-004 |  |

*******************End of Change*****************