3GPP TSG-SA1 Meeting #104	S1-23xxxx

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 00 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy Efficiency as a Service Criteria requirements update with newly agreed CPRs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergyServ |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 6.15a.2.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.15a	Energy Efficiency as a Service Criteria

6.15a.1	Description

Climate change and the rising consumption of energy motivate increased energy efficiency. Energy efficiency is a strategic priority for telecom operators around the world.

Energy efficiency as a service criteria allows services to be delivered with diverse energy efficiency and energy consumption policies. Energy consumption and efficiency information and network energy states can be exposed to third parties and energy consumption can be constrained.

6.15a.2	Energy related information as a service criteria

6.15a.2.1	Description

Energy consumption can be monitored and considered through O&M as part of network operations [47], as well as a service criteria.

For best-effort traffic, that is, without QoS criteria, policies can be defined to limit energy use for services. This is not in conflict with the principle that performance policies will not be traded off for energy efficiency, since best-effort service has no performance guarantees.

Specifically, best-effort traffic can be subject to a policy that limits the maximum energy consumption over time, or further constrained by location (so that the energy consumption limit only applies when used in a specified service area.)

Additionally, policies can be defined with a maximum energy credit limit, e.g. for best-effort services to limit the total amount of energy consumption. These policies expand the options of subscription policies to control energy consumption in the 5G system.

6.15a.2.2	Requirements

Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a means to associate energy consumption with charging information based on subscription policies for services without QoS criteria.

Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services without QoS criteria.

NOTE 1: The result of the credit control is not specified by this requirement.

NOTE 2:	Credit control [49] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed.

Subject to operator’s policy, the 5G system shall support a means to define subscription policies and means to enforce the policy that define a maximum energy consumption (i.e. quantity of energy for a specified period of time) for services without QoS criteria.

NOTE 3:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

Subject to user consent and operator policy, 5G system shall be able to provide means to adapt a communication service based on energy related information.

Subject to user consent and operator policy, the 5G system shall be able to provide means to adapt a communication service based on subscription policies containing energy consumption information.

NOTE 3x: Energy consumption information can include ratio of renewable energy and carbon emission information when available. Calculation of ratio of renewable energy as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirements do not imply that some form of 'real time' monitoring is required.