3GPP TSG SA WG1 Meeting #104	S1-23xxxx

# Chicago, USA, 13-17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.837 | CR |  | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update sensing consolidated KPI table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the latest TR22.837, the consolidated KPI values still need to further clean up based on the use cases which have been grouped into different KPI categoriaes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | it is suggested to revise the consolidated KPI tables: - Move the public safety related KPIs into cat.4 - Align the KPI values with the use cases in the TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The KPI tables are not correct and inconsistency |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.2	Consolidated potential KPIs of sensing results

| Scenario | Sensing service category | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 (use cases 5.1; 5.13 – level1) | Object to be detected indoor: Human, object to be detected outdoor: UAV | 95 | 10 | 10 | N/A | N/A | 10 NOTE 2 | 5 NOTE 3 | 1000 | 1 | 5 | 2 | intruder detection in smart home, UAV intrusion detection |
|  | 2 (use cases 5.13 – level2, 5.6, 5.14) | Object to be detected outdoor: Human, UAV | 95 | 2 | 5 | 1 | N/A | 1 NOTE 2 | 1 NOTE 3 | Tourist spot monitoring : 5000; Others: 1000 | 0.2 | 0.1 | 5 | UAV flight route intrusion detection, intruder detection in surroundings of smart home, tourist spot monitoring |
|  | 3 (use cases 5.2, 5.7, 5.10, 5.11, 5.12, 5.23) | Factory (100m2), crossroad, highway, railway [air] NOTE 4 Object to be detected: Animal, Human, UAV, Vehicle | 95 | 1 | 1 | 1 NOTE 5 | 1 | 1 NOTE 5 NOTE 8 | 1 x 1 NOTE 9 | 100 NOTE 6 1000 NOTE 10 detection in highway: 5000 | 0.05 NOTE 11 | 2 | 2 | pedestrian/animal intrusion detection on a highway/railway, sensing at crossroads with/without obstacle, UAV flight trajectory tracing UAV collision avoidance, AMR collision avoidance in smart factories |
|  | 4 (use cases 5.20, 5.22, 5.25, 5.32,5.27) | Factory and public safety Object to be detected: Animal, Human, UAV, AGV/AMR, Vehicle | 95, Public safety: 99 | 0.5 | 0.5 | 0.1; Vehicle: 15; Pedestrian: 1.5 | N/A; Pedestrian: 1.5 | 0.5m | factories: 0.5 x 0.5 | Detection for smart grid: 5000; Public safety: 1000; Factory:100 | 0.1 | 1 | 3 | Parking Space Determination, UAVs/vehicles/pedestrians detection near Smart Grid equipment (NOTE 7), immersive experience based on sensing, integrated sensing and positioning in factory hall |
|  | 5 (use cases 5.28) | ADAS Object to be detected: Vehicle Public area safety, | 95 | short-range radar:2.6 Long range radar: 1.3 | 0.5 | 0.12 | N/A; | 0.4 | 0.6 | Short range radar: 20; Long range radar: 50 | Short range radar: 0.05; Long range radar: 0.2 | 10 | 1 | public safety search and rescue or apprehend, ADAS |
| Environment monitoring | 6 (use cases 5.3 and 5.5.) | Rainfall monitoring and flooding NOTE 14 Object to be detected: Rain | 95 | 10 | 0.2 NOTE 15 | N/A | N/A | N/A | N/A | 60000 | 1<10min, application configurable | 0.1 | 3 | rainfall monitoring, flooding monitoring |
| Motion monitoring | 7 (use cases 5.15, 5.24) | Indoor human motion -sleep monitoring NOTE 12, sports monitoring NOTE 13, | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | sleep monitoring, sports monitoring |
|  | 8 (use case 5.29) | Hand gesture recognition | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 | 0.1 | 5 | 5 | Hand gesture recognition |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: To detect the UAV existance (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. NOTE 4: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. NOTE 5: The KPI values for UAVs are sourced from [25] and [40] and for factories are sourced from [47]. NOTE 6: The value 100 ms is sourced from [28] and is valid for sensing at crossroads. NOTE 7: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. The size of the park of Smart Grid depends on the real environment. NOTE 8: To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 9: To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 10: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1s. NOTE 11: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 12: Additional KPI on human motion rate accuracy of 2 times/min (0.033 Hz). NOTE 13: Additional KPI on human motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) NOTE 14: Rainfall estimation accuracy is1 mm/h[39] and describes the closeness of the measured rainfall estimation to its true rainfall value. NOTE 15: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The values in brackets are FFS