3GPP TSG SA WG 1 Meeting #110 	S1-252482

19-23 May 2025, Fukuoka, Japan	(revision of S1-252269)

Title:	Use case on integration of satellite sensing and 3GPP sensing

Agenda item:	8.1.4

Source:	CSCN

Contact:	Qiuting Li (liqt@chinasatnet.com.cn),

Xin Miao (miaox@chinasatnet.com.cn)

Abstract: This contribution is to integrate 3GPP sensing (both TN and NTN sensing) and non-3GPP sensing for wide sea area ship detection and tracking.

1. Introduction

“Integrated sensing and communication” has been identified as one of key areas for 6G study. This contribution proposes a new use case on integration of satellite sensing and 3GPP sensing for the maritime area monitoring, target recognition and tracking.

2. Reason for Change

3GPP system supports satellite sensing on board satellites to enable diverse scenarios such as rescue and disaster relief, sea area monitoring, and global target tracking.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

7.x	Use case on integration of satellite sensing and 3GPP sensing for maritime area monitoring and target tracking

7.x.1	Description

Against the backdrop of the booming marine economy and the growing demand for maritime security, existing monitoring methods struggle to meet the real-time monitoring requirements of vast sea areas. The 6G integrated communication and sensing use case proposed in this proposal provides a solution for sea surface monitoring, vessel identification, and tracking by constructing a collaborative sensing system of 3GPP sensing(including TN and NTN) and non-3GPP sensing (e.g. satellite remote sensing).

In order to realize the tracking and identification of ship targets in the wide - range sea surface, within the range where the sensing signals of ground base stations are reachable, ground base stations are used to provide sensing services. In the offshore areas where the signals of ground base stations are unreachable, on - board base station sensing technology is used to provide sensing services. At the same time, satellite remote sensing is used for auxiliary sensing to improve the sensing performance. 3GPP sensing and non-3GPP sensing can be deeply integrated through 6G integrated communication and sensing technology to achieve a full-process perception of sea area targets, from coarse to fine.

7.x.2	Pre-conditions

A) There exist two types of satellite systems: communication satellites and sensing satellites. Sensing satellites access the satellite communication network via inter-satellite links.

(B) The satellite communication network is equipped with an on-board core network, able to control the access of sensing satellites and process satellite sensing data, among other functions.

(C) Users access the network through terrestrial mobile base stations.

7.x.3	Service Flows

Figure 7.x.3-1 Integration of satellite sensing and communication for maritime area

- User A, a manager of a marine fishery company, initiates a request for sea surface monitoring, vessel identification, and tracking services in a vast sea area through a dedicated terminal to the 6G network. The goal is to plan fishing areas rationally and ensure navigation safety. User A specifies demand parameters such as the monitoring sea area range and time period.

- After receiving the request, the 6G network determines the sensing strategy based on the request scope, using satellite on-board base stations or ground base stations for sensing, and seeking auxiliary sensing from remote sensing satellites..

- Satellite remote sensing data and the sensing data from terrestrial/non-terrestrial base stations are transmitted to the network's central processing platform. Using 6G integrated communication and sensing algorithms, the macroscopic information from satellites and the precise data from base stations are deeply integrated. This process eliminates data redundancy and errors, constructing a comprehensive and accurate vessel information model for the target sea area.

- The processed monitoring data and the results of vessel identification and tracking are real-time fed back to User A's terminal device through a low-latency and highly reliable 6G network channel. User A can visually view the vessel dynamics in the sea area on the terminal interface, providing strong support for fishery resource management and navigation scheduling decisions.

7.x.4	Post-conditions

By integrating 3GPP terrestrial/non-terrestrial sensing with satellite remote sensing, wide-area sea monitoring and ship tracking can be effectively achieved.

7.x.5	Existing features partly or fully covering the use case functionality

None.

7.x.6	Potential New Requirements needed to support the use case

[PR 7.x.6-1] Subject to regulation and operator's policy, the 6G system with satellite access shall support the integration of 3GPP sensing (i.e. TN and NTN sensing) and non-3GPP sensing (e.g. satellite remote sensing) to address the sensing request for large-scale  areas as described in the following table.

Table 7.x.6:  Performance of ship tracking in large-scale sea areas

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Ship detection and tracking | ship tracking | 95 | ≤10 NOTE1 | NA | NA | NA | ≤10 | NA | 5000 | 1 | 5 | 5 | Ocean waters,near shore,off-shore |
| NOTE 1: The KPI values for ship tracking on the sensing service are referred to [xx1]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

2  References

The following documents contain provisions which, through reference in this text, constitute provisions of the present

document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or

non- specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a

GSM document), a non-specific reference implicitly refers to the latest version of that document in the same

Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[xx1] IMO Resolution A.915(22) (2002) Revised Maritime Policy and Requirements for a Future GNSS. Adopted on January 22nd, 2002, London.