3GPP TSG SA WG 1 Meeting #111 	S1-253037

25-29 August 2025, Goteborg, Sweden

Source:	ESA, Airbus

pCR Title:	Pseudo-CR on “8.10 Use case on hybrid NTN and GNSS positioning”

Draft Spec:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Felix Abel, felix.abel@esa.int

Abstract: The Editor’s Note is proposed to be replaced by performance requirements.

1. Introduction

In 3GPP TR 22.870, The use case on hybrid NTN and GNSS positioning includes an Editor’s Note on the KPIs, which needs to be replaced by performance requirements.

2. Reason for Change

The KPIs are currently FFS.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.3.1.

* * * Change * * * *

### 8.10.6	Potential New Requirements needed to support the use case

[PR 8.10.6-1]	The 6G system with satellite access shall be able to provide a positioning service supported by hybrid NTN and GNSS positioning technologies.

[PR 8.10.6-2]	The 6G system with satellite access shall be able to provide positioning service with the following KPIs:

Table 8.10.6-1: Performance requirements for positioning services with satellite access

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| UE with partly obstructed sky | [10] m | [3] m | [95 %] | [1] s | Outdoor rural and urban | Up to [250] km/h | Generic |
| Airborne and maritime | [3] m | [3] m | [99 %] | [1] s | Outdoor rural | Up to [500] km/h | Airborne or maritime |

[PR 8.10.6-3]	The 6G system with satellite access shall be able to support Network-based Assisted GNSS to improve NTN positioning.