3GPP TSG-SA WG1 Meeting #112 	S1-254024

17-21 November 2025, DALLAS, USA	(revision of S1-25xxxx)

Title:	Consolidation of KPI requirements on AI section

Agenda Item:

Source:	Moderator

Contact:	Qun Wei, weiqun5@chinaunicom.cn

Abstract: Prepare and propose a way for consolidation on KPI requirements on AI section.

1. Introduction

Based on the meeting preparations, this document only provides the consolidation way forward and discussion regarding the AI KPIs, based on TR 22.870 and do not include performance requirements with editor’s note.

2. Reason for Change

To provide communication performance requirements contribution and reflect key points regarding the performance requirements of AI sections.

3. Proposal

It is proposed to agree the following changes to new version of 3GPP TR 22.870.

Discussion Part

Besides the basic template consolidation, the following comments need to be considered：

UC 6.10, 6.48 and 6.49

CM1: the “joint e2e latency” is probably to be properly defined or decoupled?

CM2: Joint E2E latency value needs to be divided into a communication value and a compute value.

* * * First Change * * *

Y Consolidated potential KPIs

y.1. Performance requirements for AI service

Editor's Note: the following potential performance requirements from immersive session are considered for consolidation.

- TR 22.870 0.4.0 [PR 9.2.6-1] (Table 9.2.6-1)

- TR 22.870 0.4.0 [PR 9.3.6-1] (Table 9.3.6-1)

- TR 22.870 0.4.0 [PR 9.3.6-4] (Table 9.3.6-2)

- TR 22.870 0.4.0 [PR 9.5.6-5] (Table 9.5.6-1)

- TR 22.870 0.4.0 [PR 9.6.6-3] (Table 9.6.6-1)

- TR 22.870 0.4.0 [PR 9.7.2-1] (Table 9.7.2-1)

- TR 22.870 0.4.0 [PR 9.9.6-2] (Table 9.9.6-1)

- TR 22.870 0.4.0 [PR 9.12.6-6] (Table 9.12.6-1)

The following potential performance requirements from AI session are considered for consolidation.

- TR 22.870 0.4.0 [PR 6.10.6-6] (Table 6.10.6-1)

- TR 22.870 0.4.0 [PR 6.26.6-1] (Table 6.26.6-1)

- TR 22.870 0.4.0 [PR 6.48.6-1] (Table 6.48.6-1)

- TR 22.870 0.4.0 [PR 6.49.6-1] (Table 6.49.6-1)

The following potential performance requirements are excluded due to the remaining ENs and will be taken into consideration after the ENs are cleaned up:

- TR 22.870 0.4.0 [PR 9.8.6-2] (Table 9.8.6-1)

- TR 22.870 0.4.0 [PR 9.18.6-1] (Table 9.18.6-1)

- TR 22.870 0.4.0 [PR 6.38.6-3] (Table 6.38.6-1)

[CPR y.1-1] The 6G system shall provide various communication and AI services with the following KPIs.

Editor’s Note: 	Unless stated otherwise, the definition and understanding "Reliability", "Service bit rate: user-experienced data rate", and "Transfer interval" refers to [22.261] and [22.256].

Table y.1-1: Performance requirements for burst traffic

| Use Cases | Burst size | Max Allowed latency for a burst | Service bit rate: user-experienced data rate | transmission latency of a packet | Average packet size | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement (UC 9.3 A) | UL: [150 Mbps] (note A-1) | UL: [1500ms] (note A-2) | UL: [100 Mbps] | N/A | N/A | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement (UC 9.3 A) | UL: [5-20 Mbps] (note A-1) | UL: [50ms] (note A-2) | UL: [100 – 400 Mbps] | N/A | N/A | Stationary or Pedestrian | Countrywide |
| Image based GenAI app (UC 6.26 B) (note B-1) | 400 KB | 50ms | 64 Mbps | 20ms | > 800B | N/A | N/A |
| Video based GenAI app (UC 6.26 B) (note B-1) | 20 MB | 400ms | 400 Mbps | 20ms | > 800B | N/A | N/A |
| Chatbot (UC 6.26 B) (note B-1) | 0.5 KB | 20ms | 200 Kbps | 30ms | < 800B | N/A | N/A |
| NOTE A-1: Assuming 6 x 4K raw pictures and compression ratio in [21] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [23] for gaming enhancement. NOTE A-2: 1500 ms is derived from the E2E latency of 3 s (based on users' patience statistics as shown in [22]) and 1.5 s for processing in the cloud and downloading. 50ms uplink latency is derived from [24]. NOTE B-1: Max allowed latency for a burst: max latency for sending out the whole packets within a burst. |  |  |  |  |  |  |  |

Editor’s Note: The table example above serves as Option I.

We have received an alternative merging approach as Option II, which consolidates Use Case 6.26 into Table y.1-2. Keep UC 9.3 as a separate table.

Option III is to merge 9.3 and 6.26 into Table y.1-2.

The immersive and AI can be two separate documents.

Table y.1-2: Performance requirements for communication and AI services

| Use Cases | Traffic type | Average packet//frame size (byte) | Transfer interval | Service bit rate: user-experienced data rate | Joint E2E latency (note 1) | Reliability |
| --- | --- | --- | --- | --- | --- | --- |
| Home robot perceiving overall context of the scene (UC 6.10 A) (note A-5) | UL camera data (note A-1) | [<1000] | [10ms] | [20-60 Mbps] (note A-2) | [150ms] (note A-3) | [99.9 %] |
| Identifying individual objects in the scene (UC 6.10 A) (note A-5) | UL camera data (note A-1) | [<1000] | [10ms] | [20-60 Mbps] (note A-2) | [200-300ms] (note A-4) | [99.9 %] |
| Service robot (UC 6.48 B) | UL sensor data (without LiDAR) (note B-1) | 1250-12500 | 10ms | UL: 1-10 Mbps | 100-150ms (note B-4) | 99.99% |
|  | UL LiDAR | 28800 | 100ms | UL: 27.6 Mbps (note B-3) | 100-150ms (note B-4) | 99.99% |
|  | Control command (high level task, action plan, etc.) (note B-2) | 625-12500 | 50ms | DL: 0.1-2 Mbps | 100-150ms (note B-4) | 99.99% |
| Support robot conscious awareness for interacting with human user (UC 6.49 C) | UL cameras data (note C-1) | [<1000] | [10ms] | UL: [20-60 Mbps] (note C-2) | [<200ms] (note C-3) (note C-5) | [99.9%] |
| NOTE 1: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment), and UE is only considered to contribute to the communication service latency. NOTE A-1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE A-2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE A-3: Based on psychophysical and neurophysiological studies [281], [282], human beings can perceive the gist (e.g. overall meaning) of complex visual scenes within around 150ms after stimulus onset. This rapid initial perception enables us to grasp the general context of a scene, even if the stimulus is briefly presented (around 10ms). NOTE A-4: For human beings, it takes longer for a human being to identify individual objects, e.g. finer object identification requires larger than 200ms [283] and around 300ms [284]. NOTE A-5: In the target scenarios, robots are expected to have similar perception time as an average human being. NOTE B-1: Refers to the kinematic state, environment perception, manipulation status info except LiDAR to be sent from the service robot to the network to enable effective motion planning, object interaction and navigation. NOTE B-2: Refers to the control command towards service robot, e.g. high-level task, action plans, motion strategy, gripper command, etc. NOTE B-3: The data rate of LiDAR is based on frame rate 10Hz, 28800 points/frame, 12byte for one point cloud, i.e. 28800*16*8*10=27.6Mbps. NOTE B-4: E2E latency includes two parts: the round-trip latency for communication service and the latency for AI inference within the service hosting environment. The typical robot control loops require 100-150ms latency [273] for AI inference, communication and control. For example, the communication may take about 40ms while the AI inference may take about 100ms [274] for the service robot. NOTE C-1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE C-2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE C-3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making based on various stimuli. Such human brain reaction time ranges from mean auditory reaction time 140-160ms, touch 155ms, to visual reaction time 180-200ms [279]. NOTE C-5: The human to robot latency (vice versa) is not included. |  |  |  |  |  |  |

Editor’s Note: 	We have received a suggestion to merge numbers of use case 6.10 and 6.49.

* * * End of changes * * *