3GPP TSG-SA WG1 Meeting #112 	S1-254098

Dallas, USA, 17 - 21 Nov 2025	(revision of S1-25xxxx)

Title:	Update to UC 5.7.9_on Network coverage and usage verification

Agenda Item:	8.1.2.2

Source:	Qualcomm

Contact:	Francesco Pica, fpica@qti.qualcomm.com

Abstract: This contribution proposes an update to the potential requirements for the use case on network coverage and usage verification.

1. Introduction

As proposed in previous SA1 contributions (S1-253097), it would be beneficial to enable mechanisms that allow to configure and collect selected information on network coverage and/or traffic usage from UEs not only when UEs are in their home PLMN, but also when roaming, based on arrangements/agreement between HPLMN and VPLMN. This was proposed as additional note to the PR#1, later removed due to some concerns, e.g., regarding lack of VPLMN control. In our view, those concerns could be addressed by including means to enable/disable the functionality from the visited network (in addition to policies from the Home PLMN). The HPLMN can also expose the information to the VPLMN, if HPLMN policy allows that.

2. Reason for Change

Based on the above, we propose to update the potential requirements of use case 5.7, extending support of the functionality in roaming scenarios assuming proper VPLMN control.

3. Proposal

It is proposed to agree the following changes in TR 22.870.

=========================== pCR PROPOSAL =========================

#### 5.7.9.3	Potential New Requirements

[PR 5.7.9.3-1] Subject to operators’ policies, regulations and user consent, the 6G system shall provide means for a network operator to monitor network coverage and/or traffic usage, including collection of information from UEs (with subscription to the network operator).

NOTE 1:	Monitoring and collection of information from a UE is assumed to be authorized and configured by the UE’s home operator, for certain geographical area(s) and/or time(s).

NOTE 2:	The traffic usage information collected from a UE is for traffic associated with that UE.

[PR 5.7.9.3-2] Subject to operators’ policies, regulations and user consent, the 6G system shall provide means for a network operator to configure and collect network coverage and/or traffic usage from UEs (with subscription to the network operator) also when they are roaming in a visited PLMN, if allowed/enabled by the VPLMN.

NOTE 3:	The information collected by UEs when in the VPLMN can be securely exposed by the home operator to the VPLMN, based on inter-operator agreement.