3GPP TSG-SA WG1 Meeting #112 	S1-254102

Dallas, USA, 17-21 November 2025	(revision of S1-253098)

Source:	Qualcomm

New use case title:	Re-submission of Use Case on PWS delivery over non-3GPP access

Draft TS/TR:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.9.2

Document for:	Approval

Contact:	Francesco Pica

Abstract: This contribution re-proposes a uses case on delivery of Public Warning System (PWS) messages over non-3GPP access, when delivery via 3GPP access is not available. It is a re-submission from SA1#111, with a small update to the PRs (Note 2).

1. Introduction

Public Warning System (PWS) messages are messages that need to be delivered in case of emergency (e.g., earthquake, tsunami, flooding, wildfires, etc.) to the UEs connected to or camping on a PLMN. In such cases, it is important to deliver the PWS messages promptly and to as many UEs as possible present in the target geographical area. There can however be situations where UEs in a target geographical area cannot be reached via 3GPP access. For example, some UEs could be out of coverage from the cellular RAN (remote areas or indoor) or some part of the network covering that geographical area is not operational due to temporary failure, or energy saving conditions. In such situations it is paramount to have alternative ways to deliver the PWS messages to UEs that would otherwise not receive them.

2. Reason for Change

Introduce requirements to support delivering of PWS messages over non-3GPP access.

3. Proposal

It is proposed to agree the following changes toward 3GPP TR 22.870.

* * * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[X]	IEEE Std 802.11-2024: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[Y]	IEEE Std 802.11bc-2023: " EEE Standard for Information Technology--Telecommunications and Information Exchange between Systems Local and Metropolitan Area Networks--Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 7: Enhanced Broadcast Services".

* * * * Second Change * * * *

## W.Y	Use case on PWS delivery over Non-3GPP Access

### W.Y.1	Description

In case of emergency (e.g., earthquake, tsunami, wildfires) Public Warning System (PWS) messages need to be delivered to the user as soon as possible. Typically, this is done by the Cell Broadcast Entity (CBE) sending the PWS messages to the PLMN so that they can be broadcast to the users via 3GPP access. However, there may be situations in which some users are not reachable via the 3GPP access of the MNO’s PLMN. In such situations the MNO should compensate for such limitation by delivering the PWS messages over (integrated) Non-3GPP Access (see Figure W.Y.1-1).

Figure W.Y.1-1: PWS delivery over WLAN access, when RAN is not available.

NOTE:	The WLAN logical box in the figure includes Wi-Fi AP and corresponding Gateway toward the PLMN CN.

It should be noted that IEEE recently adopted enhancements to the IEEE 802.11 standards to support broadcast services (see Enhanced Broadcast Services (EBCS) in IEEE 802.11-2024 [X] and IEEE 802.11bc-2023 amendments [Y]). Such enhancements enable the multicast/broadcast of data to the UE/Stations via Wi-Fi Access Points (APs) and can be leveraged to deliver PWS warning messages.

### W.Y.2	Preconditions

The preconditions are:

- PLMN-A provides both RAN and Wi-Fi connectivity in the country.

- PLMN-A subscribers (User-A) use PLMN-A credentials to authenticate over Wi-Fi, and establish connectivity to PLMN-A.

- RAN coverage is not available in a certain area (“Area-2”, as in the Fig. above), e.g., due to deployment limitations or failure, and CN is aware of it.

- UE, Wi-Fi AP and Wi-Fi gateway support the delivery of PWS messages over Wi-Fi.

### W.Y.3	Service Flow

- User-A is at home (located within Area-2), with no cellular coverage, and his/her phone connects to PLMN-A via Wi-Fi.

- PLMN-A network receives a PWS message (e.g. evacuation alert due to wildfire) from the national Alert system, affecting also Area-2.

- CN, aware of RAN unavailability within Area-2, will forward the PWS message to the Wi-Fi network (managing Wi-Fi APs deployed within Area-2).

- User-A gets the PWS alert over Wi-Fi and evacuates the area safely.

### W.Y.4	Post-conditions

Users in Area-2 without cellular coverage, but connected to the PLMN-A Wi-Fi, can still receive PWS alerts.

### W.Y.5	Existing features partly or fully covering the use case functionality

Existing PWS requirements do not support non-3GPP access.

### W.Y.6	Potential New Requirements needed to support the use case

[PR W.Y.6-1] Subject to operator’s policy and/or regulatory requirements, the 6G system shall be able to support delivery of PWS messages to a UE registered to a PLMN via non-3GPP access, e.g., if delivery via RAN is not available (at a certain location or time).

NOTE 1: It is assumed that existing requirements related to upper-layer PWS functionalities, e.g., duplication detection and geo-fencing (defined for 3GPP access), are also applicable to a UE receiving PWS over non-3GPP access.

NOTE 2: Transfer of PWS messages over non-3GPP access can use Enhanced Broadcast Services (EBCS) (see [x, y]), in the Wi-Fi network and UE.

* * * * End of Changes * * * *