3GPP TSG SA WG 1 Meeting #112 	S1-254166

17-21 November 2025, Dallas, Texas, USA

Source:	CEWiT

pCR Title:	Pseudo-CR on Proposal on Update of CPR: Y.1.5 to Y.1.2

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.10

Document for:	Approval

Contact:	ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes the relocation of a potential requirement 5.5.5.3.1 currently placed under "Data Collection and Consumption" to the "Security and Privacy" section. Although the requirement involves data processing, its key objective is the anonymization of UE data through the removal of UE identities prior to exposure. As this requirement predominantly supports privacy goals, rather than data consumption, this realignment ensures logical consistency and better reflects the intent of the requirement within the specification.

* * * First Change * * * *

# Y	Consolidated potential functional and performance requirements

Editor’s Note:  PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Editor’s Note: CPR Table(s) can be modified (e.g. split, merged, renamed, moved) during the consolidation process.

## Y.1	Consolidated potential Functional Requirements

### Y.1.2	Security and Privacy

Editor’s Note: This clause contains consolidated potential requirements for 6G that include security, privacy and lawful interception. This includes (but not limited to) PRs from clauses 5.5 and X.1.

NEW Editor’s Note:  PR 5.5.1.2-5, PR 5.5.4.2-10, and PR 5.5.6.3-1 have editor’s notes associated with them, so they are not included in consolation into Table Y.1.2-1 at this time.

Table Y.1.2-1 – Security and Privacy

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-1-1 | The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services. | PR 5.7.1.2-4 | Fixed Wireless Access |
| Y.1.2-1-2 | The 6G system shall be able to protect UE’s subscriber identities from attacks, and potential attacks posed by quantum computing. | PR 5.5.7.3-3 PR 5.5.2.6-1 | UE Identity protection Quantum Resistant Security |
| Y.1.2-1-3 | The 6G network shall provide security mechanisms for secure access to elements of the core network of the 6G system and enable the network operator to ensure there are no unintended changes of the elements of the 6G network | PR 5.5.1.2-1 PR 5.5.1.2-3 | Network Security |
| Y.1.2-1-4 | The 6G network shall support establishment of secure communication between elements of the network and secure communication on all 3GPP defined interfaces of the core network of the 6G system while protecting network related information (e.g. network element identities, topology) from disclosure to unauthorized parties. | PR 5.5.1.2-1 PR 5.5.1.2-2 | Network Security |
| Y.1.2-1-5 | The 6G system shall provide security protection for communication (e.g. authentication methods) against the potential attacks posed by quantum computing. | PR 5.5.2.6-1 | Quantum Resistant Security |
| Y.1.2-1-6 | The 6G system shall ensure the cryptography agility (i.e. post-quantum cryptography algorithms-related smooth migration, switching, update) for the 6G system and its services to remain secure against new threats. | PR 5.5.2.6-3 | Quantum Resistant Security |
| Y.1.2-1-7 | Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a False Base Station from an authentic Base Station. | PR 5.5.3.2-1 | System Security False Base Station |
| Y.1.2-1-8 | The 6G system shall provide efficient mechanisms to support authentication and secure communication in inter-PLMN and intra-PLMN networks. | PR 5.5.4.2-1 | System Security Network Security |
| Y.1.2-1-9 | The 6G system shall support fine-grained security isolation based on the security needs of different businesses requirements for intra-PLMN networks. | PR 5.5.4.2-2 | System Security Network Security |
| Y.1.2-1-10 | The 6G system shall be able to provide flexible orchestration of security capabilities to ensure customized security requirements from customers. | PR 5.5.4.2-3 | System Security Network Security Security Management |
| Y.1.2-1-11 | The 6G network shall support security management capabilities that enable - the identification of potential security threats to communication and network elements, - security analysis and security enhancement policy generation to mitigate attacks and/or security issues in operator management system, - security response to implement enhanced security policy from management system to network elements, in order to recover network from disturbance | PR 5.5.4.2-4 PR 5.5.4.2-5 PR 5.5.4.2-6 | Security Management |
| Y.1.2-1-12 | The 6G system shall enable operators using SIM as trust anchor to provide identity and authentication service to the 3rd parties. | PR 5.5.4.2-7 | Identity Authorization Service |
| Y.1.2-1-13 | The 6G system shall support digital identities by - verifying the association between the SIM identity and the digital identity of the different digital representatives, - authorising required services of the digital identity of the digital representatives of users and revoking the digital identity if needed. | PR 5.5.4.2-8 PR 5.5.4.2-9 | Digital Identity |
| Y.1.2-1-14 | The 6G system shall support security and privacy protection of the 6G System Data. | PR 5.9.2.2-4 | 6G System Data System Security |
| Y.1.2-1-15 | Subject to operator policy and regulatory requirements, the 6G system shall support privacy protection, location privacy, identity protection for UEs accessing 6G network for services (e.g. communication, sensing, AI inferencing), and for the corresponding information for any information exposure to an authorized 3rd party. | PR 5.5.7.3-1 PR 5.5.7.3-2 | Security & Privacy for services, data & exposure |
| Y.1.2-1-16 | Subject to regulation, operator(s) policy and user consent, the 6G network shall support access from authorized third parties to processed data related to UEs served by the network (for example but not limited to number of UEs in a geographical location, their mobility pattern, application usage trends) without exposing UE identities and individual user data. NOTE 1: Processed data refers to the analysis of data to produce new data including among other statistics on the data, correlating data, aggregating data. NOTE 2: The existing security, privacy policies and procedures for collecting and processing user data (e.g. via anonymisation) will be preserved. | PR 5.5.5.3-1 | Security & Privacy for information exposure |
| Y.1.2-1-17 | Subject to regulation, operator policy and user consent, the 6G system shall support mechanisms to protect data privacy during the processing of sensing data. NOTE: The processing of sensing data is compliance with applicable regulations, and other pre-defined service criteria. | PR 7.12.6-2 | Data Privacy Sensing Data Processing |

Table Y.1.2-2– Lawful Interception

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.2-2-1 | The 6G system shall be able to support the following services: Lawful Interception, ref TS 22.261 [14] and TS 33.126 [312] and Other regulatory services, based on regional/national regulatory requirements. | Clause 5.4.2 | Could be considered as part of Table Y.1.1-1 |
| Y.1.2-2-2 | The 6G network shall provide security mechanisms that offer protection of communication and data while at the same time not hindering Lawful Interception (ref TS 33.126 [312], clause 6.4). | PR 5.5.1.2-4 | Could be considered as part of Table 1.2.-1 |
| Y.1.2-2-3 | The 6G system shall support the mapping of permanent and temporary identifiers of all subscribers, e.g. for Lawful Interception purposes. | PR 5.5.2.6-2 | Identifiers |
| Y.1.2-2-4 | Subject to national or regional regulatory requirements, the 6G system shall provide a privacy override mechanism to enable access to retained data for law enforcement purposes and Lawful Interception purposes which may otherwise be subject to the privacy protection requirements [313]. | PR 5.5.7.3-4 | Privacy override |
| Y.1.2-2-5 | Subject to operator policy and regulatory requirement, the intelligent immersive calling service shall comply to the existing regulatory requirements (e.g. lawful interception). | PR 9.10.6-2 | Intelligent immersive calling service |
| Y.1.2-2-6 | In addition to the LI requirements specified in TS 22.261 [14] the 6G system shall support Lawful Interception - Where the 6G system is operated or where the services are provided, - When (some) network functions are 3rd party provided or operated by different operators, - In all supported interworking and interoperating scenarios, including roaming (in visited networks) NOTE: These requirements apply to all services which must comply with the 3GPP SA3-LI specifications, subject to national or regional regulations. | Clause X.1 | How much of this is covered as part of Table Y.1.1-1? |
| Y.1.2-2-7 | The 6G system shall support the retention of data to meet national or regional regulatory requirements, [314], [315], [313], and [316]. | Clause X.1 | Data retention Could be considered as part of Table Y.1.5-1 – Data Collection and Consumption |

### Y.1.5	Data Collection and Consumption

Editor’s Note: This clause contains consolidated potential requirements for 6G that address data collection, management, and consumption. This includes (but not limited to) PRs from clauses 5.9.2 and W.

NEW Editor’s Note:  PR 5.9.2.2-2, -3, and -6 have editor’s notes associated with them (concerning user consent), so they are not included in consolation into Table Y.1.5-1 at this time.

Table Y.1.5-1 – Data Collection and Consumption

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| Y.1.5-1-1 | The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services. | PR 5.9.2.2-1 |  |
| Y.1.5-1-2 | Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system. | PR 5.9.2.2-7 |  |