3GPP TSG-SA WG1 Meeting #112 	S1-254210

Dallas, USA, 17th - 21st November, 2025	(revision of S1-253326)

Title:	Use case on AI-enabled satellite-UAV collaborative emergency service

Agenda Item:	8.1.3-Artifcial Intelligence

Source:	BUPT, Pengcheng Laboratory, CMCC, China Telecom, ZGC Institute of Ubiquitous-X Innovation and Application, OPPO

Contact:	Xiaodong Xu (xuxiaodong@bupt.edu.cn), Mengying Sun (smy_bupt@bupt.edu.cn), Nan Ma (manan@bupt.edu.cn), Sheng Jiang (shengjiang@bupt.edu.cn), Jinghong Huang (huangjh@bupt.edu.cn), 	Zhe Zheng (zhengzh02@pcl.ac.cn), Lifeng Xie (xielf@pcl.ac.cn),

Xu Xia (xiaxu@chinatelecom.cn)

Abstract: Use case on AI-enabled satellite-UAV collaborative emergency service.

* * * First Change* * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TS 22.261: "Service requirements for the 5G system".

- 3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks".

- 3GPP TR 22.822: "Study on using Satellite Access in 5G".

- Gao H, Cao R, Xu W, et al. Space-air-ground integrated networks with task-driven connected intelligence[J]. IEEE Wireless Communications, 2024.

- Meng S, Wu S, Zhang J, et al. Semantics-empowered space-air-ground-sea integrated network: New paradigm, frameworks, and challenges[J]. IEEE Communications Surveys & Tutorials, 2024, 27(1): 140-183.

* * * Next Change (all the text is new)* * * *

6.x	Use case on AI-enabled satellite-UAV collaborative emergency service

6.x.1	Description

Against the backdrop of intensifying global climate change, sudden natural disasters are imposing higher demands on emergency response capabilities. When disasters such as floods occur, traditional on-site reconnaissance methods struggle to meet the need for rapid and accurate decision-making due to their inefficiency and high risks. In the future, 6G technology will empower drone-based emergency rescue services by capturing high-definition images and videos to provide critical information sources for disaster assessment. Considering that ground-based 6G networks are often limited or unstable in disaster areas, the data captured by drones must be transmitted with the assistance of 6G NTN (Non-Terrestrial Networks) networks. However, the satellite-terrestrial links face transmission challenges such as limited bandwidth and high dynamics, necessitating more efficient data transmission solutions.

To address this, AI models deployed on UAVs can convert raw images/videos into key semantic features, such as submerged areas and damaged buildings, and compress them into token sequences. This achieves a fundamental shift from “transmitting bit-level data” to “transmitting semantic information.” These semantic features (e.g., token) are reliably transmitted to emergency centers via NTN networks, where decoding models reconstruct the on-site scenario in real time or generate situational reports. This technology effectively overcomes the transmission bottleneck caused by limited link resources, ensures near-real-time presentation of disaster situations, provides intelligent support for rescue decision-making, and comprehensively enhances emergency response efficiency.

6.x.2	Pre-conditions

The satellite communication system has been fully deployed and is capable of covering target areas (such as remote regions, oceans, etc.), supporting new information extraction and enhancement modules (including signal processing, information analysis, etc.). The components integrated into user terminals (satellite phones, shipborne communication terminals, UAV, etc.) can extract, identify, and efficiently encode source data, as well as accurately decode received information, interacting seamlessly with the satellite communication network.

AI plays a pivotal role in the UAV inspection process by processing the multimodal data on the UAV itself to extract meaningful insights. Token communications, a semantic communication paradigm, employ an AI-based tokenizer model to convert raw data into compact token sequences that capture essential meanings, thereby achieving high data compression and enabling interpretable, task-oriented transmission. At the base station, matching AI models decode and reconstruct the data for real-time analysis. By integrating AI with communications, the 6G-enabled UAV inspection system reduces bandwidth consumption, ensures reliable transmission under low SNR conditions.

In the absence of functional terrestrial mobile networks, UAVs capable of capturing high-resolution images of ground areas-face challenges in effectively transmitting the captured data to the emergency command center. To address this, satellite communication is leveraged. UAVs first upload the captured images to satellites, which then relay the data to ground base stations. These base stations subsequently forward the information to the emergency center.

6.x.3	Service Flows

Figure 6.x.3-1: Satellite-UAV collaborative emergency service scenario.

- A sudden flood struck a region, severely damaging local mobile communication infrastructure. The emergency center received distress calls from residents and activated rescue response measures.

- The emergency center dispatches UAVs to capture images of the disaster area. Hovering over the affected regions, the UAVs collect high-resolution ground images to identify damaged zones, blocked roads, and potential signs of life. However, due to local network disruptions, these images cannot be directly transmitted to the emergency center through the terrestrial 6G network.

- The UAVs and the emergency center deploy AI models (e.g., tokenizers). The UAV uses the AI model to convert captured images or videos into semantic feature data (e.g., token), which are transmitted via satellites to the 6G network and then forwarded to the emergency center.

- The 6G network performs differentiated protection and priority scheduling based on the importance of semantic feature data (e.g., token), ensuring the accurate and reliable transmission of critical information.

- The emergency center reconstructs corresponding images and videos from the received semantic feature data (e.g., token) using matched AI models, thereby enabling real-time visualization of the disaster scene.

6.x.4	Post-conditions

The emergency center receives AI-processed data through NTN and 6G networks, maintaining real-time transmission of disaster-site image/video even under local network disruptions. Semantic analysis performed at UAVs or NTN network extracts critical features, supporting robust transmission of critical information. The NTN network dynamically prioritizes data transmission is delivered first and bandwidth resources are efficiently utilized.

6.x.5	Existing features partly or fully covering the use case functionality

Existing satellite systems provide wide-area coverage and onboard computing for semantic analysis and data prioritization, enabling emergency support in remote or disconnected regions. UAVs equipped with high-resolution imaging and lightweight AI modes can autonomously identify critical visual information of image/video and relay it to satellites via UAV-satellite links when ground 6G networks are down. 6G network further supports high-efficiency data transmission and inference enhancement for resilient, real-time disaster response.

6.x.6	Potential New Requirements needed to support the use case

[PR 6.x.6.1] Subject to user consent and operator policy, the 6G network shall support exposing AI and computing services hosted in the Non-Terrestrial Networks (NTN) (e.g., satellite) to UEs as part of network capability exposure.

[PR 6.x.6.2] Subject to user consent and operator policy, the 6G network shall support AI-processed data (e.g., token data, semantic data) transmission that tolerates a certain level of error or packet loss while ensuring service continuity.

[PR 6.x.6.3] Subject to operator’s policy, the 6G network shall support differentiated and priority scheduling for AI-processed data (e.g., token data, semantic data) based on the importance indication of the AI-processed data.