3GPP TSG SA WG 1 Meeting #112 	S1-254286

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254039)

(revision of S1-253113)

Source:	CEWiT, Tejas

pCR Title:	Pseudo-CR on Positioning Service for Network Optimization

Draft Spec:	3GPP TR 22.870 v0.4.1

Agenda item:	8.1.2.2

Document for:	Approval

Contact:	ankita.lachhwani@cewit.org.in

Abstract: This contribution proposes a new use case (re-submission) for Positioning service for Network Optimization

* * * First Change * * * *

## 5.9 		Network Aspects

### 5.9.X	Network Optimization Service for better user experience through UE positioning

#### 5.9.X.1	Description

In the 6G system, UE(s) positioning service is expected to evolve from being a “supporting capability” for external applications to becoming a “strategic enabler” for network quality enhancement, intelligence and automation.The availability of precise and timely UE(s) location information can allow the network to better understand spatial and temporal dynamics in user behavior, service demand, and mobility patterns.Such insight is increasingly relevant as 6G networks are expected to operate in a data-driven, context-aware, and AI-assisted manner.

In current systems, the positioning service primarily supports:

- Provision of UE(s) location information to external clients (e.g. LCS clients or third-party applications); and

- Limited use of location information by certain internal procedures (e.g. for emergency call routing, regularatity services location-based barring).

However, the UE(s) positioning service is not envisioned as a network information source for general optimization or automation purposes.

Due to this several limitations arise in the network which are as follow:

- Lack of spatial context for network control

Optimization decisions (e.g. load balancing, scaling, or mobility management) are made by network without spatial awareness of user demand.This leads to reactive adjustments rather than predictive resource alignment. Without UE(s) location trends,the network cannot anticipate congestion, coverage imbalance, or service degradation in advance.

- Fragmented and domain-specific operation

Existing mechanisms for self-configuration, self-optimization, and self-healing are confined to specific domains (e.g. access or core) and lack spatial context. Without shared positioning data, mobility optimization across domains cannot be coordinated effectively.

The 6G system shall support a service capability that enables the use of UE(s) positioning information by the network to enhance its automation and self-organizing functionalities. To enable this,UE(s) positioning information should be integrated as a native, secure, and policy-controlled data utilization service for internal optimization.

This service provides authorized internal network components to access UE(s) positioning data(For Example but not limited to Coarse distribution of multiple UEs, distance from the radio transceivers) and metadata (For Example but not limited to accuracy, confidence, timestamp).

The service supports the following capabilities:

- Network automation and self-organization:

UE positioning information can enhance self-configuration, self-optimization, and self-healing processes by introducing spatial context. Examples include automatic neighbor relation creation, geography-specific mobility configuration, and location-aware interference mitigation. Position-enabled automation allows UE-specific or group-specific optimization, improving overall system adaptability.

- Contextual and data-driven operation:

UE(s) Positioning data adds a new dimension to network analytics, enabling AI/ML functions to better predict and optimize future states of the network. Spatial context can improve accuracy of mobility prediction, beam selection, and resource scheduling by allowing models to correlate user motion and density with performance trends.

- Intelligent infrastructure optimization:

UE location information complements radio and environmental measurements to improve the operation of intelligent infrastructures such as relays, network-controlled repeaters (NCRs), and reconfigurable intelligent surfaces (RIS).This enables dynamic path optimization and effective mitigation of coverage holes based on real-time user distribution.

By combining these capabilities, the Positioning-Assisted Network Optimization service allows the 6G system to transition from reactive network to predictive, context-aware, and self-optimizing network.This integration of positioning information within the system’s automation and AI frameworks enhances performance, efficiency, and sustainability while maintaining compliance with privacy and security requirements.

Such optimization extends and improves the system capability, better support for mobility, which enables the mobile operators to provide higher data rates and to improve the user experience.

To ensure the secured access of the UE positioning service for network optimization, user consent is required

#### 5.9.X.2 Existing features partly or fully covering the use case functionality

The 5G requirement on positioning service [14] include that the 5G network is able to request UE to provide its postioning related data. The corresponding 5G requirement is as below:

The 5G network shall be able to request the UE to provide its position-related-data on request together with the accuracy of its position triggered by an event or periodically and to request the UE to stop providing its position-related data periodically.

The UE(s) positioning service in 5G is not envisioned as a network information source for general optimization or automation purposes

Existing privacy mechanisms addresses external exposure of location data and internal utilization for AI/MLbased positioning. There is no privacy mechanism for network optimization.

#### 5.9.X.3 Potential New Requirements

[PR 5.9.X.3-1] Subject to regulation, operator(s) policies and user consent, the 6G system shall support providing UE(s) positioning information (For example but not limited to coarse spatial distribution of multiple UEs, distance from the radio transceivers) to the network for its own optimization.

[PR 5.9.X.3-2] 6G system shall provide mechanism for acquiring user consent for 6G network to request/consume the UE positioning service for its own optimization.