3GPP TSG-SA WG1 Meeting #112 	S1-254382

Dallas, TX, USA, 17-21 November 2025	 (revision of S1-254032r2, S1-254032r1, S1-254032)

Source:		NICT, Deutsche Telekom, Thales, Novamint, NEC, TNO, Sharp, Eutelsat, EDF, Viasat, ESA, SES, Airbus, Netherlands Police, Hispasat, JSAT, Philips, Terrestar, Gilat, SyncTechno Inc., EchoStar, Boost Mobile Network, Lockheed Martin, Skylo

pCR Title:		Use Case on Enhanced Multi-Access Connectivity

Draft Spec:		3GPP TR22.870 v0.4.1

Agenda Item:			8.1.5 Ubiquitous connectivity

Document for:		Approval

Contact:			Hikaru Kawasaki (h_kawasaki@nict.go.jp)

Hideshi Murai (hideshi.murai@nict.go.jp)

Abstract: We propose a new use case on enhanced multi-access connectivity.

1. Introduction

This contribution proposes a new use case on enhanced multi-access connectivity providing improved reliability and/or data rate improvements in out-of-coverage or scarcely covered areas (e.g., rural, remote, indoor) as well as in disaster-stricken areas, which are normally covered by terrestrial networks.

It should be noted that a performance analysis of equivalent scheme has been reported with two access links with different characteristic in terms of bandwidth and latency [1].

Furthermore, the multipath extension of QUIC is currently under standardization [2], and its potential applications to enhance multi-access connectivity in 6G are expected to be explored.

2. References

[1] 	ETSI TR 103 351 “Satellite Earth Stations and Systems (SES); Multi-link routing scheme in hybrid access network with heterogeneous links”

[2] 	Y. Liu, Y. Ma, Q.D. Coninck, O. Bonaventure, C. Huitema, and M. Kühlewind, “Multipath Extension for QUIC,” Internet-Draft draft-ietf-quic-multipath-15, Internet Engineering Task Force, July 2025. Work in Progress. https://datatracker.ietf.org/doc/draft-ietf-quic-multipath/15/

3. Reason for Change

Existing features in 5G partially cover the proposed use case but require enhanced multi-access functionality in 6G.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1

* * * First Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

//// skip ///

MultiSteer device (MS-device): A device supporting traffic steering and switching of user data (for different services) across multiple 3GPP access networks; It is composed of two separate UEs, including two subscriptions/SUPIs, sharing one subscription profile from the same operator.

* * * Next Change (new text) * * * *

## 8.x	Use Case on Enhanced Multi-Access Connectivity

### 8.x.1	Description

Let us consider a wireless access point (i.e. Wi-Fi) and/or base station (i.e. 6G) onboard a moving platform (e.g., trains or buses with passengers) or on a fixed building that would be served by different PLMNs (e.g., various Terrestrial Networks (2G, 4G and 5G as well as NTN). The service provided by a given TN can be often degraded. Having the possibility to steer and/or switch between different access links of different PLMNs would ensure highest service availability, reliability and performance.

The same feature would also benefit other applications such as telehealth, and remote control of robots or drones where reliability is key.

Figure 8.x.1-1 Examples of Multi-Access Connectivity

This requires

- the terminal served by the different PLMNs and connected to the wireless access point/base station on board the moving platforms, to support  different access links (TN and NTN).

- the 6G system to be able to provide a given service across access links of different PLMNs potentially with significant performance disparities (e.g., service rate, latency, etc…) through dynamic selection and switching.

Enhanced multi-access connectivity is necessary to provide ubiquitous connectivity with enhanced reliability and/or data rate performance in scarcely covered areas (e.g., rural, remote, indoor) as well as in disaster-stricken areas, which are normally covered by terrestrial networks.

The support of multi-access connectivity includes traffic flow control (including the ability to steer traffic flow) to meet performance requirements like high availability, throughput, reliability, low latency, and time jitter stability, which a single operator network may not provide due to coverage limitations, bandwidth constraints, and diverse network deployments.

### 8.x.2	Pre-conditions

The environment consists of heterogeneous access networks (including TNs and NTNs) in rural, remote, and sparsely populated areas as well as in disaster-stricken areas, which are normally covered by terrestrial networks.. The system supports access to networks from different operators where feasible, to enhance coverage. Users are authorized to access these networks, and a UE supports multi-access functionality for dynamic network selection and traffic flow control.

### 8.x.3	Service Flows

The UE benefits from multiple (access) networks composed of TNs and NTNs (potentially from different operators) mainly in rural, remote, indoor, and sparsely populated areas as well as in disaster-stricken areas, which are normally covered by terrestrial networks, using multi-access functionality. The multi-access functionality enables traffic flow control considering performance disparities to meet performance requirements such as network availability, throughput, reliability, low latency, time jitter stability and others. For example:

- Passengers onboard trains and/or buses would benefit from a reliable connectivity when accessing internet.

- In a telehealth scenario, the multi-access connectivity provides high reliability, as well as steering or switching to a link with higher bandwidth to support the high-quality video communication for remote healthcare.

- In the remote control of robots/drones, the multi-access connectivity provides the necessary high reliability.

- In a heavy data usage service scenario (e.g., streaming or gaming), the multi-access connectivity (e.g., NTN/TN) provides high data rate services and perceived low latency for best user experience.

### 8.x.4	Post-conditions

The UE achieves ubiquitous connectivity in scarcely covered areas (e.g., rural, remote, indoor) as well as in disaster-stricken areas, which are normally covered by terrestrial networks, through enhanced multi-access functionality. The functionality includes traffic flow control via TNs and NTNs, which may belong to different operators. This ensures the target performance is met for parameters such as network availability, throughput, reliability, low latency, time jitter stability - important for applications like telehealth or remote control of robots/drones.

### 8.x.5	Existing features partly or fully covering the use case functionality

The 5G supports multi-access functionality, as defined in TS 22.261 [14] Clause 6.3.2 (multi-access technologies requirements) and Clause 6.50.2 (Traffic steering and switching over two 3GPP access networks).

NOTE: The DualSteer feature, Clause 6.50.2 Traffic steering and switching over two 3GPP access networks, in previous release aimed to enable multi-access support between two 3GPP access networks. However, the feature has not progressed in Stage 2, and hence it is expected that it will be entirely removed from the SA1 specifications as a part of Release 19 alignment process.

DualSteer device (Rel-19 definition): A device supporting traffic steering and switching of user data (for different services) across two 3GPP access networks; it can be a single UE, in case of non-simultaneous data transmission over the two networks, or two separate UEs in case of simultaneous data transmission over the two networks.

### 8.x.6	Potential New Requirements needed to support the use case

[PR8.x.6-1] Subject to HPLMN policy, the 6G system shall support mechanisms for a MS-device to register simultaneously to multiple access links of different PLMNs, including TNs and/or NTNs, assuming a business/roaming agreement between network operators.

[PR8.x.6-2] Subject to HPLMN policy, the 6G system shall be able to support mechanisms to enable traffic steering and /or seamless switching (i.e., without noticeable service interruption) of the MS-device’s data (for different services) between the registered 3GPP access links of different PLMNs.

[PR8.x.6-3] The 6G system shall support means for the HPLMN to provide configuration information for a MS-device to select one or multiple additional networks, including a list of allowed additional networks in priority order. The configuration information may include optional restrictions (e.g. service area, time).

NOTE 1: The additional network is the network (PLMN/access technology combination) that a MS-device may select in addition to the initial network (i.e. an existing network connection).

[PR8.x.6-4] The MS-device shall only select an additional network which is part of the list of allowed additional networks and which is different from the current network which was initially selected. If the MS-device changes the initial network (i.e. moves to a new network), this may result in new configuration information provided to the UE.

NOTE 2:	The additional network(s) might be from the same or a different country than the initial network.

[PR8.x.6-5] The HPLMN shall be able to configure whether the configuration information shall remain valid when the initial network changes and no new configuration is received.

End of CHANGES