3GPP TSG-SA WG1 Meeting #112	S1-254422

17-21 November 2025, Dallas, Texas, USA	(revision of S1-254017r2)

Source:	Ericsson, Itron, AT&T, Verizon, Sony, Nokia, EDF

pCR Title:	New use case for Gas and Water Metering

Draft Spec:	3GPP TR 22.870v0.4.1

Agenda item:	8.1.7 Massive Communication

Document for:	Agreement

Contact:	Daniel.lonnblad at ericsson.com

Abstract: This contribution complements the electricity metering use case in 10.3 and ensures full coverage of multi-utility smart metering in 6G Massive Communication objectives.

The use case introduces new KPI and service requirements to support next-generation gas and water metering as part of 6G Massive Communications. These meters are predominantly battery-powered, require 20+ year battery life, and will form an integral part of utility-critical infrastructure monitoring in both urban and rural environments.  The focus for this category of products is low power and low complexity with the additions of achieving reliable communications in indoor locations or locations with low coverage as well as with the ability to be immediately reached from the network in case of urgent messages from an application server.

## 10.X	Use Case on Gas and Water Metering Infrastructure

### 10.X.1	Description

Gas and water meters are foundational infrastructure assets deployed globally across residential, commercial, and industrial premises. Unlike electricity meters, which often benefit from power availability, gas and water meters are typically battery-powered and placed in basements, pits, vaults, or other difficult-to-access and RF-challenged environments. This creates demanding requirements on coverage, energy efficiency, and device longevity.

The next generation of Advanced Metering Infrastructure (AMI 2.0) will integrate gas and water metering into broader smart utility platforms, enabling not only billing and leak detection, but also dynamic control, demand-response water conservation strategies, outage and tamper detection, and operational efficiencies. These systems rely today on 4G LPWA communications, but see increasing demands for low and predictable latency, while still supporting bidirectional communications in often challenging coverage environments and with constrained energy budgets.

Future gas and water meters are expected to integrate adjunct sensors that extend their functionality beyond basic consumption measurement. Examples include acoustic leak sensors, pressure-transient monitors, soil or moisture probes, and corrosion or strain indicators mounted as mezzanine modules on the meter. These sensors can generate richer datasets during abnormal events (e.g., transient pressure spikes, acoustic leak signatures, or early indicators of pipe failure). When such events occur, the meter may need to upload short, high-integrity data bursts to backend systems for AI-based analysis and prediction. This evolution increases the required uplink data volume and the timeliness of certain transmissions, while the device must still operate under strict battery constraints.

To ensure practical feasibility, gas and water meters typically operate with a total energy budget equivalent to 16 Ah over a 20-year lifetime of which 12 Ah is set aside for communications. For this reason, they often utilize a 10-minute eDRX cycle and report consumption data once every 4 hours, with occasional device or network-triggered alerts.

6G shall enable both massive deployment scale and unprecedented energy performance, while allowing from day 1 to support network-initiated traffic with sub-1 minute latency.

Two new technical capabilities are required beyond what is currently offered in 4G and 5G IoT:

1. Long device battery life for devices in challenging coverage: Current IoT technology typically utilize repetitions to reach coverage extensions for downlink traffic. This has its issues since both the energy consumption and the latency increases. The 6G system needs to balance the technology to achieve deep coverage together with low energy consumption and low latency.

2. Fast Network-Initiated Alarm Communication for power restricted devices: Use cases, such as remote gas shut-off in the event of a leak, require the ability for the network to initiate communication with battery-powered devices within less than one minute, even for devices that are configured for 20 years of battery lifetime. Existing LPWA solutions typically need a long sleep time to manage the 20 years lifetime on a battery. The concept of Wake up receivers are only defined for short range communication and is not possible to use beyond 1km

### 10.X.2	Pre-conditions

Company AquaGrid is a multi-utility operator deploying next-generation gas and water meters across the region of Flowville. Devices needs to operate on battery for 20+ years and:.

- Transmit meter readings every 4 hours

- Support over-the-air firmware updates at least once per year.

- Be installable in underground or deep-basement environments, often requiring strong indoor and underground coverage.

- Respond to network-initiated communication within 1 minute in urgent scenarios.

### 10.X.3	Service Flows

- Operator BlueCom seeks to offer utility meters not only for electricity but also water and gas. Water and Gas meters are typically battery powered but may not need the same short latency as when monitoring and controlling the electric grid. The latency for uplink and downlink traffic may also differ. BlueCom sees the only option to achieve this by deploying a 6G system. Typical parameters needed are:

  - Regular flow: periodic (e.g., 4-hour) reporting from gas/water meters.

  - Occasional on-demand reads or device or network-triggered alarms (e.g., leak detected).

  - Over-the-air software upgrade annually

  - Urgent communication for gas shutoff triggered by backend with low latency.

- Company AquaGrid offers the possibility to connect all water and gas meters with BlueCom’s 6G network.

### 10.X.4	Post-conditions

AquaGrid achieves massive meter deployment with the 6G network. Meters perform securely and reliably for 20 years. Leak events and remote disconnects are handled in a timely, secure fashion.

### 10.X.5	Existing features partly or fully covering the use case functionality

In TS 22.261 [14] clause 6.4 Resource efficiency states some general requirements for IoT devices.

The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

The 5G system shall minimize control and user plane resource usage for stationary UEs (e.g. lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of continuous uplink data that requires both high data rate (e.g. 10 Mbit/s) and very low end-to-end latency (e.g. 1-10 ms).

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g. 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	type of mobility support;

-	communication pattern (e.g. send-only, frequent or infrequent);

-	characteristics of payload (e.g. small or large size data payload);

-	characteristics of application (e.g. provisioning operation, normal data transfer);

-	UE location;

-	timing pattern of data transfer (e.g. real time or non-delay sensitive).

The 5G system shall efficiently support service discovery mechanisms where UEs can discover, subject to access rights:

-	status of other UEs (e.g. sound on/off);

-	capabilities of other UEs (e.g. the UE is a relay UE) and/or;

-	services provided by other UEs (e.g. the UE is a colour printer).

The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g. consolidating data transmissions to 1 larger rather than many smaller), when applicable (e.g. providing service in an area subject to power outages).

The 5G system shall support small form factor UEs with single antenna.

NOTE:	Small form factor UEs are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wavelength.

In TS 22.261 [14] clause 6.15 Resource efficiency states some general requirements for IoT devices.

The 5G system shall support UEs using small rechargeable and single coin cell batteries (e.g. considering impact on maximum pulse and continuous current)In TS 22.261[14] clause 8 Security, security requirement for IoT devices is listed.

In TS 22.104 [64] Annex A4.7 Advanced metering following proposed requirement can be found.

Table 10.2.5-1: Communication KPI for advanced metering (from TS 22.104 [64] Table A.4.7-1)

| Use case# | Characteristic parameter |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bit rate: user experienced data rate | Message size [byte] |  | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |
| 1 – Advanced Metering | > 99.99 |  | Accuracy fee control: < 100 (NOTE 1); General information data collection: < 3000 | UL: < 2 M DL: < 1 M | – |  | – | – | stationary | < 10 000/km2 (NOTE 2) | – |
| NOTE 1: One-way delay from 5G IoT device to backend system. The distance between the two is below 40 km (city range). NOTE 2: It is the typical connection density in today city environment. With the evolution from centralised meters to socket meters in the home, the connection density is expected to increase 5 to 10 times. |  |  |  |  |  |  |  |  |  |  |  |

### 10.X.6	Potential New Requirements needed to support the use case

[PR 10.X.6-1] The 6G system shall support communication services with the KPI's according to Table 10.X.6-1.

[PR 10.X.6-2] The 6G system shall support energy-efficient and reliable communication for battery-powered devices deployed in underground, pit-installed, or RF-challenged locations, with at least similar coverage as what is supported for practical deployments of existing IoT technology while keeping the low energy consumption and short latency.

[PR 10.X.6-3] The 6G system shall support network-initiated communication with battery-powered devices in urgent scenarios, with the ability to reach the device and complete the exchange timely (e.g. within a minute) even when the device is in an energy-saving state.

Table 10.X.6-1: KPIs for Massive Communication  Gas and Water meter

| Profile |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link (Mbit/s) | Bit rate up link (Mbit/s) | End-to-end latency: maximum (ms) | Payload size (Kbyte ) | # of UEs connection | Communication service availability: target value | Transfer Interval |
| Smart Grid monitor and control (NOTE 3) | Peak 5 NOTE 4 | Peak 5 NOTE 4 | DL <1 min (NOTE 1) UL<1s (NOTE 2) | 0.1 - 2 | 10 000/km2 | > 99.9% | 4 hours |
| NOTE 1: The 6G system need to enable network-initiated communication with battery-powered devices configured to support the 20-year battery life. These communications may occur very infrequently. NOTE 2: For device-initiated uplink transmissions, the reporting interval — and therefore the effective latency — needs be configurable within a range from seconds to minutes, with a default that balances battery life and responsiveness NOTE 3: These performance requirements aim energy-efficient transmissions performed using a device powered with battery capacity of < 12Ah that can last at least 20years without recharging. NOTE 4: Lessons from IoT deployments show that higher peak data rates reduce the total radio-on time, which directly reduces battery consumption. A peak rate of 5 Mbit/s UL/DL enables occasional large, event-driven bursts and configuration updates for gas/water meters and adjunct sensors to complete within a single short wake window, thereby preserving long battery life. |  |  |  |  |  |  |  |