3GPP TSG-SA WG1 Meeting #112	S1-254078r1

Dallas, USA, 17-21 November 2025	(revision of S1-25xxxx)

Source:	Nokia, InterDigital

pCR Title:	Update efficient data collection use case 5.9.2

Draft Spec:	3GPP TR 22.870 v.0.4.1

Agenda Item:	8.1.2

Document for:	Approval

Contact:	Laurent-Walter Goix

Abstract: This contribution proposes to add new requirements related to efficient data collection in the following areas:

- Optimization of energy consumption & carbon when doing data collection (to contribute to the target of energy saving on the network and the UE)

- Discovery of data available for collection at data providers (e.g. at the UE)

1. Introduction

The use case introduces data collection in 6G, emphasizing the need for efficiency. This updates ocuses on 2 aspects of efficiency in data collection: 1) energy on the UE / in the network, 2) usage of radio resources

2. Reason for Change

Two new requirements are proposed to further emphasize the need for efficiency in the process of data collection (and processing etc)

- Energy saving is indicated in the use case as one of the target scenarios that the efficient data collection and consumption mechanism aims to serve, similar as sensing, AI and etc. We believe that data collection will trigger more energy/carbon consumption de facto on the network, and so the *efficient* design of the system for data collection also goes via being energy efficient, for example to optimize the amount of data to be collected or processed, so that the amount of energy saved thanks to the collected data is greater than the energy consumed to collect it. Data collection will impact the energy consumed in the UE (eg if always collecting a lot of data even if not useful for the network), and in the various network functions/elements by also occupying resources (communication & compute) where these data may not be needed in such quantity, granularity, accuracy etc

- The use case, and some requirements, already refer to data producers and data consumers exchanging data within the 6G system. We believe that the 6G system shall support means to discover the available data at producers to bring “efficiency” in such transfer of data. This is particularly true with respect to the UE for which the data collection process uses radio resources, which could hence be optimized.

R1

- Added InterDigital as supporting company

- PR8/x: clarified the purpose [Huawei], remove reference to data consumer (responsibility of the 6G system to allow to discover) [ZTE]

- PR9/y: rewording & added NOTE to clarify that this is not about measuring/quantifying the difference/savings [vivo, Huawei, ZTE]

- Renumbered PRs as x/y [TMO US]

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.4.1

* * * First Change * * * *

### 5.9.2	Efficient data collection and consumption for 6G system

#### 5.9.2.1	Description

To better serve the users and manage the network, as well as provide non-connectivity service (e.g. Location Services (LCS)), 3GPP system needs to conduct collection of data associated with the network and services provided by the 3GPP system. The data collection in the 5GA network for network/service management and non-connectivity service is defined for specific use cases, resulting in varying solutions across different use cases, different layers and different domains. Here is a brief summary of different data collection work done in RAN/SA2/SA4 and corresponding security mechanism in SA3:

-	In RAN domain, data collection from UE is based on MDT (Minimization of drive testing) method as defined in TS 37.320 [237] and TS 38.331 [238]. The user consent of UE location data collection for MDT is defined in TS 32.422 [239].

-	For data collection for AI/ML in core network, the data collection feature permits the Network Data Analytics Function (NWDAF) to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, NSACF, GMLC, and Application Function (AF), OAM, etc.), as a basis of the computation of network analytics as defined in TS 23.288 [114]. The operator can leverage these network analytics to optimize the management of network operations. For instance, as outlined in clause 6.22 of TS 23.288 [114], the operator can collect network data (such as OAM data and CP signalling) to detect or predict a signalling storm and use the analytics to mitigate or prevent its impact. The data collection is performed via SBI between CN NFs, DCCF (Data Collection Coordination Function) is introduced as logical functionality to coordinate data collection in CN, and Analytics Data Repository Function (ADRF) is defined to store the collected data. The user consent of UE data collection for AI/ML is defined in TS 33.501 [250].

-	For UE positioning, the input data is collected from UE/RAN to CN via CP/UP path as the basis of the computation of UE location as defined in TS 23.273 [240]. The privacy check of UE location collection is also defined in TS 23.273 [240].

-	To collect data and to expose data between CN domain and AF domain, the network exposure mechanism in core network has been defined in clause 5.20, TS 23.501 [140] and clause 4.15, TS 23.502 [30], where the Network Exposure Function (NEF) is used as the termination node to isolate both sides. Especially for edge, exposure of UE data is defined in TS 33.558 [241].

-	If the data collection is triggered by an over the top (OTT) server in application layer from UE, the UP path may be used with the assistance of DCAF, as defined in TS 26.531 [242].

The aforementioned mechanisms are defined for different network functions to support data collection in different scenarios, leading to high standardization overhead. There is lack of coordination between these network functions, leading to isolated data collection and duplicated data collection. Meanwhile, the data transmission based on the control plane are not suitable for transmitting large amounts of data, leading to inefficient data transmission. Therefore, the data collection and control mechanism need to be improved in 6G.

Table 5.9.2.1-1 lists several potential use cases for data collection in 6G, and illustrates the corresponding data type, data provider, data consumer and data volumes for each use case. Take AI/ML data as example, there is a large amount of data types defined in 3GPP apart from those non-standardized ones, and the scale of data volume is usually large (from 10k to 100M).

Table 5.9.2.1-1: Heterogeneous 6G System Data

| Use case category | Data types | Data provider | Data Consumer | Data volume |
| --- | --- | --- | --- | --- |
| Sensing data collection/sensing result exposure (e.g. measurement) | Sensing measurement data requires further study and discussion. The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. | UE, RAN | Sensing Function | Related to use cases: e.g. data rate of measurement is about 10Mbps for objects creating hazards on roads |
| AI/ML data collection / analytics exposure (e.g. AI/ML for air interface, AI/ML for core network) | - For RAN data collection for Air interface, see TR 38.843 [243] - For core network data collection, see the data collection framework in clause 6.2, TS 23.288 [114], and the detailed data type are defined from clause 6.3 to clause 6.21, TS 23.288 [114]. | CN, RAN, UE, AF | CN, RAN, UE | Typically, the internal data collection for AI model training yields tens of thousands of samples. Related to AI model: e.g. the data volume is about 10k bytes to 100M bytes for AI model of beam management |
| Energy Efficiency (EE information exposure to AF) | still under discussion in SA2 R19 EnergySys WID, see SP-250069. | CN, OAM | EIF (Energy Information Function) | Related to use cases: e.g. could be similar to data volume for charging |
| Positioning, | See clause 6, TS 23.273 [240] | UE, RAN | LMF, UE | Related to use cases |
| Network Exposure in core network | See clause 5. 20, TS 23.501 [140], and clause 4.15, TS 23.502 [30] | 5GC NF, AF | 5GC NF, AF | Related to use cases |

All the above use cases for data collection and control share the following commonality:

- The data collection and control mechanisms need to support procedures such as data collection, data storage, data exposure and data deletion.

- The collected data could be requested or consumed by multiple data consumers within 3GPP system to support multiple services; for example, a single spatiotemporal environmental dataset can be concurrently utilized by multiple services, including IoT-enabled pervasive sensing service, latency-sensitive XR applications requiring <10ms motion-to-photon delays, and the Intelligent Transportation System (ITS) with centimetre-level geolocation precision.

- The upper bond of the collected data volume is very high, e.g. on the order of 100 M byte.

- Latency requirements for data collection and transmission exhibit variations across diverse use cases.

- The collected data transmission path between different entities within 3GPP system can vary widely, encompassing scenarios such as UE to RAN node，UE to CN node, RAN node to CN node, and CN node to CN node.

- All these use cases that involve collecting UE related data need to support security, privacy protection and user consent. In addition, security requirements need to consider different data types.

- All these use cases that involve data exposure need to be subject to operator policy and user consent.

Figure 5.9.2.1-1: Use case specific data mechanisms in 5G  common data mechanism in 6G

As shown in Figure 5.9.2.1-1, use case specific mechanisms are defined for each service scenario in 5G, leading to system complexity and inefficient data collection. As 6G networks integrate advanced capabilities progressively, such as AI, enhanced computing power, multi-dimensional sensing and digital twin network, they will need to process massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Consequently, 6G system needs to support an efficient data collection and control mechanism capable of addressing both universal requirements and scenario-specific demands across heterogeneous use cases such as AI/ML, sensing, XR service, digital twin network and energy saving objectives. The efficient data collection and control mechanism needs to support various data providers and consumers, which maintains service-agnostic characteristics while embedding extensibility for emerging services. In parallel, the energy saving objectives of the system also call for efficiency when collecting data, for example to avoid unnecessary drain of UE battery.

In conclusion, the 6G system needs to support an efficient data collection and control mechanism, which includes the following characteristics:

-	Common data framework that can serve the need of multiple use cases, to improve data collection efficiency and avoid duplication data collection;

-	Support data discovery and collection from various data providers and data exposure to various data consumers;

-	Support user consent for data collection; and

-	Support data exposure subject to operator policy and user consent.

#### 5.9.2.2	Potential New Requirements

Editor’s Note: user consent in this section is FFS.

[PR 5.9.2.2-1] The 6G system shall support mechanisms for 6G System Data collection and consumption minimizing the impact to 6G services.

[PR 5.9.2.2-2] Subject to operator’s policy, regulation and user consent, the 6G system shall support processing of 6G System Data.

NOTE:	Examples of data processing are use case dependent, e.g. data fusion, data anonymization and data analysis.

[PR 5.9.2.2-3] Subject to user consent, regulation and operator's policy, the 6G system shall support secure means to expose 6G System Data to authorized trusted third-party, authorized network function or authorized UE.

[PR 5.9.2.2-4] The 6G system shall support security and privacy protection of the 6G System Data.

[PR 5.9.2.2-5] The 6G system shall be able to provide charging and accounting mechanisms for 6G System Data.

[PR 5.9.2.2-6] Subject to regulation, user consent and operator policy, the 6G system shall support storage and retrieval of 6G System Data.

[PR 5.9.2.2-7] Subject to regulation and operator policy, the 6G system shall support transfer 6G System Data between different data providers and data consumers within the 6G system.

[PR 5.9.2.2-x] The 6G system shall support mechanisms for the discovery of 6G System Data available from data providers (e.g. at the UE) in order to optimize network and resource usage, subject to authorization.

[PR 5.9.2.2-y] The 6G system shall support mechanisms utilizing available information on network and UE energy consumption, carbon emissions and on the involved data providers and data consumers to optimize data collection, processing and consumption.

NOTE:	It is not assumed that the 6G system needs to measure or estimate the energy consumption or carbon emissions related to data collection or processing to quantify or compare saving. Mechanisms to limit the amount or frequency of data collected, to use low-carbon computing resources for data processing etc are examples of such optimizations without the need for quantification.