3GPP TS 22.261 V0.2.0 (2016-11)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Service requirements for next generation new services and markets;

Stage 1

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword, …]>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Overview	8

5	High-level requirements	9

5.1	Migration to 5G	9

5.1.1 	Description	9

5.1.2	Requirements	9

5.1.2.1	Interworking between 5G systems	9

5.1.2.2	Legacy Service Support	9

6	Basic capabilities	9

6.1	Network slicing	9

6.1.1	Description	9

6.1.2	Requirements	10

6.2	Diverse mobility management	10

6.2.1	Description	10

6.2.2	General requirements	11

6.2.3	Service continuity requirements	11

6.3	Multiple access technologies	11

6.3.1	Description	11

6.3.2	Requirements	11

6.3.2.1	General	11

6.3.2.2	E-UTRA access	12

6.3.2.3	Satellite access	12

6.3.2.4	Fixed broadband	12

6.4	Resource Efficiency	12

6.4.1	Description	12

6.4.2	Requirements	13

6.4.2.1	General	13

6.4.2.2	Efficient bulk operations for IoT	14

6.4.2.3	Efficient management for IoT	14

6.4.2.4	Efficient control plane	14

6.5	Efficient User Plane	14

6.5.1	Description	14

6.5.2	Requirements	14

6.6	Efficient content delivery	15

6.6.1	Description	15

6.6.2	Requirements	15

6.7	 Priority, QoS and Policy Control	16

6.7.1	 Description	16

6.7.2	 Requirements	16

6.8	Dynamic Policy Control	17

6.8.1	Description	17

6.8.2	Requirements	17

6.9	Connectivity models	17

6.9.1	Description	17

6.9.2	Requirements	17

6.10	Network Capability Exposure	18

6.10.1	Description	18

6.10.2	Requirements	18

6.11	Context aware network	18

6.11.1	Description	18

6.11.2	Requirements	18

6.12	Self backhaul	19

6.12.1	Description	19

6.12.2	Requirements	19

6.13	Flexible Broadcast/Multicast Service	19

6.13.1	Description	19

6.13.2	Requirements	19

6.14	Subscription aspects	20

6.14.1	Description	20

6.14.2	Requirements	20

6.15	Energy Efficiency	20

6.15.1	Description	20

6.15.2	Requirements	20

6.16	Markets requiring minimal service levels	21

6.16.1	Description	21

6.16.2	Requirements	21

6.17	Extreme long range coverage in low density areas	21

6.17.1	Description	21

6.17.2	Requirements	21

6.18	eV2X aspects	22

6.18.1	Description	22

6.18.2	Requirements	22

7	Performance requirements	22

7.1	High data rates and traffic densities	22

7.2	Low latency and high reliability	23

7.2.1	Overview	24

7.2.2	Services and KPIs	24

7.2.3	Other requirements	24

7.3	eV2X performance requirements	25

8	Security	25

8.1	Description	25

8.2	General	25

8.3	Authentication	25

8.4	Authorization	25

8.5	Identity management	26

8.6	Regulatory	26

8.7	Fraud protection	26

8.8	Resource efficiency	26

9	Charging aspects	26

Annex <A> (normative): <Normative annex title>	28

Annex <B> (informative): <Informative annex title>	28

Annex <X> (informative): Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The continuous need to support different kinds of UEs (e.g., for IoT), services, and technologies is driving the technology revolution to a high performance and highly efficient 3GPP system. Significant drivers include the Internet of Things (IoT), virtual reality, industrial control, ubiquitous on-demand coverage, as well as the opportunity to meet customized market needs. All of these drivers require enhancements to the devices, services, and technologies well established by 3GPP. The key objective with the 5G system is to be able to support new deployment scenarios to address diverse market segments.

This document compiles requirements that define a 5G system.

The 5G System is characterised, for example, by:

- 	Support for multiple access technologies

- 	Scalable and customizable network

-	Advanced (in comparison with 4G) KPIs (e.g., availability, latency, reliability, user experienced data rates, area traffic capacity)

- 	Flexibility and programmability (e.g., network slicing, diverse mobility management, Network Function Virtualization)

- 	Resource efficiency (both user plane and control plane)

-	Seamless mobility in densely populated and heterogeneous environment

- 	Support for all kinds of real time and non-real time multimedia services and applications with advanced (in comparison with 4G) QoE of mobile users

# 1		Scope

The present document describes the service and operational requirements for a 5G system.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN 5G White Paper v1.0, February 2015.

[3]	3GPP TR 22.891: "Feasibility Study on New Services and Markets Technology Enablers; Stage 1".

[4]	3GPP TR 22.864: "Feasibility Study on New Services and Markets Technology Enablers - Network Operation; Stage 1".

[5]	3GPP TR 22.863: "Feasibility Study on New Services and Markets Technology Enablers - Enhanced Mobile Broadband; Stage 1".

[6]	3GPP TR 22.862: "Feasibility Study on New Services and Markets Technology Enablers - Critical Communications; Stage 1".

[7]	3GPP TR 22.861: "Feasibility Study on New Services and Markets Technology Enablers for Massive Internet of Things; Stage 1".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Editor's Note: The definitions of active communication and service continuity propose changes to 21.905 and need to be propagated once agreed in SA1. To add to 21.905 we need to consider another note for service continuity, e.g.,

NOTE x: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.

3GPP connection: The connection between a device and a 3GPP network.

Active communication:  A UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g., PDP contexts, active PDN connections).

Availability (%): Percentage value of the amount of time the 3GPP system is delivering services divided by the amount of time it is expected to deliver services in a specific area.

Direct 3GPP connection: One mode of 3GPP connection, where there is no relay UE between the device and the 3GPP network.

Direct device connection: The connection between two devices without any network entity in the middle.

Hosted Service: A service containing the operator’s own application(s) and/or trusted 3rd party application(s) in the Service Hosting Environment, which can be accessed by the user.

Indirect 3GPP connection: One mode of 3GPP connection, where there is a relay UE between the device and the 3GPP network.

Network slice: A set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

Reliability (%): Percentage value of the amount of sent network layer packets successfully delivered to a given node within the time constraint required by the targeted service, divided by the total number of sent network layer packets.

Satellite access: Direct connectivity between the user terminal and the satellite.

Service Continuity: The uninterrupted user experience of a service that is using an active communication when a UE undergoes an access change without, as far as possible, the user noticing the change.

NOTE 1: 	In particular Service Continuity encompasses the possibility that after a change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the change.

NOTE 2: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.

Service Hosting Environment: The environment, located inside of 3GPP network and fully controlled by the operator, where Hosted Services are offered from.

Tactile internet: System where a human controls real or virtual objects by tactile input, and receives tactile, audio, or visual feedback of his actions. The feedback has to arrive fast enough to give the impression of zero-latency control.

Wireless Backhaul: Provides interconnection link between 3GPP network nodes and/or 3GPP transport network using 3GPP radio access technology.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3D	3 Dimensional

5G	5th Generation

E2E	End to End

eFMSS	Enhancement to Flexible Mobile Service Steering

FMSS	Flexible Mobile Service Steering

ICP	Internet Content Provider

IOPS	Isolated E-UTRAN Operation for Public Safety

IoT	Internet of Things

KPI	Key Performance Indicator

MBB	Mobile BroadBand

MIoT	Massive Internet of Things

MNO	Mobile Network Operator

MPS	Multimedia Priority Service

MVNO	Mobile Virtual Network Operator

NGMN	Next Generation Mobile Networks

QoE	Quality of Experience

SEES	Service Exposure and Enablement Support

UAV	Unmanned Aerial Vehicle

UHD	Ultra High Definition

VR	Virtual Reality

# 4	Overview

Unlike previous 3GPP systems that attempted to provide a 'one size fits all' system, the 5G system is expected to be able to provide optimized support for a variety of different services, different traffic loads, and different end user communities. Various industry white papers, most notably, the NGMN 5G White Paper [2], describe a multi-faceted 5G system capable of simultaneously supporting multiple specific combinations of reliability, latency, throughput, location accuracy, and availability. This technology revolution is achievable with the introduction of new technologies, both in access and the core, such as flexible, scalable assignment of network resources. In addition to increased flexibility and optimization, a 5G system needs to support stringent KPIs for latency, reliability, and throughput. Enhancements in the air interface contribute to meeting these KPIs as do enhancements in the core network, such as network slicing, in-network caching and hosted services closer to the end points.

As outlined in TR 22.891 [3], a 5G system also supports new business models such as those for IoT and enterprise managed networks. Drivers for the 5G KPIs include support of use cases such as drone control, augmented reality, and factory automation. Network flexibility enhancements support self-contained enterprise networks, installed and maintained by network operators while being managed by the enterprise. Enhanced connection modes and evolved security facilitate support of massive IoT, expected to include tens of millions of devices sending and receiving data over the 3GPP 5G network.

Flexible network operations are the mainstay of the 5G system. The capabilities to provide this flexibility have been described in TR 22.864 [4], including network slicing, network capability exposure, scalability, and diverse mobility. Other network operations requirements address the necessary control and data plane resource efficiencies, as well as network configurations that optimize service delivery by minimizing routing between end users and application servers. Enhanced charging and security mechanisms handle new types of devices connecting to the network in different ways.

MBB enhancements have been identified in TR 22.863 [5] to meet a number of new KPIs. These pertain to high data rates, high density, high user mobility, highly variable data rates, and deployment and coverage. High data rates are driven by the increasing use of data for services such as streaming (e.g., video, music, and user generated content) and interactive applications (e.g., augmented reality). These services come with stringent requirements for user experienced data rates (including downlink and uplink) as well as associated requirements pertaining to latency to meet service performance expectations. Additionally, increased coverage in densely populated areas such as sports arenas, urban areas, and transportation hubs has become essential for pedestrians and users in urban vehicles. New KPIs on density enable both the transport of high volumes of data traffic per area (traffic density) and transport of data for a high number of connections (device density or connection density). Many 5G devices are expected to support a variety of services which exchange either a very large (e.g., streaming video) or very small amount of data. The 5G system will handle this variability in a resource efficient manner. All of these cases introduce new deployment requirements for indoor and outdoor, dense environment, local area connectivity, wide area connectivity, and UEs travelling at high speeds.

Editor's Note: 5G devices can be changed if we agree to a new definition for UE.

Another aspect of 5G KPIs includes requirements for various combinations of latency and reliability, as well as higher accuracy for positioning. These KPIs, as identified in TR 22.862 [6], are driven by support for both commercial and public safety services. On the commercial side, new use cases are expected for factory automation, industrial process automation, drone and other UAV control, augmented reality, and more. Services such as UAV controls will require more precise positioning information that includes altitude, speed, and direction, in addition to horizontal coordinates.

Support for MIoT brings many new requirements in addition to those for the enhanced KPIs, as have been identified in TR 22.861 [7]. The expansion of connected things introduces a need for significant improvements in resource efficiency in all system components (e.g., devices, radio, access network, core network).

# 5	High-level requirements

## 5.1	Migration to 5G

### 5.1.1 	Description

The 5G system supports most of the existing EPS services, in addition to many new services. The existing EPS services may be accessed using the new 5G access technologies even where the EPS specifications might indicate E-UTRA(N) only. Only new or changed service requirements for new or changed services are specified in this TS. The few EPS capabilities that are not supported by the 5G system are identified in clause 5.1.2.2 below.

### 5.1.2	Requirements

#### 5.1.2.1	Interworking between 5G systems

The 5G system shall provide support for a device with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the device's 5G Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming device, (e.g., based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane traffic over the partnership network, subject to a bilateral agreement between the operators being in place.

#### 5.1.2.2	Legacy Service Support

The 5G system shall support all EPS capabilities (e.g., from TSs 22.011, 22.101, 22.278, 22.185, 22.071, 22.115, 22.153, 22.173) with the following exceptions:

-	CS voice service continuity and/or fallback to GERAN or UTRAN,

-	Seamless handover between the 5G RAN and GERAN or UTRAN, and

-	Access to a 5G core network via GERAN or UTRAN.

Editor's Note: Device can be changed to UE if we can agree on a common definition.

# 6	Basic capabilities

## 6.1	Network slicing

### 6.1.1	Description

Network slicing allows the operator to provide customised networks. For example, there can be different requirements on functionality (e.g., charging, policy control, security, and mobility), differences in performance requirements (e.g., latency, mobility, and data rates), or they can serve only specific users (e.g., corporate customers, roamers, or hosting an MVNO).

A network slice can provide the functionality of a complete network, including radio access network functions and core network functions (e.g., potentially from different vendors). One network can support one or several network slices.

### 6.1.2	Requirements

The 5G system shall allow the operator to create, modify, and delete a network slice.

The 5G system shall allow the operator to define and update the set of services supported in a network slice.

The 5G system shall allow the operator to assign a device to a network slice based on subscription, device type, and services provided by the network slice.

The 5G system shall allow the operator to configure the information which associates a device or a service to a network slice.

The 5G system shall allow the operator to assign a device to a network slice, to move a device from one network slice to another, and to remove a device from a network slice.

The 5G system shall support a mechanism for the VPLMN to assign a device to a network slice with the needed services and authorised by the HPLMN, or to a default network slice.

The 5G system shall enable a device to be simultaneously assigned to and access services from more than one network slice of one operator.

Traffic and services in one network slice shall have no impact on traffic and services in other network slices in the same network.

Creation, modification, and deletion of a network slice shall have no or minimal impact on traffic and services in the same network.

The 5G system shall support the adaptation of capacity, i.e. elasticity of capacity of a network slice.

The 5G system shall ensure that elasticity of capacity of one slice has no impact on services provided by other network slices.

The 5G system shall support means by which the operator can add and remove network functions to the network such that they can be used in a network slice.

The 5G system shall support providing connectivity to home and roaming users in the same network slice.

Editor's Note: Use of "device type" needs to be clarified.

## 6.2	Diverse mobility management

### 6.2.1	Description

A key feature of 5G is support for devices with different mobility management needs. 5G will support devices with a range of mobility management needs, including devices that are

-	Stationary during their entire usable life (e.g., sensors embedded in infrastructure),

-	Stationary during active periods, but nomadic between activations (e.g., fixed access, asset tracking),

-	Mobile within a constrained and well-defined space (e.g., in a factory), and

-	Full mobility.

Moreover, some applications require the network to ensure seamless mobility of the UE so that mobility is hidden from the application layer to avoid interruptions in service delivery while other applications have application specific means to ensure service continuity. But these other applications may still require the network to minimize interruption time to ensure that their application-specific means to ensure service continuity work effectively.

With the ever-increasing multimedia broadband data volumes, it is also important to enable the offloading of IP traffic from the 5G network onto traditional IP routing networks via an IP anchor node close to the network edge. As the UE moves, changing the IP anchor node may be needed in order to reduce the traffic load in the system, reduce end-to-end latency and provide a better user experience.

The flexible nature of a 5G system will support different mobility management methods that minimize signalling overhead and optimize access for these different types of devices.

### 6.2.2	General requirements

The 5G network shall allow operators to optimize network behaviour (e.g., mobility management support) based on the mobility patterns (e.g., stationary, nomadic, spatially restricted mobility, full mobility) of a device or group of devices.

The 5G system shall enable operators to specify and modify the types of mobility support provided for a device.

The 5G system shall optimize mobility management support for a device or group of devices that use only mobile originated communications.

### 6.2.3	Service continuity requirements

The 5G system shall enable packet loss to be minimized during inter- and/or intra- access technology changes for some or all connections associated with a device.

For applications that require a constant IP address, the 5G system shall enable maintaining the IP address assigned to a device when moving across different cells and access technologies for connections associated with a device.

The 5G system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections associated with a device.

The 5G system shall support service continuity for a remote UE, when the remote UE changes from one relay UE to another and both relay UEs use 3GPP access to the 5G core network.

## 6.3	Multiple access technologies

### 6.3.1	Description

The 5G system will support 3GPP access technologies, including one or more 5G RATs (e.g., optimized for different purposes) and E-UTRA as well as non-3GPP access technologies. Interoperability among the various access technologies will be imperative. For optimization and resource efficiency, the 5G system will select the most appropriate 3GPP or non-3GPP access technology for a service, potentially allowing multiple access technologies to be used simultaneously for one or more services active on a device.  New technology such as satellite and wide area base stations will increase coverage and availability. This clause provides requirements for interworking with the various combinations of access technologies.

Editor's Note: The naming as well as the consistency of usage of the different radio access technologies needs further analysis and alignment.

Editor's Note: The service requirements regarding the need for or level of interworking with earlier generation of 3GPP systems needs to be clarified.

### 6.3.2	Requirements

#### 6.3.2.1	General

Based on operator policy, the 5G system shall enable the UE to select, manage, and efficiently provision services over the 3GPP or non-3GPP access.

Based on operator policy, the 5G system shall support steering a UE to select certain 3GPP access network(s) (e.g., 5G RAN, evolved E-UTRAN).

Based on operator policy, the 5G system shall be able to dynamically offload part of the traffic (e.g. from 3GPP RAN to non-3GPP access), taking into account traffic load and traffic type.

Based on operator policy, the 5G system shall be able to provide simultaneous data transmission via different access technologies, (e.g., 5G RAT, evolved E-UTRA, non-3GPP), to access one or more 3GPP services.

When a UE is using two or more access technologies simultaneously, the 5G system shall be able to select different access technologies taking into account e.g., service, traffic characteristics, radio characteristics, and UE’s moving speed.

The 5G system shall be able to support data transmissions optimized for different access technologies (e.g., 5G RAT, evolved E-UTRA, non-3GPP) for UEs that are simultaneously connected to the network via different accesses.

Based on operator policy, the 5G system shall be able to add or drop the various access connections for a UE during a session.

The 5G system shall be able to support inter-system mobility between the supported access networks (e.g., 5G, E-UTRAN, WLAN, fixed broadband).

The 5G system shall support devices with multiple radio and single radio capabilities.

The 5G system shall support dynamic and static network address allocation of a common network address to the UE over all supported access types.

The 5G system shall support a set of identities for a single user in order to provide a consistent set of policies and a single set of services across 3GPP and non-3GPP access types.

#### 6.3.2.2	E-UTRA access

The 5G system shall be able to support seamless handover between 5G RAT and evolved E-UTRA.

The 5G system shall support UEs with dual radio capability (i.e., a UE that can transmit on the 5G RAT and the evolved E-UTRA RAT simultaneously) as well as UEs with single radio capability (i.e., a UE that cannot transmit on the 5G RAT and the evolved E-UTRA RAT simultaneously).

#### 6.3.2.3	Satellite access

The 5G system shall be able to provide services using satellite access.

The 5G system shall support service continuity between land based 5G access and satellite based access networks owned by the same operator or by an agreement between operators.

#### 6.3.2.4	Fixed broadband

The 5G system shall be able to efficiently support connectivity using fixed broadband access.

NOTE:	The specification of fixed broadband access network is outside the scope of 3GPP.

The 5G system shall support use of a relay UE that supports multiple access types (e.g., 5G RAT, WLAN, fixed broadband).

The 5G system shall support use of a home base station that supports multiple access types (e.g., 5G RAT, WLAN, fixed broadband).

## 6.4	Resource Efficiency

### 6.4.1	Description

5G introduces the opportunity to design a 3GPP system to be optimized for supporting diverse devices and use cases. While support for IoT is provided by EPS, there is room for improvement in efficient resource utilization that can be designed into a 5G system whereas they are not easily retrofitted into an existing system. Some of the underlying principles of the potential service and network operation requirements associated with efficient configuration, deployment, and use of devices in the 5G network include bulk provisioning, resource efficient access, optimization for device originated data transfer, and efficiencies based on the reduced needs related to mobility management for stationary devices and devices with restricted range of movement.

As sensor and monitoring devices are deployed more extensively, the need to support devices that send data packages ranging in size from a small status update in a few bits to streaming video increases. A similar need exists for smart phones with widely varying amounts of data. Specifically, to support short data bursts, the network should be able to operate in a mode where there is no need for a lengthy and high overhead signalling procedure before and after small amounts of data are sent. The system will, as a result, avoid both a negative impact to battery life for the device and wasting signalling resources.

For small form factor devices it will be challenging to have more than 1 antenna due to the inability to get good isolation between multiple antennas. Thus these devices need to meet the expected performance in a 5G network with only one antenna.

Additional resource efficiencies will contribute to meeting the various KPIs defined for 5G. Control plane resource efficiencies can be achieved by minimizing signalling overhead, particularly for small data transmissions. Mechanisms for minimizing user plane resources utilization include in-network caching and application in a Service Hosting Environment closer to the end user. These optimization efforts contribute to achieving lower latency and higher reliability.

Diverse mobility related resource efficiencies are covered in clause 6.2.

Security related resource efficiencies are covered in clause 8.8.

### 6.4.2	Requirements

#### 6.4.2.1	General

The 5G system shall minimize control and user plane resource usage for data transfer from send only devices.

The 5G system shall minimize control and user plane resource usage for stationary device (e.g., lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g., 1 million connections per square kilometre) taking into account, for example, the following criteria:

-	Type of mobility support

-	Communication pattern (e.g., send only)

-	Characteristics of payload (e.g., infrequent or not, small size data payload, etc.)

-	Characteristics of application (e.g., provisioning operation, normal data transfer, etc.)

-	Geographic information of device

The 5G system shall efficiently support service discovery mechanisms where devices can discover, subject to access rights:

-	Status of other devices (e.g., sound on/off);

-	Capabilities of other devices (e.g., the device is a relay device) and/or;

-	Services provided by other devices (e.g., the device is a colour printer).

The 5G system shall be able to minimise the amount of wireless backhaul traffic (e.g., consolidating data transmissions to a larger rather than many smaller), when applicable (e.g., providing service in an area subject to power outages).

The 5G system shall support small form factor devices with single antenna.

NOTE:	Small form factor devices are typically expected to have the diagonal less than 1/5 of the lowest supported frequency wave length.

#### 6.4.2.2	Efficient bulk operations for IoT

The 5G network shall optimize the resource use of the control plane and/or user plane to support high density connections (e.g., 1 million connections per square kilometre) of grouped devices i.e. grouped in the same small geographical area.

The 5G network shall optimize the resource use of the control plane and/or user plane to manage (e.g., provide service parameters, activate, deactivate) groups of devices.

The 5G system shall support a timely, efficient, and/or reliable mechanism to transmit the same information to multiple grouped devices.

#### 6.4.2.3	Efficient management for IoT

The 5G network shall optimize the resource use of the control plane and/or user plane to manage (e.g., provide service parameters, activate, deactivate) a device.

#### 6.4.2.4	Efficient control plane

The 5G system shall minimize the signalling that is required prior to user data transmission.

NOTE:	The amount of signalling overhead may vary based on the amount of data to be transmitted, even for the same device.

## 6.5	Efficient User Plane

### 6.5.1	Description

5G is designed to meet diverse use cases with different and enhanced performances (e.g., high throughput, low latency and massive connections) and data traffic model (e.g., IP data traffic, non-IP data traffic, short data bursts and high throughput data transmissions).

User plane should be more efficient for 5G to support differentiated requirements. On one hand, a Service Hosting Environment located inside of operator’s network can offer Hosted Services closer to the end user to meet localization requirement like low latency, low bandwidth pressure. These Hosted Services contain applications provided by operators and/or trusted 3rd parties. On the other hand, user-plane paths can be selected or changed to improve the user experience or reduce the bandwidth pressure, when a UE or application changes location during an active communication.

### 6.5.2	Requirements

Based on operator policy and/or application needs, the 5G system shall support an efficient user-plane path between UEs attached to the same network, modifying the path as needed when a UE changes location during an active communication.

Based on operator policy and/or application needs, the 5G system shall support an efficient user-plane path between a UE attached to the mobile network and communication peers (e.g., Internet hosts) outside of the mobile network, modifying the path as needed when a UE changes location during an active communication.

The 5G network shall enable a Service Hosting Environment provided by operator.

The 5G network shall be able to support routing of data traffic between a UE attached to the mobile network and the application in a Service Hosting Environment for specific services, modifying the path as needed when a UE changes location during an active communication.

Based on operator policy and/or application needs, the 5G system shall support an efficient user-plane path between a UE and application in a Service Hosting Environment, modifying the path as needed when a UE or application changes location during an active communication.

The 5G network shall maintain user experience (e.g., QoS, QoE) when a UE or application in a Service Hosting Environment changes location during an active communication.

The 5G network shall be able to interact with applications in a Service Hosting Environment for efficient network resource utilization.

The 5G network shall support configurations of the Service Hosting Environment in the network (e.g., access network, core network), that provide application access close to a UE.

The 5G system shall support mechanisms to enable a UE to access the closest Service Hosting Environment for a specific application or service.

The 5G network shall enable instantiation of applications for a UE in a Service Hosting Environment close to the UE’s location.

NOTE: 	Not all applications will always be available in all Service Hosting Environments. Therefore, it may be needed to instantiate an application at a Service Hosting Environment nearby for serving a particular UE.

Based on operator policy, the 5G system shall provide a mechanism such that one type of traffic (from a specific application or service) to/from a UE can be offloaded close to the UE's point of attachment to the access network, while not impacting other traffic type to/from that same UE.

## 6.6	Efficient content delivery

### 6.6.1	Description

Video based applications (e.g., live streaming, VR), personal data storage applications have been instrumental for the massive growth in mobile broadband traffic. Subject to service agreement between the operator and content provider, the information of content and/or content itself can be aware by operator. In-network content caching including both operator provided and 3rd party provided can improve user experience, reduce backhaul resource usage and utilize limited radio resource efficiently.

The operation of in-network caching includes flexible management of the location of the content cache within the network and efficiently delivery of content from/to the appropriate content caching application. For example: delivering popular video content from a content caching application via broadcast, and securely storing the personal data/files of a user in distributed caching application. Such a service could also provide a student wireless backpack, where students can resume their work through the same or a different device at a time convenient to them, with very fast response times from the network.

### 6.6.2	Requirements

The 5G system shall enable efficient delivery of content from a content caching application under the control of the operator (e.g., a cache located close to the UE).

The 5G system shall support a content caching application in a UE under the control of the operator.

The 5G system shall support configurations of content caching applications in the network (e.g., access network, core network), that provide content close to the UE.

Based on operator policy, the 5G system shall support an efficient mechanism for selection of a content caching application (e.g., minimize utilization of radio, backhaul resources and/or application resource) for delivery of the cached content to the UE.

The 5G system shall support a mechanism for the operator to manage content distribution across content caching applications.

The 5G system shall support delivery of cached content from a content caching application via the broadcast/multicast service.

## 6.7		Priority, QoS and Policy Control

### 6.7.1		Description

The 5G network will be supporting many commercial services (e.g., medical) and regional or national regulatory specific services (e.g., MPS, Emergency, Public Safety) with requirements for priority treatment. Some of these priority services share common QoS characteristics such as latency and packet loss rate, but may have different priority requirements.  For example, drone control and air traffic control may have stringent latency and packet loss rate requirements but not necessarily the same priority requirements. In addition, voice based services for MPS and Emergency share common QoS characteristics as applicable for normal public voice communications, yet may have different priority requirements. The 5G network will need to support mechanisms that enable the decoupling of the priority of a particular communication from the associated QoS characteristics such as latency and packet loss rate to allow flexibility to serve different priority services (that need to be configurable to meet operator needs, consistent with operator policies and corresponding national / regional regulatory policies).

The network needs to support flexible means to make priority decisions based on the state of the network (e.g., during disaster events and network congestion) recognizing that the priority needs may change in time during a crisis. The priority of any given service may need to be different (e.g., elevated) for a particular user of that service based on operational needs and regional or national regulations. Therefore, the 5G system should allow a flexible means to make and enforce priority decisions among the different services (e.g., MPS, Emergency, medical, Public Safety) and amongst users of these services.

The network must offer a means to provide the required QoS (e.g., high reliability, latency, bandwidth, etc.) for a service and the ability to prioritize resources when necessary to meet the specific service requirements. Existing QoS and policy frameworks handle latency and improve reliability by traffic engineering. In order to support diverse 5G service requirements, it is necessary for the 5G network to offer QoS and policy control for reliable communication with latency required for a service and enable the resource adaptations as necessary.

Also, as 5G network is expected to operate in a heterogeneous environment with multiple access technologies, multiple types of devices, etc., it should support a harmonised QoS and policy framework that applies across multiple accesses.

Further, for existing EPC, QoS control only covers RAN and core network, but for 5G network E2E QoS (e.g., RAN, backhaul, core network, network to network interconnect) is needed to achieve the 5G user experience (e.g., ultra-low latency, ultra-high bandwidth).

### 6.7.2		Requirements

The 5G system shall allow flexible mechanisms to establish and enforce relative priority policies among the different services (e.g., MPS, Emergency, medical, Public Safety).

NOTE 1:	Relative priority between different services is subject to regional or national regulatory and operator policies.

The 5G system shall be able to provide the required QoS (e.g., reliability, latency, and bandwidth) for a service and support prioritization of resources when necessary for that service.

The 5G system shall allow decoupling of the priority of a particular communication from the associated QoS characteristics such as latency and packet loss rate.

The 5G system shall be able to support a harmonised QoS and policy framework applicable to multiple accesses.

The 5G system shall be able to support E2E (e.g., UE to UE) QoS for a service.

NOTE 2:	E2E QoS needs to consider QoS in the access networks, backhaul, core network, and network to network interconnect.

The 5G system shall be able to support QoS for applications in Services Hosting Environment.

## 6.8	Dynamic Policy Control

### 6.8.1	Description

Editor's Note: Text in this section is to be added.

### 6.8.2	Requirements

The 5G system shall support the creation and enforcement of prioritisation policy for users and traffic, during connection setup and when connected.

NOTE:	Relative prioritization, pre-emption, and precedence of critical traffic associated with certain priority services (e.g., MPS and Emergency) are subject to regional/national regulatory and operator policies.

The 5G system shall support optimised signalling for prioritised users and traffic where such signalling is prioritized over other signalling traffic.

Based on operator policy, the 5G system shall allow flexible means to create and enforce relative priority among the different service flows by authorized users.

Based on operator policy, the 5G system shall allow real-time, dynamic, secure, limited interaction with the QoS and policy framework for modification of the QoS and policy framework by authorized users.

Based on operator policy, the 5G system shall allow modifications by authorized users and context aware network functionality to the QoS and policy framework to have a variable duration.

## 6.9	Connectivity models

### 6.9.1	Description

The UE can connect with the network directly or connect with the network using another UE as a relay UE, or they may be capable of using both types of connections. The scope of such UEs can be from simple wearables, such as a smart watch or a set of sensors embedded in clothing, to a more sophisticated wearable device monitoring biometrics. They can also be non-wearable devices that communicate in a Personal Area Network such as a set of home appliances (e.g., smart thermostat and entry key), or the electronic devices in an office setting (e.g., smart printers), or a smart flower pot that can be remotely activated to provide water to the plant. The relay UE can access the network using 3GPP or non-3GPP access (e.g., WLAN, fixed broadband access). 3GPP and non-3GPP radio technologies can be supported for connectivity scenarios between the remote UE and the relay UE. 3GPP RAT over licensed band or unlicensed band can be supported for connectivity scenarios.

At the time of connecting a remote UE via a relay UE, there might be several relay UEs, supporting discovery and remote UE-to-relay UE connection establishment, available and in proximity of the remote UE. A remote UE may support selection of an appropriate relay UE among the available relay UEs.

### 6.9.2	Requirements

The connection between a remote UE and a relay UE shall be able to use 3GPP RAT or non-3GPP RAT and use licensed or unlicensed band.

The 5G system shall support indirect 3GPP communication in a VPLMN when a remote UE and a relay UE subscribe to different PLMNs and both PLMNs have a roaming agreement with the VPLMN.

The 5G system shall be able to support a UE using simultaneous indirect and direct 3GPP communication.

The 5G system shall be able to support QoS for a user traffic session between the remote UE and the network using 3GPP access technology.

The 5G system shall support selection of a relay UE, in proximity of the remote UE and available for discovery and relay connection establishment, based on selection criteria such as the relay UE identity, capabilities of the relay UE, which 3GPP or non-3GPP access the relay UE uses to connect to the network, or pre-paired relay UEs.

NOTE:	The list of selection criteria is non-exhaustive.

## 6.10	Network Capability Exposure

### 6.10.1	Description

3GPP SEES and (e)FMSS features allow the operator to expose network capabilities e.g., QoS policy to 3rd party ISPs/ICPs. With the advent of 5G, new network capabilities need to be exposed to the 3rd party (e.g., to allow the 3rd party to customize a dedicated network slice for diverse use cases;  to allow the 3rd party to manage trusted 3rd party application in hosting service environment to improve user experience, efficient utilize backhaul and/or application  resources).

Applications may be frugal at periods when resources are rare and can compensate and anticipate as much as they can if timely network status information is aware by application; otherwise. Applications may be disconnected in case of user mobility.

### 6.10.2	Requirements

Based on operator policy, a 5G network shall provide suitable APIs to allow a 3rd party to create, modify and delete network slices used for the 3rd party.

Based on operator policy, the 5G  system shall allow a 3rd party to define and update the set of services supported in a network slice.

Based on operator policy, the 5G  system shall allow a 3rd party to assign a device to a network slice based on subscription, device type, and services provided by the network slice.

The 5G network shall provide a mechanism to expose broadcasting capabilities to 3rd party broadcasters’ management systems.

Based on operator policy, a 5G network shall provide suitable APIs to allow a trusted 3rd party to manage this trusted 3rd party owned application(s) in the operator's Service Hosting Environment.

## 6.11	Context aware network

### 6.11.1	Description

A variety of sensors such as accelerometer, gyroscope, magnetometer, barometer, proximity sensor, GPS, etc. can be integrated in a UE. Also, different applications running on the UE can have different communication needs (e.g., different traffic time). In addition, a UE can support different access technologies such 5G RAT, WLAN, fixed broadband, etc. The information gathered by sensors, the utilized access technologies, the application context, and the application traffic characteristics can provide useful information to the applications installed in the UE and can also help the 5G system utilize resources in an efficient and optimized way.

### 6.11.2	Requirements

The 5G system shall support network resource utilization efficiently and network optimization based on system information, including:

-	Network conditions, such as network load and congestion information

-	Information on served UEs such as access information (e.g., 3GPP access (e.g., 5G, E-UTRA), non-3GPP access (e.g., WLAN, fixed broadband)), cell type (e.g., macro cell, small cell), UE experienced data rate;

-	Application’s characteristics (e.g., expected traffic over time);

-	Information on prioritized communication such as user subscription profile and priority level, priority services (e.g., MPS, Emergency, and Public Safety), application used for priority communications (e.g., voice, video, and data) and traffic associated with priority communications (signalling and media);

-	Subject to user consent, enhanced traffic characteristic of UE (e.g., Mobility information (e.g., no mobility, nomadic, spatially restricted mobility, full mobility), location, sensor-level information (e.g., direction, speed, power status, display status, other sensor information installed in the UE), application-level information (e.g., foreground applications, running background application, and user settings).

The 5G system shall support mechanisms to collect system information for network optimization within an operator configured time scale.

## 6.12	Self backhaul

### 6.12.1	Description

The increasingly high densification of access nodes needed to meet future performance objectives poses considerable challenges in deployment and management (e.g., backhaul availability, backhaul capacity and scalability). The use of wireless backhaul for such access nodes helps to address some of the challenges.

Wireless self-backhauling can enable simpler deployment and incremental rollout by reducing reliance on the availability of wired backhaul at each access node location. Network planning and installation efforts can be reduced by leveraging plug and play type features -- self-configuration, self-organizing, and self-optimization.

### 6.12.2	Requirements

The 5G network shall enable operators to support wireless backhaul function using 5G RAT.

The 5G network shall support flexible and efficient backhaul for both indoor and outdoor scenarios.

The 5G network shall support flexible partitioning of radio resources between access and backhaul functions.

The 5G network shall support autonomous configuration of access and backhaul functions.

The 5G network shall support autonomous adaptation on wireless backhaul network topologies to minimize service disruptions.

## 6.13	Flexible Broadcast/Multicast Service

### 6.13.1	Description

Editor's Note: Text in this section is to be added.

### 6.13.2	Requirements

The 5G system shall support operation of a downlink only broadcast/multicast system over a wide geographic area in a spectrally efficient manner for stationary and mobile UEs.

The 5G system shall enable the operator to reserve 0% to 100% of radio resources of one or more radio carriers for the delivery of broadcast/multicast content.

The 5G network shall allow the UE to receive content via a broadcast/multicast radio carrier while a concurrent data session is ongoing over another radio carrier.

The 5G system shall be able to support broadcast/multicast of UHD streaming video (e.g., 4k/8k UHD).

NOTE:	Taking into account the bandwidth needs for different streaming video resolution.

The 5G network shall allow the operator to configure and broadcast multiple quality levels (i.e., video resolutions) of broadcast/multicast content for the same service in a stand-alone 3GPP based broadcast/multicast system.

The 5G network shall support parallel transfer of multiple quality levels (i.e., video resolutions) of broadcast/multicast content for the same service to the same UE taking into account e.g., UE capability, radio characteristics, application information.

## 6.14	Subscription aspects

### 6.14.1	Description

With the Internet of Things, it is expected that the diversity of device types and the usage models will largely vary. Moreover, when the device is manufactured, the deployment location and specific usage may not be known. Sometimes the devices will be added to existing subscriptions, other times they may be part of a new subscription for the user. Sometimes the devices may be leased. During their life cycle these devices go through different stages, involving the change in ownership when the device is deployed and possibly afterwards, the activation of the device by the preferred operator, a possible change of operators, etc. These stages need to be managed securely and efficiently. A method of dynamic subscription generation and management is needed in addition to statically provisioned subscription. Once the subscription is established, subscription management becomes necessary, for example, to modify the subscription when the ownership of the device changes, to update or refresh credentials due to suspected leakage or theft of security keys or as a preventive measure, etc.

The Internet of Things will also support various connectivity models: The devices can connect with the network directly or connect with the network using another device as a relay UE, or they may be capable of using both types of connections. The direct connection between the device and the relay UE can be using 3GPP or non-3GPP RAT. The relay UE can access the network also using 3GPP or non-3GPP access networks (e.g., WLAN, fixed broadband access). In order to identify and manage the devices, a subscription with the 5G network is needed, even if the access is done via non-3GPP access.

### 6.14.2	Requirements

A device which is able to access the 5G network in direct 3GPP connection mode using a 3GPP RAT shall have a 3GPP subscription.

The 5G system shall be able to provide mechanisms to change the association between a subscription and address/number of an IoT device (e.g., changing the owner and subscription information associated with the device) within the same operator and in between different operators in an automated or manual way.

The 5G system shall be able to support identification of subscriptions independently of identification of devices. Both identities shall be secure.

An IoT device which is able to connect to a UE in direct device connection mode shall have a 3GPP subscription, if the IoT device needs to be identifiable by the core network (e.g., for device management purposes or to use indirect 3GPP connection mode).

Based on operator policy, the 5G system shall support a mechanism to provision on-demand connectivity. This on-demand mechanism should enable means for a user to request on-the-spot network connectivity while providing operators with identification and security tools for the provided connectivity.

Editor’s Note: The terms “IoT device” and “device” need to be defined.

## 6.15	Energy Efficiency

### 6.15.1	Description

Energy efficiency is a critical issue in 5G. The potential to deploy systems in areas where there is not a reliable energy source requires new methods of managing energy consumption not only in the devices but throughout all components of the 5G system.

Small form factor devices also typically have a small battery and this not only puts constrains on general power optimization but also on how the energy is consumed. With smaller batteries it is more important to understand and follow the limitations for the both the max peak and continuous current drain.

### 6.15.2	Requirements

The 5G access network shall support an energy saving mode with the following characteristics:

-	The energy saving mode can be activated/deactivated either manually or automatically.

-	Service can be restricted to a pre-defined set of users (e.g., public safety user, emergency callers).

NOTE:	When in energy saving mode the UE’s and Access transmit power may be reduced or turned off (deep sleep mode), latency and jitter may be increased with no impact on set of users or applications still allowed.

The 5G system shall support mechanisms to improve battery life for a device over what is possible in EPS.

The 5G system shall optimize the battery consumption of a relay UE via which a device is in indirect 3GPP connection mode.

The 5G system shall support devices using small rechargeable and single coin cell batteries (e.g. considering impact on max pulse and continuous current).

## 6.16	Markets requiring minimal service levels

### 6.16.1	Description

A key aspect of 5G system flexibility is the ability to support both the very high end markets as well as very low end markets. Some systems will be deployed in areas where there are constraints on energy resources (e.g., sporadic access to power supplies) and lower end user expectations for availability, reliability, and capacity. In such cases, the system needs additional flexibility to adapt power consumption needs based on fluctuations in power availability. The system should be efficient in order to provide essential services in harsh environments (e.g., far remote rural areas, very large territories) while taking into account the local constraints (adapting resources consumptions to long distances, dealing with variable conditions and possibly disconnections). Content delivery should be optimized in order to reduce constraints on transport networks, on low-end user devices (e.g., limited screen size, limited energy consumption), variable network conditions, and client profiles.

### 6.16.2	Requirements

In constrained circumstances (e.g., reduced power supply), the 5G system shall be able to support a minimal user experience (e.g., user experienced data rate of [100]kbps, E2E latency of 50ms, lower availability of the network of 95%).

The 5G system shall support centralized automation and management of the network in order to reduce local management tasks.

The 5G system shall support a mechanism to reduce data transfer rate at the cell edge for very large coverage area (e.g., 100kbps for more than 100km cell coverage, 1Mbs for 100km cell coverage).

The 5G system shall be able to give priority to services (e.g., e-Health) when resources are limited.

## 6.17	Extreme long range coverage in low density areas

### 6.17.1	Description

A fully connected society is expected in the near future. The network access everywhere over long distances (e.g., at extreme rural areas or at sea) including both humans and machines need to be supported.

### 6.17.2	Requirements

The 5G system shall support the extreme long range coverage in low density areas (e.g., up to 100 km).

Editor’s note: It is FFS what is the impact on other KPIs of the extreme long range.

## 6.18	eV2X aspects

### 6.18.1	Description

Editor's Note: to be updated.

### 6.18.2	Requirements

Editor's Note: to be updated.

# 7	Performance requirements

## 7.1	High data rates and traffic densities

Several scenarios require the support of very high data rates or traffic densities of the 5G system. The scenarios address different service areas: urban and rural areas, office and home, and special deployments (e.g., massive gatherings, broadcast, residential, and high-speed vehicles). The scenarios and their performance requirements can be found in table 7.1-1.

-	Urban macro - The general wide-area scenario in urban area

-	Rural macro - The general wide-area scenario in rural area

-	Indoor hotspot - The scenario for offices and homes, and residential deployments.

-	Broadband access in a crowd - The scenario for very dense crowds, for example, at stadiums or concerts. In addition to a very high connection density the users want to share what they see and hear, putting a higher requirement on the uplink than the downlink.

-	Dense urban - The scenario for pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The users in vehicles can be connected either directly or via an onboard base station to the network.

-	Broadcast-like services - The scenario for stationary users, pedestrian users, and users in urban vehicles, for example, in offices, city centres, shopping centres, and residential areas. The passengers in vehicles can be connected either directly or via an onboard base station to the network.

-	High-speed train - The scenario for users in trains. The users can be connected either directly or via an onboard base station to the network.

-	High-speed vehicle - The scenario for users in road vehicles. The users can be connected either directly or via an onboard base station to the network.

-	Airplanes connectivity - The scenario for users in airplanes. The users can be connected either directly or via an onboard base station to the network.

Table 7.1-1 Performance requirements for high data rate and traffic density scenarios.

|  | Scenario | Experienced data rate (DL) (NOTE 4) | Experienced data rate (UL) (NOTE 4) | Area traffic capacity (DL) (NOTE 5) | Area traffic capacity (UL) (NOTE 5) | User density | Activity factor (NOTE 6) | UE speed | Coverage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | Urban macro | 50 Mbps | 25 Mbps | 100 Gbps/km2 | 50 Gbps/km2 | 10000/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 1b | Rural macro | 50 Mbps | 25 Mbps | 1 Gbps/km2 | 0.5 Gbps/km2 | 100/km2 | 20% | Pedestrians and users in vehicles (up to 120 km/h | Full network (NOTE 1) |
| 2 | Indoor hotspot | 1 Gbps | 500 Mbps | 15 Tbps/km2 | 2 Tbps/km2 | 250000/km2 | NOTE 2 | Pedestrians | Office and residential (NOTE 2) (NOTE 3) |
| 3 | Broadband access in a crowd | 25 Mbps | 50 Mbps | [3.75] Tbps/km2 | [7.5] Tbps/km2 | [150000]/km2 | 30% | Pedestrians | Confined area |
| 4 | Dense urban | 300 Mbps | 50 Mbps | 750 Gbps/km2 | 125 Gbps/km2 | 25000/km2 | 10% | Pedestrians and users in vehicles (up to 60 km/h) | Downtown (NOTE 1) |
| 5 | Broadcast-like services | Maximum 200 Mbps (per TV channel) | N/A or modest (e.g., 500 kbps per user) | N/A | N/A | [15] TV channels of [20 Mbps] on one carrier | N/A | Stationary users, pedestrians and users in vehicles (up to 500 km/h) | Full network (NOTE 1) |
| 6a | High-speed train | 50 Mbps | 25 Mbps | 15 Gbps/train | 7.5 Gbps/train | 1000/train | 30% | Users in trains (up to 500 km/h) | Along railways (NOTE 1) |
| 6b | High-speed vehicle | 50 Mbps | 25 Mbps | [100] Gbps/km2 | [50] Gbps/km2 | 4000/km2 | 50% | Users in vehicles (up to 250 km/h) | Along roads (NOTE 1) |
| 7 | Airplanes connectivity | 15 Mbps | 7.5 Mbps | 1.2 Gbps/plane | 600 Mbps/plane | 400/plane | 20% | Users in airplanes (up to 1000 km/h) | (NOTE 1) |
| NOTE 1: For users in vehicles, the device can be connected to the network directly, or via an on-board moving base station. NOTE 2: A certain traffic mix is assumed; only some users use services that require the highest data rates [2]. NOTE 3: For interactive audio and video services, for example, virtual meetings, the required two-way end-to-end latency (UL and DL) is 2-4ms while the corresponding experienced data rate needs to be up to 8k 3D video [300Mbps] in uplink and downlink. NOTE 4: User experienced data rate is the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed. NOTE 5: Area traffic capacity is the product of the user experienced data rate, the user density, and the activity factor. NOTE 6: Activity factor is a percentage value of the amount of simultaneous active UEs to the total number of UEs where active means the UEs are exchanging data with the network. NOTE 7: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |  |

## 7.2	Low latency and high reliability

Editor's Note: Text in this section is still to be elaborated

### 7.2.1	Overview

Several service scenarios demand very low latency and very high reliability. The overall latency depends on the delay on the radio interface, transmission within the 3GPP system, transmission to a server which may be outside the 3GPP system, and data processing. Some of these factors depend directly on the 3GPP system itself, whereas for others the impact can be reduced by suitable interconnections from the 3GPP system to applications or servers outside of the 3GPP system, for example, to allow local hosting of the applications. The use cases and their performance requirements can be found in table 7.2.2-1. Latency in the table refers to the one-way latency on the radio layer.

### 7.2.2	Use cases and KPIs

Use cases requiring low latency and high reliability can be categorized in the following groups:

-	Industrial control - Conventional industrial control is characterised by high requirements on the communications system regarding latency, reliability, and availability. Systems supporting industrial control are usually deployed in geographically limited areas but may also be deployed in wider areas (e.g., city- or country-wide networks), access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Industrial automation - Industrial automation is characterised by high requirements on the communications system regarding reliability and availability. Systems supporting industrial automation are usually deployed in geographically limited areas, access to them may be limited to authorised users, and they may be isolated from networks or network resources used by other cellular customers.

-	Tactile interaction - Tactile interaction is characterised by a human being interacting with the environment or people, or controlling a device, and relying on tactile feedback.

-	Audio-visual interaction - Audio-visual interaction is characterised by a human being interacting with the environment or people, or controlling a device, and relying on audio-visual feedback.

-	Remote control - Remote control is characterised by a device being operated remotely, either by a human or a computer.

Table 7.2.2-1 Performance requirements for low-latency and high-reliability use cases.

| Use case | Latency | Reliability | Experienced data rate | Payload | Traffic density | Connection density | Coverage NOTE 1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Industrial control | 1ms | 99,999 % | 1 Mbps | Small | 1 Gbps/km2 | 1 000/km2 | Very high | NOTE 2 |
| Industrial automation | 10ms | 99,999 % | 1 Mbps | Small | 100 Mbps/km2 | 100/km2 | High |  |
| Tactile interaction | 0,5ms | [99,999 %] | [Low] | [Small] | [Low] | [Low] | [Very high] | NOTE 2 |
| Audio-visual interaction | [10ms] | [99,9 %] | [250 Mbps] | [Big] | [Low] | Low] | [High] | NOTE 3 |
| Remote control | [5ms] | [99,999 %] | [From low to 10 Mbps] | [Small to big] | [Low] | [Low] | [Very high] |  |
| NOTE 1: The coverage requirement applies to the service area, e.g., inside a factory. NOTE 2: Traffic prioritization and hosting services close to the end-user may be helpful in reaching the lowest latency values. NOTE 3: Audio-visual interaction requires very low-delay audio and video coding, and high video frame rates (e.g., 120 fps). NOTE 4: All the values in this table are targeted values and not strict requirements. |  |  |  |  |  |  |  |  |

Editor's Note: All values in this table are FFS.

### 7.2.3	Other requirements

To support virtual reality environments with low motion-to-photon capabilities, the 5G system shall support equivalent low motion-to-sound delay (<20ms).

NOTE: 	in virtual reality the motion-to-sound latency is the latency between the physical movement of a user’s head and updated sound waves from a head mounted speaker reaching their ears.

To support interactive task completion during voice conversations the 5G system shall support low-delay speech coding for interactive conversational services (100ms, one way mouth-to-ear).

## 7.3	eV2X performance requirements

Editor's Note: to be updated.

# 8	Security

## 8.1	Description

IoT introduces new devices with different life cycles, including devices with no user interface (e.g., embedded sensors), long life spans during which the device may change ownership several times (e.g., consumer goods), and which may not be pre-provisioned (e.g., consumer goods). These drive a need for secure mechanisms to dynamically establish or refresh credentials and subscriptions. New access technologies, including licensed and unlicensed, 3GPP and non-3GPP, drive a need for access independent security that is seamlessly available while the device is active. High-end smartphones, drones, and factory automation drive a need for protection against theft and fraud.  Expansion into enterprise, vehicular, and public safety markets drive a need for increased end user privacy protection. 5G security addresses all of these new needs while continuing to provide security consistent with prior 3GPP systems.

## 8.2	General

The 5G system shall support a secure mechanism to store cached data.

The 5G system shall support a secure mechanism to access a content caching application.

The 5G  system shall support a secure mechanism to access a service or an application in an operator’s Hosting Service Environment.

The 5G  system shall enable support of an access independent security framework.

The 5G system shall support a mechanism for the operator to authorize subscribers of other PLMNs to receive temporary service (e.g., mission critical services).

The 5G system shall be able to provide temporary service for authorized users without access to their home network (e.g., IOPS, mission critical services).

The 5G system shall allow the operator to authorize a 3rd party to create, modify and delete network slices, subject to an agreement between the 3rd party and the network operator.

A relay UE should not be able to intercept any relayed data.

## 8.3	Authentication

The 5G network shall support a resource efficient mechanism for authenticating groups of devices.

The 5G system shall support an efficient means to authenticate a user to a device (e.g., biometrics).

The 5G system shall be able to support authentication over a non-3GPP access technology using 3GPP credentials.

The 5G system shall support operator controlled alternative authentication methods (i.e., alternative to AKA) with different types of credentials for network access for IoT devices to allow new isolated deployment scenarios (e.g., for industrial automation).

## 8.4	Authorization

Based on operator policy, before establishing a direct device communications using a non-3GPP access technology, devices may use 3GPP credentials to determine if they are authorized to engage in direct device communication.

Based on operator policy, the 5G system shall provide a means to verify whether a device is authorized to use prioritized network access for a specific service.

## 8.5	Identity management

The 5G system shall provide a mechanism for an operator to allow access from a device using a temporary identifier that hides its subscriber identity.

The 5G system shall provide a mechanism for an operator to allow access from a device connected in an indirect connection using a temporary identifier that hides its subscriber identity.

The 5G HPLMN shall be able to associate a temporary identifier to a device's subscriber identity.

The 5G system shall be able to protect subscriber identity and other user identifying information from passive attacks.

Subject to regulatory requirements, the 5G system shall be able to protect subscriber identity and other user identifying information from active attacks.

The 5G system shall be able to allow the equipment identifier to be collected by legitimate entity regardless of device's user interface, when required.

The 5G system shall be able to support identification of subscriptions independently of identification of equipment.

The 5G system shall support a secure mechanism to collect system information while ensuring end-user and application privacy (e.g., application level information is not to be related to an individual user identity or subscriber identity and device information is not to be related to an individual subscriber identity).

For a private network using 3GPP technology, the system shall support network access using identities, credentials, and authentication methods provided and managed by a 3rd party and supported by 3GPP.

Editor’s Note: The term “private network” needs to be defined.

## 8.6	Regulatory

The 5G system shall support regulatory requirements for 5G and fixed broadband access.

The 5G system shall support Lawful Intercept regulatory requirements.

## 8.7	Fraud protection

Subject to regulatory requirements, the 5G system shall support a secure mechanism allowing an authorized entity to disable from normal operation of a device reported as stolen.

Subject to regulatory requirements, the 5G system shall support a secure mechanism allowing an authorized entity to re-enable a recovered stolen device to normal operation.

The 5G system shall be able to protect user location information from passive attacks.

Subject to regulatory requirements, the 5G system shall be able to protect user location information from active attacks.

## 8.8	Resource efficiency

The 5G system shall minimize security signalling overhead without compromising the security level of the 3GPP system.

The 5G system shall support a secure mechanism to transmit the same data (e.g., service provisioning multiple sensors) to multiple devices.

# 9	Charging aspects

The following requirements apply for both home and roaming cases.

The 5G core network shall support collection of charging information for alternative authentication mechanisms.

The 5G core network shall support collection of charging information associated with each serving MNO when multi-network connectivity is used under the control of the home operator.

The 5G core network shall support charging for services/applications in an operator's Service Hosting Environment.

The 5G core network shall support charging for content delivered from a content caching application.

The 5G core network shall support collection of charging information based on the access type (e.g., 3GPP, non-3GPP).

The 5G core network shall support collection of charging information based on the network performance metrics, when the information is available.

Annex <A> (normative):
<Normative annex title>

Annexes are only to be used where appropriate:

Annex <B> (informative):
<Informative annex title>

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content (informative annexes do not comprise requirements for the implementation of the specification).

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-05 | SA1#74 |  |  |  | Skeleton |  | 0.1.0 |
| 2016-08 | SA1#75 |  |  |  | Agreements in SA1#75: S1-162342, S1-162305, S1-162480, S1-162345, S1-162481, S1-162471, S1-162313, S1-162347, S1-162486, S1-162349, S1-162546, S1-162322, S1-162323, S1-162450, S1-162547, S1-162548, S1-162549, S1-162492, S1-162550, S1-162520, S1-162485, S1-162517, S1-162551, S1-162505. And rapporteur’s clean-up. | 0.1.0 | 0.1.1 |
| 2016-11 | SA1#76 |  |  |  | Agreements in SA1#76: S1-163412, S1-163445, S1-163446, S1-163256, S1-163070, S1-163415, S1-163258, S1-163293, S1-163260, S1-163416, S1-163296, S1-163264, S1-163265, S1-163266, S1-163267, S1-163268, S1-163297, S1-163298, S1-163417, S1-163418, S1-163299, S1-163274, S1-163275, S1-163277, S1-163278, S1-163280, S1-163448, S1-163421, S1-163029, S1-163112, S1-163285, S1-163286, S1-163287, S1-163422, S1-163289, S1-163382, S1-163132, and rapporteur’s clean-up. | 0.1.1 | 0.2.0 |