3GPP TSG-SA WG1 Meeting #76bis	S1-170285

Spokane, US, 16-20, Jan 2017	(revision of S1-170024)

Title: 	gap analysis for interworking use cases in section 12

Agenda Item:	6.2

Source: 	Huawei

Contact: 	Carry Liu (liujianning.liu@huawei.com )

Laurence Meriau (laurence.meriau@huawei.com )

Abstract: This document is aim to propose detailed gap analysis for the agreed interworking use cases in section 12 with united gap analysis.

Proposal

------------------------------- Start of Change------------------------------

12.2	Area Broadcast Group Communication interworking between GSM-R and FRMCS users12.2.1	Description

This use case describes an area broadcast group communication between FRMCS User(s) and GSM-R User(s), and vice versa.

12.2.2 	Pre-condition

FRMCS User A and GSM-R User B and User C are authorized Users to perform group communication together by the FRMCS system.

User A and User B and User C are in the same group communication area defined by the FRMCS system.

12.2.3	Service flow

FRMCS system to GSM-R

FRMCS User A initiates a new area broadcast group communication (based on location, presence etc, of GSM-R User B and C).

FRMCS User A broadcasts group communication invitations to all Users in the same area.

GSM-R User B and User C receive the group communication invitations.

GSM-R User B and User C accept the invitation and join the group communication. Or GSM-R User B and User C automatically join this group communication.

GSM-R to FRMCS

GSM-R User B initiates a new group call (based on location, presence etc, of FRMCS User A and GSM-R User C).

GSM-R User B broadcasts group call invitations to all Users in the same area.

FRMCS User A and GSM-R User C receive the group call invitations.

FRMCS User A and GSM-R User C accept the invitation and join the group call. Or FRMCS User A and GSM-R User C automatically join this group call.

12.2.4	Post-condition

FRMCS User A, GSM-R User B and User C can communicates among each other in a group call.

12.2.5	Potential requirements

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [12.2.5-001] | FRMCS System shall provide a means to initiate group communication between FRMCS Users and GSM-R Users. | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |
| [12.2.5-002] | FRMCS system shall provide means to FRMCS users to join the group calls of GSM-R | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |

12.3	Location Service interworking between GSM-R and FRMCS users

12.3.1	Description

This use case allows FRMCS system and GSM-R system to obtain and share the location information of their users..

12.3.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R User.

User A and User B are affiliated to a group that consists of FRMCS User(s) and GSM-R User(s).

12.3.3	Service flow

FRMCS system to GSM-R

FRMCS User A sends a location request to the specific GSM-R User B.

GSM-R User B accepts and return his location information as requested.

GSM-R to FRMCS system

GSM-R User B sends a location request to the specific FRMCS User A.

FRMCS User A accepts and returns his location information as requested.

12.3.4	Post-condition

FRMCS User A’s location information is sent to GSM-R User B.

GSM-R User B’s location information is sent to FRMCS User A.

12.3.5	Potential requirements

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [12.3.5-001] | FRMCS System shall provide a means to obtain and share with FRMCS User the location information of GSM-R User(s). | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |
| [12.3.5-002] | FRMCS system shall provide means to FRMCS users to join the group calls of GSM-R | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |

12.4	Emergency group Communication between GSM-R and FRMCS users

Editor’s Note: this is FFS

12.5	Presence interworking between GSM-R and FRMCS users

12.5.1	Description

This use case allows FRMCS User(s) to see the status of GSM-R user(s), and vice versa. For example, FRMCS User(s) can see the status of a GSM-R user, including user ID, states (available, busy, etc.), vice versa.

12.5.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R Users.

GSM-R system detects the change status of GSM-R User B

FRMCS system detects the change status of FRMCS User A

12.5.3	Service flow

FRMCS system to GSM-R

FRMCS System receives the GSM-R User B’s changed status.

FRMCS System sends the changed status to FRMCS User A.

GSM-R to FRMCS system

GSM-R receives the User A’s changed status.

GSM-R sends the changed status to User B

12.5.4	Post-condition

The changed status of GSM-R User B is presented in FRMCS User A.

The changed status of FRMCS User A is presented in GSM-R User B.

12.5.5	Potential requirements

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [12.5.5-001] | FRMCS System shall provide a means for an FRMCS User to present the presence status of GSM-R User, including user ID, states (e.g., available, busy, etc.), etc. | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |
| [12.5.5-002] | FRMCS System shall provide a means to share the presence status of FRMCS User(s) to GSM-R, including user ID, states (e.g., available, busy, etc.), etc. | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |

12.6	Point to Point communication between GSM-R and FRMCS users

12.6.1	Description

This use case allows a FRMCS User to communicate with a GSM-R User, vice versa.

12.6.2 	Pre-condition

User A is an authorized FRMCS User.

User B is an authorized GSM-R Users.

12.6.3	Service flow

FRMCS system to GSM-R

FRMCS User A initiates a point to point communication to GSM-R User B.

GSM-R User B accepts the communication and joins the communication.

GSM-R to FRMCS system

GSM-R User B initiates a point to point call to FRMCS User A.

FRMCS User A accepts the call and joins the call.

12.6.4	Post-condition

GSM-R User B and FRMCS User A are communicating with each other.

12.6.5	Potential requirements

[12.6.5-001] FRMCS System shall provide a means for point to point communication between an authorized FRMCS User and a GSM-R User,

[Gap Analysis: This requirement is not covered by 3GPP existing specification.]

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [12.6.5-001] | FRMCS System shall provide a means for point to point communication between an authorized FRMCS User and a GSM-R User, | A | Not covered | Transport of the interconnection is needed, but not consider in this use case |

12.7	Seamless handover between GSM-R and FRMCS system

Editor’s Note: this is FFS

------------------------------- End of Change------------------------------