3GPP TSG-SA WG1 Meeting #86	S1-191567

Suzhou, China 06-11 May 2019	(revision of S1-191373)

Title:	Proposal for consolidated performance requirements of AVPROD

Agenda Item:	8.2

Source: 	EBU, BBC, Sennheiser

Contact:	ian.wagdin@bbc.co.uk

# Introduction

This contribution proposes final consolidated performance requirements for AVPROD. These requirements are first introduced in the discussion section with the tables containing the data from the use cases. It is proposed to include 3 tables with performance requirements. 1 for audio use cases using point to point communications, another for audio use cases using group communications, and another for video use cases.

# Discussion

- Audio use cases with unicast

The performance requirements of “On-site Live Audio Presentation” are presented in the table below:

Table 5.1.6-1: Performance requirements of low latency periodic deterministic communication service

| Profile | # of active UEs | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ad-hoc | 20 | 5 km/h | 300 m x 300 m | 3 ms | 3 ms | 0.01 % | 200 kbit/s | - |
|  | 8 | stationary | 300 m x 300 m | 3 ms | 3 ms | 0.01 % | - | 200 kbit/s |
| Campus | 1000 | 5 km/h | 2 km x 2 km | 5 ms | 5 ms | 0.01 % | 200 kbit/s | - |
| Conference | 10 | 5 km/h | 100 m x 100 m | 3 ms | 3 ms | 0.01 % | 1.5 Mbit/s | - |
|  | 4 | stationary | 100 m x 100 m | 3 ms | 3 ms | 0.01 % | - | 1.5 Mbit/s |
| Lecture room | 4 | 5 km/h | 10 m x 10 m | 3 ms | 3 ms | 0.01 % | 50 kbit/s | - |
|  | 2 | stationary | 10 m x 10 m | 3 ms | 3 ms | 0.01 % | - | 50 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (15 – 2 × Tframe) for 1-way communication and ≤ (15 – 3 × Tframe))/2 for 2-way communication profiles. If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. |  |  |  |  |  |  |  |  |

The performance requirements of “On-site Live Audio Presentation” are presented in the table below:

Table 5.2.6-1: Performance requirements of low latency periodic deterministic communication service

| Profile | # of active UEs | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Festival | 200 | 10 km/h | 500 m x 500 m | 600 µs | 250 µs | 0.01 % | 500 kbit/s | - |
|  | 100 | 10 km/h | 500 m x 500 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
| Musical | 30 | 50 km/h | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | 500 kbit/s | - |
|  | 20 | 50 km/h | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
|  | 10 | - | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | - | 500 kbit/s |
| Garage | 10 | 5 km/h | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | 100 kbit/s | - |
|  | 10 | 5 km/h | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | - | 200 kbit/s |
|  | 2 | - | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | - | 100 kbit/s |
| AV production | 20 | 5 km/h | 30 m x 30 m | 600 µs | 250 µs | 0.01 % | 1.5 Mbit/s | - |
|  | 10 | 5 km/h | 30 m x 30 m | 600 µs | 250 µs | 0.01 % | - | 3 Mbit/s |
| Audio Studio | 30 | - | 10 m x 10 m | 600 µs | 250 µs | 0.01 % | 5 Mbit/s | - |
|  | 10 | 5 km/h | 10 m x 10 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (2 – 3 × Tframe)/2. If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. |  |  |  |  |  |  |  |  |

- Audio use cases with multicast

The performance requirements of “Live production with integrated audience services” are presented in the table below:

Table 5.3.6-1: Performance requirements for low latency deterministic periodic traffic with multicast service.

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrated audience services | 50000 | - | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 5 ms DL | 5 ms | 0.1% | - | 200 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (15 – 2 × Tframe). If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. |  |  |  |  |  |  |  |  |  |  |

The performance requirements of “Intercom system for large live events” are presented in the table below:

Table 5.4.6-1: Performance requirements for low latency deterministic periodic traffic with multicast service.

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intercom system | 1000 | 240 | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 3 ms DL | 3 ms | 0.1% | 100 kbit/s | 100 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (15 – 3 × Tframe)/2 which yields a mouth-to-ear latency of 20 ms. If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. |  |  |  |  |  |  |  |  |  |  |

- Video use cases

The performance requirements of “Single- Source uncompressed Outside Broadcast Contribution” are presented in the table below:

Table 5.5.6-1: Potential new requirements.

| Use Case | mobility | Uni or bi directional |  | Downlink data rate | Uplink data rate | Packet size (bytes) | link latency note 2,3 | Total delay (inc. application) | Reliability Note 1 | # of active UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.5.6-001] Uncompressed UHD video | stationary | bi | 20 Mbit/s |  | 12 Gbit/s | 1500 | 400 ms | <800 ms | QEF | 1 | 1KM2 |
| [PR 5.5.6-002] Uncompressed HD video | stationary | bi | 20 Mbit/s |  | 3.2 Gbit/s | 1500 | 400 ms | <800 ms | QEF | 1 | 1KM2 |
| Note 1: The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. Note 2: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3: Latency is less important than QEF, longer latency can be accepted to avoid errors (e.g. existing satellite delays can be up to 5 seconds). Note 4: Use case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. |  |  |  |  |  |  |  |  |  |  |  |

The performance requirements of “Single- source compressed Outside Broadcast Contribution” are presented in the table below:

Table 5.6.6-1: Potential new requirements.

| Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | link latency (note 2,3) | Packet size (bytes) | Total delay inc. application | Reliability (note 1) | # active of UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.6.6-001] Mezzanine compression UHD video | stationary | bi | 20 Mbit/s | 3 Gbit/s | < 1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-002] Mezzanine compression HD video | stationary | bi | 20 Mbit/s | 1 Gbit/s | < 1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-003] Tier one events UHD | stationary | bi | 20 Mbit/s | 500 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-004] Tier one events UHD | stationary | bi | 20 Mbit/s | 200 Mbit/s | <1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-005] Tier two events UHD | slow (< 7km/h) | bi | 20 Mbit/s | 100 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-006] Tier two events HD | slow (< 7km/h) | bi | 20 Mbit/s | 80 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-007] Tier three events UHD (note 6) | slow to high (<7km/h to <200 km/h) | bi | 10 Mbit/s | <20 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <10 | 1000m2 |
| [PR 5.6.6-008] Tier three events HD (note 6) | slow to high (<7 km/h to <200 km/h) | bi | 10 Mbit/s | 10 Mbit/s | < 1sec | 1500 | <1.5 sec | QEF | <10 | 1000m2 |
| Note 1: The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. Note 2: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3: Latency is less important than QEF, longer latency can be accepted to avoid errors (e.g. existing satellite delays can be up to 5 seconds). Note 4: Use case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. Note 6: Could use either professional equipment or mobile phone equipped with dedicated newsgathering app (SMPTE requirements do not apply). |  |  |  |  |  |  |  |  |  |  |

The performance requirements of “Professional TV Production Contributions from an Off-Site, Remotely-Produced, Multi-Camera Outside Broadcast” are presented in the table below:

Table 5.7.6-1: Potential new requirements.

| Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | link latency (note 2:3) | Max packet size | Total delay (inc.application) | reliability | # of active UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.7.6-001] Remote OB | stationary or slow <7km/h | bi | 20 Mbit/s | 200Mbit/s | 6 ms | 1500 bytes | <20 ms | QEF (note 1) | <5 | 1000m2 |
| Note 1: The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. Note 2: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3: Latency is less important than QEF, longer latency can be accepted to avoid errors. Note 4: User case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. |  |  |  |  |  |  |  |  |  |  |

The performance requirements of “Video Streaming of Live Events using an Airborne Relay” are presented in the table below:

Table 5.9.6-1: Potential new requirements.

| Use Case | Mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | Max Packet size | link latency | Total delay (inc. application) | reliability | # of active UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.9.6-001] NPN ground to air UHD up Link | up to 500 km/h | bi | 20 Mbit/s | 100 Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 5-10 | 700km2 x 6000 m (note 6) |
| [PR 5.9.6-002] NPN ground to air HD up link | up to 500 km/h | bi | 20 Mbit/s | 80 Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 5-10 | 700km2 x 6000 m (note 6) |
| [PR 5.9.6-003] NPN air to ground UHD down Link | up to 500 km/h | bi | 100 Mbit/s | 20Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 2 | 700km2 x 6000 m (note 6) |
| [PR 5.9.6-004] NPN air to ground HD down link | up to 500 km/h | bi | 80 Mbit/s | 20 Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 2 | 700km2 x 6000 m (note 6) |
| Note 1:The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement.Note 2:This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3:Latency is less important than QEF, longer latency can be accepted to avoid errors. Note 4:User case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. Note 6:6000 m = height but in a cone formation (i.e. ground coverage with a circle of diameter 30 KM) |  |  |  |  |  |  |  |  |  |  |

The performance requirements of “Video Streaming in Professional Coverage of Live Performances” are presented in the table below:

| Table 5.6.6-1: Potential new requirements.Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | Max Packet size | link latency | Total delay (inc. application) | reliability | # of active UE | Service area |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.11.6-001] NPN radio Camera UHD | <200 km/h | bi | 20 Mbit/s | 100 Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 5-10 | 1000m x 1000m |  |
| [PR 5.11.6-002] NPN radio camera HD | <200 km/h | bi | 20 Mbit/s | 80 Mbit/s | 1500 bytes | 3 ms (note 2,3) | <40 ms | QEF (note 1) | 5-10 | 1000m x 1000m |  |
| Note 1: The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. Note 2: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3: Latency is less important than QEF, longer latency can be accepted to avoid errors. Note 4: Use case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. |  |  |  |  |  |  |  |  |  |  |

***** BEGIN CHANGE *****

# 8	Consolidated potential requirements

Performance requirements

Table 8.1-1: Performance requirements of low latency periodic deterministic communication service

| Profile | # of active UEs | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ad hoc | 20 | 5 km/h | 300 m x 300 m | 3 ms | 3 ms | 0.01 % | 200 kbit/s | - |
|  | 8 | stationary | 300 m x 300 m | 3 ms | 3 ms | 0.01 % | - | 200 kbit/s |
| Campus | 1000 | 5 km/h | 2 km x 2 km | 5 ms | 5 ms | 0.01 % | 200 kbit/s | - |
| Conference | 10 | 5 km/h | 100 m x 100 m | 3 ms | 3 ms | 0.01 % | 1.5 Mbit/s | - |
|  | 4 | stationary | 100 m x 100 m | 3 ms | 3 ms | 0.01 % | - | 1.5 Mbit/s |
| Lecture room | 4 | 5 km/h | 10 m x 10 m | 3 ms | 3 ms | 0.01 % | 50 kbit/s | - |
|  | 2 | stationary | 10 m x 10 m | 3 ms | 3 ms | 0.01 % | - | 50 kbit/s |
| Festival | 200 | 10 km/h | 500 m x 500 m | 600 µs | 250 µs | 0.01 % | 500 kbit/s | - |
|  | 100 | 10 km/h | 500 m x 500 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
| Musical | 30 | 50 km/h | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | 500 kbit/s | - |
|  | 20 | 50 km/h | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
|  | 10 | - | 50 m x 50 m | 600 µs | 250 µs | 0.01 % | - | 500 kbit/s |
| Semi-professional | 10 | 5 km/h | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | 100 kbit/s | - |
|  | 10 | 5 km/h | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | - | 200 kbit/s |
|  | 2 | - | 5 m x 5 m | 600 µs | 250 µs | 0.01 % | - | 100 kbit/s |
| AV production | 20 | 5 km/h | 30 m x 30 m | 600 µs | 250 µs | 0.01 % | 1.5 Mbit/s | - |
|  | 10 | 5 km/h | 30 m x 30 m | 600 µs | 250 µs | 0.01 % | - | 3 Mbit/s |
| Audio Studio | 30 | - | 10 m x 10 m | 600 µs | 250 µs | 0.01 % | 5 Mbit/s | - |
|  | 10 | 5 km/h | 10 m x 10 m | 600 µs | 250 µs | 0.01 % | - | 1 Mbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (15 – 2 × Tframe) for 1-way communication, ≤ (15 – 3 × Tframe))/2 for 2-way communication semi-professional profiles, and ≤ (2 – 3 × Tframe)/2 for the professional profiles. If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. |  |  |  |  |  |  |  |  |

Table 8.1-2: Performance requirements for low latency deterministic periodic traffic with multicast service.

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrated audience services | 50000 | - | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 5 ms DL | 5 ms | 0.1% | - | 200 kbit/s |
| Intercom system | 1000 | 240 (Note 3) | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 3 ms DL 3 ms UL | 3 ms | 0.1% | 100 kbit/s | 100 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation and can be defined as being the same as the Tframe of Annex A. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is smaller than ≤ (15 – 3 × Tframe)/2 for the 2-way communication profiles and ≤ (15 – 2 × Tframe) for the 1-way communication profiles, which yields a mouth-to-ear latency of 20 ms. If there is an interface for synchronizing audio devices and radio interface as described in Section 7, the latency requirement can be relaxed as described in the Annex A.1. NOTE 2: Packet error rate is related to a packet size of transfer interval × data rate. Packets that do not conform with the end-to-end latency are also accounted as error. NOTE 3: The UL stream originating from a UE may be the source of a DL multicast stream. |  |  |  |  |  |  |  |  |  |  |

Table 8.1-3: Performance requirements for low latency video.

| Profile | # of active UEs | UE Speed | Service Area | E2E latency | Packet error rate (Note 1) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uncompressed UHD video | 1 | 0 km/h | 1 km2 | 400 ms | 10-8% UL 10-5% DL | 12 Gbit/s | 20 Mbit/s |
| Uncompressed HD video | 1 | 0 km/h | 1 km2 | 400 ms | 10-7% UL 10-5% DL | 3 .2 Gbit/s | 20 Mbit/s |
| Mezzanine compression UHD video | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 3 Gbit/s | 20 Mbit/s |
| Mezzanine compression HD video | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 1 Gbit/s | 20 Mbit/s |
| Tier one events UHD | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 500 Mbit/s | 20 Mbit/s |
| Tier one events UHD | 5 | 0 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 200 Mbit/s | 20 Mbit/s |
| Tier two events UHD | 5 | 7 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| Tier two events HD | 5 | 7 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| Tier three events UHD (Note 2) | 5 | 200 km/h | 1000 m2 | 1 s | 10-5 % UL 10-5 % DL | 20 Mbit/s | 10 Mbit/s |
| Tier three events HD (Note 2) | 5 | 200 km/h | 1000 m2 | 1 s | 10-5 % UL 10-5 % DL | 10 Mbit/s | 10 Mbit/s |
| Remote OB | 5 | 7 km/h | 1000 m2 | 6 ms | 10-6 % UL 10-5 % DL | 200 Mbit/s | 20 Mbit/s |
| NPN ground to air UHD up Link | 10 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| NPN ground to air HD up link | 10 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| NPN air to ground UHD down Link | 2 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-5 % UL 10-6 % DL | 20 Mbit/s | 100 Mbit/s |
| NPN air to ground HD down link | 2 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-5 % UL 10-6 % DL | 20 Mbit/s | 80 Mbit/s |
| NPN radio Camera UHD | 10 | 200 km/h | 1 km2 | 3 ms | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| NPN radio camera HD | 10 | 200 km/h | 1 km2 | 3 ms | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| NOTE 1: Packet error rate calculated considering 1500 B packets, and 1 error per hour as 𝐏𝐄𝐑=(𝟖 ∗𝒑𝒂𝒄𝒌𝒆𝒕 𝒔𝒊𝒛𝒆)⁄(𝒅𝒂𝒕𝒂 𝒓𝒂𝒕𝒆 𝟑𝟔𝟎𝟎). NOTE 2: Could use either professional equipment or mobile phone equipped with dedicated newsgathering app NOTE 3: 6000 m = height but in a cone formation (i.e. ground coverage with a circle of diameter 30 KM) |  |  |  |  |  |  |  |

***** END of  CHANGE *****