3GPP TSG-SA WG1 Meeting #91e	S1-203338S1-203104

Online, 24 August – 02 September 2020                revision of S1-203104,3167

Title:	Use Case of Advanced Metering for Smart Grid

Agenda Item:	7.9.1

Source:	China Southern Power Grid, CEPRI, ZTE, China Telecom，Samsung

Contact:	xu.ling@zte.com.cn,

wangh26@chinatelecom.cn,

Abstract: This contribution proposes a new use case of advanced metering in smart grid for FS_5GSEI

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[y]	China Southern Power Grid, China Mobile, HuaWei: "White Paper: 5G enables smart grid applications".http://www.sgcio.com/technology/anli/2018/0716/86656.html

…

5.x	 Use case of advanced metering

5.x.1	Description

Instead of recording and sending the metering data from a traditional wired electricity meter unit, electricity metering collecting can be executed by an UE integrated smart meter unit, including UE integrated community facilities, smart household appliances, etc. Smart meter units can send real-time metering data to the server in the Power Enterprise through mobile networks. In this way, the Power Enterprise will reduce the cost of cable for data transiting and make the monitoring flexible that helps meet the benefits and bring the new benefit chance. This is also a possible usage of AMI.

Advanced Metering utilizes AMI (Advanced Meter Infrastructure) system to collect, count, analyze, distribute and manage abnormal electrical energy data in the generation, transmission, distribution and using stages. It can collect real-time data of user power, monitor line loss, and power outage, real-time identify the change of household, calculate of three-phase imbalance, and monitor station voltage.

NOTE: The information obtained from smart appliances are never considered accurate nor guaranteed by certification authorities. This information is useful to customers to understand and influence their consumption patterns.

The AMI is usually comprised of smart meter, two-way communication network, measurement data management system (MDMS), and an optional user indoor network (HAN).

Accurate fee control is a key advanced metering function enabled by AMI. The electric smart meter co-works with MDMS to deliver and perform control command for measuring instruments. The electric smart meters monitor relevant user energy status and deliver these data to MDMS. The MDMS sends control commands according to its policy and status data collected. The remote commands from the MDMS include: tripping, closing permission, alarm, alarm release, power protection, power protection release.

NOTE: Billing may or may not be correlated with use of MBMS service, as in many geographies the DSO is a distinct business entity from the Retailer, who has the direct business relationship with the customer. In such a case, the smart meter information will not be correlated with any billing information and cannot therefore provide feedback to customers concerning the billing impact of their energy usage.

Instead of recording and sending the metering data from a traditional wired electricity meter unit, electricity metering collecting can be executed by an UE integrated smart meter unit, including UE integrated community facilities, smart household appliances, etc. Smart meter units can send real-time metering data to the server in the Power Enterprise through mobile networks. In this way, the Power Enterprise will reduce the cost of cable for data transiting and make the monitoring flexible that helps meet the benefits and bring the new chance.

Due to massive electricity meter boxes and household appliances keep online at the same time, the amount of connections the mobile network maintains simultaneously is so enormous that challenging to the mobile network capacity. It is estimated that the increased connections will up to 50-100 times.

Accurate fee control is a function that the electric smart meter co-works with electricity management center and delivers and performs control command for measuring instruments. The electric smart meters of public transformers and special transformers are required to carry out relevant status monitoring operation. The remote commands from the electricity management center include: tripping, closing permission, alarm, alarm release, power protection, power protection release.

5.x.2	Pre-Conditions

SS is a power company and it constructs AMI system to deploy advanced meter service for its users. Operator TT has assigned a contract with the power company SS to supply communication service for the AMI system.

Tom is one of the user of SS. In his house, all the electric equipment’s energy data can be collected by smart meter and then reported to the MDMS of SS.

The smart meter is located in Tom’s house or nearby and the MDMS is located in the remote city center. The distance between the smart meter and the energy management center can be as far as a middle size city range (e.g. Tens of km)

5.x.3	Service Flows

Scenario1:

- The smart meter in Tom’s house report related energy usage data to SS.

- The MDMS of SS sends commands to smart meter in Tom’s home to adjust smart meter report frequency and content considering its whole energy working status.

- The smart meter in Tom’s home changes the report scheme accordingly.

- When the MDMS detects that the user's remaining electricity will be run out, it will send an alarm command to remind the user to recharge as soon as possible;

- When the user's remaining electricity is exhausted, the MDMS will issue a trip command and the smart meter need implement the trip action accordingly which requires the latency less than 200ms (command delivery + command action);

- When there are special circumstances that cannot be cut off, the MDMS can issue a power protection command in advance to ensure that the smart meter will not trip for related circumstances.

Scenario2:

- After the smart meter in Tom’s house reports the status data to the MDMS, it detects that something is wrong with Tom’s home electricity supply system.

- The MDMS asks the smart meter to report more information for troubleshooting.

- The smart meter co-works with MDMS to resolve the problem.

5.x.4	Post-Conditions

The energy related information from electric equipment can be reported to the MDMS on demand with required content and required communication performance.

The electric accurate fee control function can be implemented.

Co-working with smart meter, the MDMS can remotely troubleshoot energy problem for its user.

5.x.5	Existing features partly or fully covering the use case functionality

The 5G system shall be able to support resilience to dynamic adjust connection service performance considering advanced metering demand.

The 5G system shall be able to provide connection service wherever indoor, outdoor, low and medium altitude for advanced metering applications.

Editor note: It is FFS whether and how to address providing connection service to underground access for advanced metering applications.

The 5G system shall be able to provide required communication service to advanced metering according to KPI given in table x.x (note1)

Table x.x Communication KPI for advanced metering

| User experienced Data rate (bps) | Latency (ms) | Reliability % | Connection density | coverage |
| --- | --- | --- | --- | --- |
| UL:<2M DL:<1M | Accuracy fee control: < 100 (note2); General information data collection: <3000 | >99.99 | <[x]*10000/km2 | <40km (city range) |
| NOTE1: refer [y]. NOTE2: the accuracy fee control latency here is for communication latency. The command implementation need 100ms. Editor Note: the "x" in the table with real value will FFS. |  |  |  |  |

5.x.6	Potential New Requirements needed to support the use case

NOTE1: refer [x] published by China Southern Power Grid.

NOTE2: the accuracy fee control latency here is for communication latency. The command implementation need 100ms.