3GPP TSG-SA WG1 Meeting #92e	S1-204013

Electronic Meeting, 11-20 November 2020	(revision of S1-20xxxx)

Title:	FS_AMMT – Removal of Editor’s Notes on other DNN models for the use-case on real time media editing with on-board AI inference

Agenda Item:	7.4.1

Source:	InterDigital

Contact:	Atle Monrad <Atle.Monrad@InterDigital.com>

Abstract:

In the description of the use-case on real time media editing with on-board AI inference it is an Editor’s note that indicate the FFS for additional DNN models. No additional DNN models are identified, thus the related Editor’s Notes are removed.

The description of the service flow is improved/clarified.

A couple of editorial improvements are introduced.

Proposal

It is proposed to include the below improvements to the use case for real time media editing with on-board AI inference in 3GPP TR 22.874 version 0.1.0.

* * * *   First Change   * * * *

## 6.2	Real time media editing with on-board AI inference

### 6.2.1	Description

Smartphone is the #1 device that people carry and use everywhere for audio and video recording. It also becomes the first device to exchange media content with friends and family, to publish on social media. High end smartphones embed more and more powerful CPU and GPU and even dedicated AI hardware accelerators. As camera and picture/video quality become a differentiator among high end smartphones, AI/ML models to enhance photo shoots locally emerge on these high-end devices. AI accelerators are expected to enable the execution of complex AI/ML models directly on end-users connected devices; not only photo enhancements but high-quality audio and video content analysis and enhancement are expected to be executed locally on smartphones. Smartphones will consequently become a device to edit media content prior to sharing over the network. With the advent of 5G, new services relying on on-demand downloads of large AI/ML models to be executed in (near) real time on end user device will emerge; depending on the service, the environment, the user’s preference, the device characteristics, etc., these DNN models will need to be adapted or updated under strict latency constraints which prevent all of them to be stored locally in advance.

Media content analysis combines tasks as object detection, segmentation, face recognition, people counting, human activity tracking. In table 6.2.1-1, examples of commonly used DNN models for object detection (with their respective sizes) are listed.

Table 6.2.1-1. Sizes of typical object detection models

| Model for object detection | Number of parameters (Million) | Size of the model (MByte) 32 bits parameters | Size of the model (MByte) 8 bit parameters |
| --- | --- | --- | --- |
| MobileNet | 3.2 | 12.8 | 3.2 |
| DarkNet | 20 | 80 | 20 |
| SE ResNet | 26 | 104 | 26 |
| Inception v4 | 41 | 164 | 41 |
| YOLONet | 64 | 256 | 64 |
| VGGNet | 134 | 536 | 134 |

Media content edition combines tasks as audio and video quality improvement, language translation, face anonymisation. In tables 6.2.1-2 and 6.2.1-3, examples of commonly used DNN models for image super-resolution and for video super-resolution are listed.

Table 6.2.1-2. Sizes of typical image super-resolution models

| Model for image super-resolution | Number of parameters (Million) | Size of the model (MByte) 32 bits parameters | Size of the model (MByte) 8 bits parameters |
| --- | --- | --- | --- |
| RCAN | 15.44 | 61.78 | 15.44 |
| SAN | 15.71 | 62.82 | 15.71 |
| RDN | 22.12 | 88.48 | 22.12 |
| EDSR | 40.73 | 162.92 | 40.73 |
| OISR-RK3 | 41.91 | 167.64 | 41.91 |

Table 6.2.1-3. Sizes of typical video super-resolution models

| Model for video super-resolution | Number of parameters (Million) | Size of the model (MByte) 32 bits parameters | Size of the model (MByte) 8 bits parameters |
| --- | --- | --- | --- |
| RBPN/4-PF | 12.7 | 50.8 | 12.7 |
| RBPN/6-PF | 12.7 | 50.8 | 12.7 |
| VSR-DUF | 6.8 | 27.2 | 6.8 |
| DRDVSR | 0.7 | 2.8 | 0.7 |

Two settings are considered for the use case:

- Independent user: a person takes a video or starts a video call on its UE in a noisy environment, with difficult lighting conditions, and automatic tagging of scene and objects are embedded in the video.

- Crowd: During a large event, like a live concert, several thousand people use their UEs to film or photograph the musician band at the same time, and request additional information on the concert like band discography, lyrics, artist facial recognition, instrument/equipment brand. In this context, UEs request the downloads of DNN models to improve the capture and recording of the concert, and to provide information requested by people attending the concert. Several DNN models can be requested by each UE to e.g. execute the following tasks: image shooting and video optimization, artist face recognition, musical instrument identification, audio improvement and lyrics generation. Given the heterogeneous fleet of UEs, thousands of DNN models required by the application/service can be requested for download. These DNN models are adapted or updated to the UEs operating system type and version, hardware characteristics and environment.

### 6.2.2	Pre-conditions

The setting for this use case is as follows. Alice is attending a crowded live concert. She is eager to get movie clips and pictures as a great souvenir of the concert, but she is worried about difficult conditions to get this great souvenir as the conditions for light and sound are very variable, with limited light for the spectators and too much light on the scene. The audio stereo is also variable and not well-balanced dependant on where she is among the audience and has a very noisy background.

Alice would like to store good quality movie clips and pictures on her private account on the internet for the future, and also post photos and videos tagged with artist name and other relevant information during the concert. As Alice is also an amateur musician, she also wants to get detailed real-time information about the structure of the song, the lyrics and the instruments.

The pre-conditions are:

a)	Alice is attending a crowded live concert.

b)	Her UE is registered to the 5G network.

c)	Applications of Alice’s smartphone can rely on fine-tuned machine learning models that are available via the network that covers the concert hall. These models are:

1)	An ML model improving photo capture for this concert hall (a model specially fine-tuned for this concert place).

2)	An ML model improving audio capture for this concert hall (a model specially fine-tuned for this concert place).

3)	An ML model specialized in the discography and the lyrics of the band.

4)	An ML model specialized in the artists face recognition.

5)	An ML model specialized in music instrument identification.

NOTE:	The listed models above are examples for this use-case and is not an exhaustive list.

### 6.2.3	Service Flows

1)	Shortly after the start of the concert, Alice, as most of the fans, launches the camera application on her mobile phone to film the scene and to get additional information about the band, the individual artists or the songs or instruments.

2)	She points her device’s camera towards the scene.

3)	The environment is very dark with strong light spots. To be fully functional and to render the best user experience, the camera application downloads ad-hoc ML models.

4)	The proposed ML models are very performant in this environment but also very heavy in size.

5)	The ML models can be used for the whole capture especially if environment remains stable. If the environment changes substantially, or better ML models become available, ML models can be progressively updated accordingly with respect of some operating rules (like a maximum number of ML update per second). In all cases, the camera application continues working seamlessly.

6)	The audio and video streams are captured, improved in quality, processed to extract and display additional information, and stored in real-time on the mobile phone itself.

* * * *   End-of Changes   * * * *