3GPP TSG-SA WG1 Meeting #92e	S1-204091

Online, 09 November – 18 November 2020

Title:	 Active and Inactive consideration for ranging service

Agenda Item:	7.10.1

Source:	ZTE

Contact:	 xu.ling@zte.com.cn

1. Discussion

In SA1 #91emeeting, some new use cases have been introduced into TR22.855 to study Ranging-based Services requirements.

The description of all use cases existed in the TR are focus on the scenarios after the ranging operation is active, so the “Ranging Interval” is required as [50ms] and “Effective Ranging distance” is required as [20m,10m,100m,0-1m,0.1~10m,0.1~2m,2-10m,5m] depend on different scenarios.

In fact, this kind of ranging operation (active) is only needed conditionally e.g. in Effective Ranging Distance scope, and doesn’t need operate so frequently during all the ranging service period. So it is reasonable that the ranging service has inactive stage and active stage. During the active stage, the 5G system operates ranging operation with ranging interval as [50ms]. And during the inactive stage, the ranging operation can slow down and prepare to enter active stage.

The 5G system can detect the condition which meet some criterion to enter active ranging operation, e.g. the measured ranging distance <= Active Ranging distance; or to enter inactive ranging operation, e.g. the measured ranging distance >= Active Ranging distance, or some other conditions which can be configured by ranging service application.

2. Proposal

5.x	Ranging active and inactive

5.x.1	Description

Ranging service requires ranging UE to frequently operate ranging action e.g. with ranging interval of [50ms] when it in efficient ranging distance. In fact, this kind of ranging operation (active) is only needed in effective ranging distance field. It doesn’t need the ranging service operates so frequently during all the service period.

So it is reasonable that the ranging service has inactive stage and active stage. During the active stage, the 5G system operates ranging operation with ranging interval as [50ms]. And during the inactive stage, the ranging operation can slow down and prepare to enter active stage.

The 5G system can detect the condition which meet some criterion to enter active ranging operation, e.g. the measured ranging distance <= Active Ranging distance; or to enter inactive ranging operation, e.g. the measured ranging distance >= Active Ranging distance, or some other conditions which can be configured by ranging service application. Figure 5.x.1-1 illustrates two stages of ranging service.

5.x.2	Pre-Conditions

The Tom’s car support ranging service. Its paired ranging UE is Tom’s smart phone.

Tom’s car is usually parked in his house during the night. And in working days, it is always parked in Tom’s office underground parking port.

Tom’s car and its smart phone have registered one of Opertor’s Ranging service: Smart Car Key.  The “inactive ranging interval” is configured as 5min.

The “Active Ranging Distance” for this ranging service can be calculated with following formulas:

“Active Ranging Distance” <= “Effective Ranging Distance” of active ranging operation + “Relative UE velocity” of active ranging operation × “inactive ranging interval” and

“Active Ranging Distance” >= “Effective Ranging Distance” of active ranging operation

So, if the “inactive ranging interval” is 5min, from user experience aspect, the “Effective Ranging Distance” is usually required up to 100m, the “Relative UE velocity” is usually 1m/s, we can deduce that the “Active Ranging Distance” is in the scope from 100m to 400m.

5.x.3	Service Flows

In the morning, Tom’s car is parking at home and operating ranging with large ranging interval e.g. 5min.

Tom goes to his car carrying his smart phone and the car identifies that the distance with the paired smart phone is less than “Active Ranging Distance”.

The car starts active ranging operation with required ranging service KPI and the Ranging Interval is 50ms.

The car automatically unlocks and opens the door when Tom is nearby.

Tom drives the car to his office, then parks his car in the office underground parking port.  Tom leaves his car. His car identifies the distance between the car and Tom’s smart phone exceeds the configured “lock door distance”. The car automatically locks.

The car continuously operates active ranging with Ranging Interval is 50ms until it detects that the distance exceeds minimum of “Active Ranging Distance” (e.g. 100m) which can be calculated through above formula. It enters the inactive ranging operation the ranging interval is 5min.

Tom leaves his office and goes to his car. His car identifies the distance less than maximum of “Active Ranging Distance” (e.g. 400m).

The car enters active ranging operation with the ranging interval back to 50ms.

The car automatically opens the door when Tom is nearby. Tom drives his back home.

5.x.4	Post-Conditions

Tom’s car can be automatically lock and unlock with more power efficiently.

5.x.5	Existing features partly or fully covering the use case functionality

TS 22.261 clause 6.27 Positioning services

The 5G system shall support mechanisms to configure dynamically the update rate of the position-related data to fulfil different performances (e.g. power consumption, position service latency) or different location modes.

NOTE 5: 	for example, the 5G System needs to be able to request the UE to provide its location periodically with an update rate ranging from one location every [1 s to 10 s] in location normal mode to one location every [30 s to 300 s, or more] in location power saving mode. The 5G System needs to allow UEs to sleep for extended periods (e.g. one week), without requiring the UE to update its position data.

5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6 -1] The 5G system shall be able to support suitable API to enable application layer to configure conditions for active or inactive ranging operation conversion, e.g. when the ranging distance equals or is less than maximum of the “Active Ranging Distance”, it enters active ranging operation.

[PR 5.x.6 -2] The 5G system shall be able to dynamically adjust ranging operations with different ranging KPIs.

Note: The active ranging KPIs refers other use cases in this TR. The inactive ranging KPIs is in the table 5.x.6-1

Table 5.X.6-1 Inactive Ranging KPIs

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Inactive Ranging | [1]m | - | [95 %] | [5min] | >=[400]m | IC/PC/OOC | - | Static/ Moving (<1m/s) | [5min] | - | [40]UEs/ (104m2) |

NTE: 00- d through above fomular