3GPP TSG-SA WG1 Meeting #92e	S1-204165

Electronic Meeting, 24 - 28 Nov. 2020	(revision of S1-20xxxx)

Title:	update for 5.9 distance based intelligent perception for public safety

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

Abstract: This contribution proposes gap analysis for the use case about distance based intelligent perception for public safety in TR 22.855.

===================Proposed Changes==========================

## 5.9	Distance based Intelligent Perception for Public Safety

### 5.9.1	Description

Nowadays, COVID-19 outbreaks all over the world without mercy, it has seriously affected people life and economics. Even we are capable to find out who has gone to the infected zone, or the track of infected people based on the interaction between UE and base station, there is still no clue to figure out who is close contact with the one infected.

A smart device with distance based technique brings chances, it can record smart devices which appears nearby and stays for a long time. Ranging module makes it possible. Under emergency circumstances, people can active such a function to help government find out potential infected people more precisely and plot spreading diagram so as to stop serious spread at early time. In this way, many medical resources can be saved for testing or curing only the individual who is close exposure to the infected rather than wasting on everyone appears in high risk area.

There is no difference for the ranging in both indoor and outdoor case. When the two UEs are getting close to each other less than specific distance (e.g., 5m), one UE will mark the presence of the other UE (vice versa), so it is assumed that ranging distance is less than 20m. The availability is relax with 99%, since it is just used to track the presence of other UEs.  It assumes that there are 100UEs within the specific area (e.g., radius = 20m). The static/low moving UEs are still possible to get close to each other and being infected. The low moving UE could be for example pedestrian UE, or UE on bike/electrical bike, so the relative UE velocity is small than 20km/h.

### 5.9.2	Pre-conditions

Alice and Bob both wear smart devices (e.g. smarts phone, smart watch, etc), UE A and UE B, which are ranging capable and discoverable.

The function of distance based intelligent perception for public safety is already switched on.

### 5.9.3	Service Flows

- Alice with UE A meets Bob with UE B, they walk or run towards to each other under a particular relative velocity (e.g. 5km/h).

- UE A discovers UE B and UE B discovers UE A.

- Both UE A and UE B estimates the distance between each other. If the distance meets the criteria (e.g. smaller than 5 meter), both start a timer.

- After a talk, Alice and Bob say goodbye to each other, they walk towards different directions. If the distance between UE A and UE B exceed the limit (e.g. larger than 8 meter), the timers stop. At the same time, UE A generates a record including UE B’s identifier, location, time length, etc. And UE B performs same actions.

- UE A and UE B periodically upload records to the server in department responsible for public safety.

### 5.9.4	Post-conditions

Alice feels bad and goes to hospital, and medical testing shows Alice is infected. Hospital report Alice as one infected to the public safety department. Then, the department alarms people Alice met based on the record UE A uploaded. Bob is told to go to hospital for testing infection.

### 5.9.5	Existing features partly or fully covering the use case functionality

None.

### 5.9.6	Potential New Requirements needed to support the use case

The 5G system shall be able to support for a UE to discover other UEs supporting Ranging.

Table 5.9.6-1 – KPIs for distance based intelligent perception for public safety

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Coverage, environment of use, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation support in area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation support in area |
| distance based intelligent perception for public safety | 50cm | - | 99 % | - | 20m | IC/PC/OOC | LOS | Static/ Moving (<20km/h) | - | 100 | - |

=================== Proposed Changes ==========================