3GPP TSG-SA WG1 Meeting #98e 	S1-221105r5

Electronic Meeting, 9 – 19 May 2022	(revision of S1-22xxxx)

Source:	China Telecom, Huawei

pCR Title:	Use Case of Rainfall Monitoring

Draft Spec:	3GPP TR 22.837

Agenda item:	7.2

Document for:	Approval

Contact:	Yuying Zhang, zhangyy45@chinatelecom.cn

Abstract: This contribution presents the use case of rainfall monitoring for the FS_Sensing.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	 	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[y] 	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[z] 	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

* * * Second Change * * * *

5.x	Use case of rainfall monitoring

5.x.1	Description

Rainfall monitoring is a topic of great importance for several application contexts: hydraulic structure design, agriculture, weather forecasting, climate modelling, etc. At present, the most widely used measurement method is rain gauge.

Traditional rainfall monitoring use rain gauges, which are located at a particular location. Wide-area rainfall monitoring using traditional rain gauges would be costly. The base stations are deployed by the operators with radio cell planning that could cover a wider area. With base stations monitoring the rainfall, for example rain rate (mm/h), it could obtain a horizontally wider-area measurement.

Radio signals, as they propagate through the atmosphere, are reduced in intensity by constituents of the atmosphere. Oxygen and water vapor are the two major components which are responsible for the signal absorption. If it is a rainy day, an additional attenuation caused by rain further increases the propagation path loss.[z] The rain attenuation depends on the size and distribution of the water droplets, hence, by quantifying and modelling the base station signal measurements, we are able to know the rain rate.

The mmWave bands, such as 28GHz and 38GHz have been used to assess coverage, large-scale path loss, and fading and multipath effects[y]. Since the 28 GHz and 38 GHz bands are also licensed for wireless backhaul communications, these frequencies can used for rainfall monitoring[z].

The granularity of the rainfall monitoring could be smaller than the traditional measurements.

5.x.2	Pre-conditions

Peter is a farmer who takes care of a big farm that grows different crops. Peter needs to monitor the rainfall of his farm to manage reasonable irrigation, drainage and fertilizer. When there is less rainfall, Peter can select reasonable irrigation plans to improve the farmland water content condition. When there is high rainfall, Peter should improve the drainage system and fertilize the crops to avoid crop losses.

5.x.3	Service Flows

- Peter has a subscription for the premium service of rainfall monitoring for a more granular location.

- Peter is at daily working routine, and wants to check the timely rainfall information from the weather application on his phone.

- The base station obtains the NR based sensing measurement data every hour and the 5G system exposes NR based sensing measurement data or sensing resluts to the weather application via the core network.

- Based on the results of the NR based sensing measurement data, the application server obtains the rainfall associated with location information.

- Peter obtains timely rainfall information from weather application on his phone.

5.x.4	Post-conditions

Peter could check the rainfall information at any time on his phone. Based on the timely rainfall information, Peter could plan the irrigation, drainage and fertilizer for the crops in his farm.

5.x.5	Existing feature partly or fully covering use case functionality

None

5.x.6	Potential New Requirements needed to support the use case

[PR. 5.x.6 - 001] The 5G system shall support  NR based sensing, including collection of the NR based sensing measurement data.

[PR. 5.x.6 - 002] The 5G system shall support mechanisms for the operator to provide NR based sensing measurement data signal processing capabilities to derive the sensing results.

[PR. 5.x.6 - 003] The 5G system shall support to process the NR based sensing measurement data and associate it with other assistance information (e.g. location).

[PR. 5.x.6 - 004] The 5G system shall provide mechanisms for the operator to expose NR based sensing results to a trusted 3rd party application via the core network.

[PR. 5.x.6 – 005] The 5G system shall support sensing services with performance requirements as given in Table 5.x.6-1.

Table 5.x.6-1: Performance requirements for the use case of rainfall monitoring

| Scenario | Sensing Range | Sensing Accuracy | Sensing Interval |
| --- | --- | --- | --- |
| Rainfall monitoring | 200m (Note 1) | [±12mm/h] | [1min~15min] (Note 2) |
| Note 1: The sensing range for rainfall monitoring refers to the reference [x]. Note 2: The sensing interval depends on the requirement of the weather application. |  |  |  |

Editor’s Note: Performance requirements of the sensing accuracy and sensing interval need to be further studied.

* * * End of Change * * * *