3GPP TSG SA WG 1 Meeting #99e 	S1-222219

Electronic Meeting, 22 August – 1 September 2022

Title:	Use case store and forward - MO

Agenda item:	7.8

Source:	Novamint, Sateliot, GateHouse

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This contribution proposes a new use case to be captured in TR 22.865 v.0.0.0.

===================Proposed Change==========================

## 5.x	Use case on store and forward - MO

### 5.x.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Originated message.

A description of store and forward operation is provided in Annex B.

### 5.x.2	Pre-conditions

Company TrackingInc is offering a service of remote monitoring of fields and is deploying and tracking many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed are including a 5G communication with satellite access. Some of them are deployed in remote area where there is no coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc is using the service of IoTSAT for the 5G IoT connectivity by satellite and IoTSAT is using a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs are regularly sending information related to the area they are monitoring to the application server of TrackingInc and some times receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

In the present use case, the IoT remote monitoring UE is in a remote area with no ground stations available for feeder link connectivity and the IoT remote monitoring UE is aware that IoTSAT constellation operates in S&F mode.

### 5.x.3	Service Flows

The IoT remote monitoring UE needs to send a message to the TrackingInc application server. The UE waits for satellite network coverage and sends its message when the satellite is passing by.

The IoT remote monitoring UE and the satellite providing coverage interact over the service link, allowing the UE to transfer the message to the satellite, which has not connectivity to the ground segment and, consequently, has to store locally the received message.

At this point:

- Limitations to the size/amount of data that can be sent from the UE could be enforced.

- Delivery priority for the exchanged data could be established.

- Acknowledgement of the received data by the satellite could be issued.

At a later time, the satellite with the stored message establishes connectivity with the ground network via a feeder link and relays/forwards/downloads the message to the ground network.

The ground network, based on established connectivity configuration and routing, delivers message to the TrackingInc application server.

### 5.x.4	Post-conditions

The message generated by the IoT remote monitoring UE has been delivered successfully to the TrackingInc application server without relying on a continuous end-to-end network connectivity path between them.

### 5.x.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261, clause 6.3.2.3 on satellite access includes the following requirements:

“The 5G system shall be able to provide services using satellite access.”

“The 5G system with satellite access shall be able to support low power MIoT type of communications.”

However, it is not sufficient in regards of S&F operation especially for the delivery of delay-tolerant/non-real-time IoT NTN services with NGSO satellites.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] The 5G system with satellite access shall be able to support store and forward operation

[PR 5.x.6-002] The 5G system with satellite access shall be able to inform a UE that the satellite is operating in S&F mode.

[PR 5.x.6-003] If the 5G system with satellite access supports store and forward operation, it shall be able to inform a UE using the satellite access that the message will be stored due to no feeder link available.

[PR 5.x.6-004] If the 5G system with satellite access supports store and forward operation, it shall be able to inform a UE using the satellite access that there is a change of situation regarding the availability of the feeder link.

[PR 5.x.6-005] If the 5G system with satellite access supports store and forward operation, it shall be able to provide a validity period for the message to be stored.

[PR 5.x.6-006] If the 5G system with satellite access supports store and forward operation, it shall be able to inform a UE that the maximum capacity of storage is reached.

[PR 5.x.6-007] If the 5G system with satellite access supports store and forward operation, it shall support a mechanism to configure and provision specific required QoS and priority policies for S&F services (eg. delivery priority, acknowledgment policy, integrity and maximum allowed transfer delay…)