TSG-SA WG2 meeting #4	TSG-S2#4 Tdoc S2-99293

Yokohama, Japan

19th-21st April, 1999

Agenda Item: 	23.20 Key issues (CC/SM)

Source: 	Ericsson, Nokia and Nortel Networks

Title: 	Modified text of Section 7.4, UMTS 23.20 (modified S2-99271)

Document for:	Approval

## 7.4.	UMTS call control

### 7.4.1	Technical Requirements

The following technical requirements are applied to support multimedia in GSM/UMTS.

- P1) GSM/UMTS shall enable the provisioning of multimedia services and with multivendor interworking between UE and network.

- P2) Basic voice and PDP-context establishment shall be based on GSM CC/SM respectively.

- P3) Handover and roaming to and from GSM shall be supported provided GSM is capable of supporting the ongoing media service.

- P4) Ideas, concepts and procedures developed by other fora e.g. other standards bodies such as ITU, IETF etc. shall be included or referenced in the GSM/UMTS CC/SM when found suitable.

- P5) For multimedia services a multimedia CC/SM protocol could be run transparently via a PDP-context or a circuit-switched connection established using GSM SM/CC which would allow transparent handover and roaming between GSM and UMTS provided that GSM supports the QoS requirements.

- Figure X illustrates the realisation of the multimedia service based on P5).

- Figure X:  Support of multimedia making use of GSM SM/CC

Based on the requirements listed above, GSM CC/SM represented by GSM 04.08 forms a solid foundation for UMTS CC/SM for Release 99. UMTS CC/SM for Release 99 is to be developed from GSM CC/SM by introducing some well defined enhancements.

Existing (and future) multimedia protocols can be supported by the UMTS CC/SM as application layer protocols, with no (or in some instances only minor) impact to UMTS CC/SM.

## 7.4.2	Typical Scenarios for Multimedia Control and User Plane

Two typical call scenarios to support multimedia services, H.324 and H.323, respectively, are presented as examples. As an assumption, the calls are between the peer multimedia terminals over an IMT-2000 network. As shown  in the following sections, the multimedia signalling protocol and data transmission for both call scenarios can be performed end-to-end on the IMT-2000 user plane and is thereby transparent to the IMT-2000 Core Network. The IMT-2000 operators still control the multimedia service towards the end-user by providing the service via a service node (gateway, gatekeeper or application server) inside its own domain. Some other call scenarios e.g. IMT-2000 to ISDN/PSTN and/or IMT-2000 to the IP network can also be illustrated in a similar fashion.

### 7.4.2.1	H.324M to H.324M Call

In Figure Y, the H.324M IMT-2000 terminal initiates the call set-up procedure by sending a 04.08 SET-UP message to the originating MSC/VLR.

After the received 04.08 SETUP message, the originating MSC/VLR sends an ISUP Initial Address Message (IAM) to the terminating MSC/VLR. The terminating MSC/VLR performs a 04.08 SETUP procedure towards the H.324M UMTS terminal. The communication link is now established between the two H.324M endpoints.

The logical channels can now be established using the H.245 open logical channel procedure.

No gateway is needed in this case. This case is simple to support and requires little standardization.

The 04.08 Bearer Capability is used to indicate 64 kbits/s bit transparent case described in GSM 07.01 can be used, as well as H.223/H.245.

The 04.08 LLC is used to indicate H.223/H.245. This makes the called IMT-2000 mobile terminal activate its H.324M application when receiving the SETUP (LLC:H.223/H.245).

| Information Transfer Capability | Unrestricted Digital Information |
| --- | --- |
| Sync/Async | Synchronous |
| Connection Element | Transparent |
| Fixed Network User Rate | 64 kbit/s |

Figure Y:  UMTS H.324M – UMTS H.324M call example

### 7.4.2.2	IMT-2000 H.323 to H.323 call

Figure Z shows a Multimedia Call between two H.323 terminals within an IMT-2000 operator domain. Terminal A and B have performed Gatekeeper Identification and Gatekeeper Registration using a Best Effort (BE) Radio Access Bearer (RAB). Thereafter, the terminal A sets up a Real Time (RT) Radio Access Bearer (RAB) to decrease the time for the H.323 control signalling. This need of a Real Time Radio Access Bearer can be indicated from the terminal application to the mobile terminal through the Application Programming Interface (API). The terminal A performs PDP Context activation (see Figure Z) to set up the Real Time Radio Access Bearer. From now on, the established Real Time Radio Access Bearer can be used for H.225.0 RAS control signalling and Q.931 control signalling. After the Real Time Radio Access Bearer is established, the H.323 terminal A performs an Admission Request (ARQ) towards the Gatekeeper. If the terminal A is admitted the Gatekeeper answers with AdmissionConfirm (ACF) otherwise AdmissionReject (ARJ). Terminal A initiates the H.323 connection by sending a Q.931 Setup message to the Gatekeeper when the ACF has been received. The Gatekeeper answers with a Q.931 Call Proceeding to terminal A and sends a Q.931 Setup message to terminal B on a Best Effort (BE) Radio Access Bearer (RAB). Terminal B performs PDP Context Activation to SGSN and GGSN to set up a Real Time Radio Access Bearer and performs an Admission Request towards the Gatekeeper on this Real Time Radio Access Bearer. After this terminal B answers the received Q.931 Setup message with a Q.931 Alert and Connect message to the Gatekeeper on receipt of the Admission ConFirm (ACF) message. The Gatekeeper forwards these two messages to the terminal A. The Real Time Protocol (RTP) is now established between the terminal A and B for transmission of audio, video or data streams (see Figure Z). The Gatekeeper relays the data streams but  is not always completely transparent. The Gatekeeper can perform interworking functions e.g. Network Address Translation (NAT) etc between different networks. The voice transcoding is performed end-to-end in the H.323 terminal.

### Figure Z:  IMT-2000 H.323 – H.323 call example