SA WG2 Meeting #101	S2-140192

# 20 - 24 January 2014, Taipei, Taiwan	(revision of S2-14xxxx)

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.402 | CR | 1233 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Attribute alignment with HS2.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | WLAN_NS |  |  |  |  |  |  | Date: |  | 2014-01-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | In the definition of “PreferrdRoamingParterList” in HS2.0 Release 2 specification, the priority for each FQDN is mandatory, thus WLANs matching different FQDNs may have different priorities. The attribute in WLANSP should be aligned with HS2.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Each FQDN in the PerferredRoamingList in WLANSP is assigned a priority. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The priority for the FQDNs in PreferredRoamingParterList in WLANSP is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

4.8.2.1.6	WLAN Selection Policy

The WLAN Selection Policy (WLANSP) is a set of operator-defined rules that determine how the UE selects and reselects a WLAN access network. The UE may be provisioned with WLANSP rules from multiple PLMNs.

Each WLANSP rule includes the following information:

-	Validity conditions, i.e. conditions indicating when the provided rule is valid. The validity conditions can include the time of day, geolocation, network location (e.g. PLMN, Location Area), etc.

-	A list of SSIDs not preferred for selection. This list is the same as the SPExclusionList specified in the HS2.0 Rel-2 specification [75].

-	One or more groups of WLAN selection criteria in priority order. Each group contains one or more criteria that should be fulfilled by a WLAN access network in order to be eligible for selection. Such criteria are based on the following and only the following WLAN attributes:

a)	Attributes defined in the HS2.0 Rel-2 specification [75]:

-	PreferredRoamingPartnerList,

-	MinimumBackhaulThreshold,

-	MaximumBSSLoad,

-	RequiredProtoPortTuple.

b)	Additional attributes:

-	PreferredSSIDList: A list of SSIDs preferred for selection.

For example, a WLANSP rule may include the following groups of selection criteria:

-	Group of selection criteria with priority 1:

-	PreferredRoamingPartnerList = partner1.com with priority 1, partner2.com with priority 2

-	MaximumBSSLoad = 60

-	Group of selection criteria with priority 2:

-	PreferredSSIDList = myoperator

-	MinimumBackhaulThreshold = 2Mbps in the downlink

A WLAN access network meets a group of selection criteria when it concurrently fulfills all the criteria in the group.

END OF CHANGES