SA WG2 Meeting #S2#106	S2-143945

17 - 21 November 2014, San Francisco, USA	(revision of S2-14xxxx)

Source:	Ericsson

Title:	Using Cancel Location with Reattach at UE Usage Type updates

Document for:	Approval

Agenda Item:	7.6

Work Item / Release:	DECOR / REL-13

Abstract of the contribution: A new simple 5.2.4 solution is provided

***** First Change *****

### 5.2.4	HSS-initiated Dedicated CN Node Reselection

#### 5.2.4.1	Scenario Description

In this case, based on subscription parameter changes in the HSS, the CN nodes for the UE need to be changed.

The subscription changes may be applied to a large number of devices and the same considerations as in the case of MME rebalancing as in 23.401 [4] section 4.3.7.3 should be considered. Gradual rather than sudden redirection of the UEs should be performed as a sudden redirection of large number of subscribers could overload the core network nodes (and possibly the RAN if paging is needed) in the redirected core-network pool. With minimal impact on network and the user's experience, the subscribers should be off-loaded within a reasonable amount of time.

The approach shall also enable transfer of large groups of subscribers to another CN when the operator of the serving network changes the configuration, which determines which dedicated CN serves which UE Usage Types.

#### 5.2.4.2	Solutions

##### 5.2.4.2.x	Solution x: Using Cancel Location with Reattach for HSS-initiated Dedicated CN Node Reselection

Given that changing the UE Usage Type in the HSS subscription information should be a rare event, it is considered acceptable to do a reattach in such rare occasions. This allows existing procedures to be used. There are no impacts on any interfaces.

HSS

MME/SGSN

UE

1. Cancel Location Request

2. Cancel Location Answer

3. Detach with re-attach required

4. Re-Attach to correct Dedicated CN (i.e. using new UE Usage Type in updated HSS information)

Figure 5.2.4.2.x-1: HSS Initiated CN Node Change based on CLR/CLA

1. 	The HSS detects that the UE Usage Type has changed in the HSS. The HSS may send a Cancel Location Request (Cancellation Type = Subscription Withdrawal, CLR-flag = Reattach-Required) message to the MME/SGSN.

2.	The MME/SGSN acknowledges the CLR by sending a Cancel Location Answer message to the HSS.

3. 	The MME/SGSN makes a network initiated Detach. The re-attach required flag is set to the UE.

4.	The UE makes a new attach to the network and gets steered to the correct Dedicated CN based on the updated CN Usage Type in the HSS subscription information.

***** End of Changes *****