SA WG2 Meeting #106	S2-144138

17 - 21 November 2014, San Francisco, California, USA	(revision of S2-14xxxx)

Source:	Samsung

Title:	Voice with WebRTC and QCI-1 handling

Document for:	Approval

Agenda Item:	7.12

Work Item / Release:	eWebRTCi / Rel-13

Abstract of the contribution: This P-CR discusses how to avoid false SRVCC trigger if QCI-1 is used by WIC for voice session.

1. Introduction

In addition to IMS based voice services over LTE (VoLTE), WebRTC/WIC can also be used to provide voice communication services and QCI-1 usage should be allowed. However, UE can also be SRVCC capable and the QCI-1 usage may become a false trigger for SRVCC. For example, eNB determines the handover to CS RAT is required based on the presence of QCI-1 bearer and “SRVCC possible indication” from MME but the voice session is initiated by WIC which means the QCI-1 bearer will be deactivated by MME and UE will not be able to continue the voice session over PS RAT or convert the voice to CS RAB like SRVCC operation.

This paper discussed how to avoid SRVCC false trigger.

2. Proposal

To add the following to TR 23.706.

*** begin next texts ***

### 7.2.x	Solution 2: Avoiding SRVCC false trigger

#### 7.2.x.1	Solution Principles

eNB will not trigger the SRVCC handover if certain pre-condition(s) are not met and/or an explicit indication is given to eNB that SRVCC is not allowed.

Option 1: If voice session is initiated by WIC, UE ensures its SRVCC network capability is set to “not support” in the MME. This can be done by UE initiating TAU with an indication that SRVCC is not supported. However, this will not take effect until the next IDLE to ACTIVE state because “SRVCC operation possible” indication is only given in S1 AP Initial Context Setup Request.

Option 2: When QCI-1 is established for a session handled by eP-CSCF, PCRF includes an explicit indication in addition to QCI-1 (similar to video SRVCC) to indicate to eNB that this QCI-1 is not eligible for SRVCC.

Option 3: define new QCI-x with same characteristic as QCI-1.

Option 4: When MME receives the SRVCC handover required message from source eNB, MME determines that this QCI-1 is not established for the PDN connection that is established for well-known IMS APN and rejects the SRVCC handover request or perform the PS handover including the QCI-1.

Option 5: During dedicated bearer setup with QCI-1, MME aware that this is not related to the well-know IMS APN and gives an explicit indication to eNB that this bearer is not eligible for SRVCC.

## 7.3	Evaluation

TBD

## 7.4	Conclusion

TBD