SA WG2 Meeting #116	S2-164811

29 August - 2 September 2016, Sanya, China

Source:	Sandvine

Title:	TDF User plane selection mechanism unsolicited mode

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	CUPS / Rel-14

Abstract of the contribution: This contribution proposes a process allowing TDF-U to initiate a session towards TDF-C in case of TDF operating in unsolicited application reporting mode

### Introduction

The proposal in S2-163658 is already addressing some issues in the Sx session initiation process between control and user plane element. However, the TDFs unsolicited application reporting mode was missed from that proposal. In section 5b.2 of 3GPP TS 29.212 is says:

“The Diameter session on Sd is established either at the request of the PCRF in case of solicited application reporting or at the request of the TDF in case of unsolicited application reporting. Session modifications may be initiated by either TDF or PCRF. Session termination is initiated at the request of the PCRF as specified in clause 4b.5.4.”

Since the trigger (in case unsolicited application reporting) comes from the detection of application, and not driven by PDN connection establishment from the PCRF, the TDF-C cannot be aware of the need to initiate the Sx session towards the TDF-U. Therefore, for compliance of separated control and user place mode with the exiting unsolicited application reporting, it is proposed to add the following texts into the TS 23.214 v0.1.? (in addition to the text proposed in S2-163658):

* * * Start of Change * * * *

### 5.x.6	Session Establishment in case of Unsolicited Application Reporting Mode

In case that TDF is operating in unsolicited application reporting mode, upon IP-CAN session establishment, the PCRF does not establish an Sd session towards the TDF-C. Once traffic starts, and the TDF-U detect that an application needs to be reported to the PCRF (based on a predefined ADC rule), it will establish an Sx session towards the TDF-C, which, in turn, establish an (unsolicited) Sd session towards the PCRF.

The session will be established towards the TDF-C to which the TDF-U is connected.

* * * End of Changes * * * *