SA WG2 Meeting #S2-120	S2-171660

27 - 31 March 2017, Busan, South Korea

3GPP TSG-RAN WG2#97			R2-1702368

Athens, Greece, 13 - 17 February 2017

Title:	Reply to LS on user plane security termination

Response to:	LS (R2-1702205/ S3-170408) on user plane security termination

Release:	Release 14

Work Item:	FS_NR_newRAT

Source:	TSG RAN WG2

To:	TSG SA WG3

Cc:	TSG RAN WG3, SA WG2

Contact Person:

#### Name:	Esa Malkamäki

E-mail Address:	esa.malkamaki@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA3 for their LS on “user plane security termination.”

RAN2 would like to inform SA3 that RAN2 assumes (and will continue to assume in work item phase) that the security will be provided by PDCP (=Proposal 1) unless a different or additional requirement comes from SA3.

RAN2 has discussed the four different proposals. As indicated above, Proposal 1 is the baseline solution and the preference for RAN2. Furthermore, RAN2 understanding of terminating the security in a new logical entity (UP-STF) (Proposal 2, 3 and 4) is the following:

- Because ciphering (and integrity protection) requires a COUNT, the UP-STF would need its own sequence number.

- Because header compression cannot be performed after ciphering, the UP-STF would also need to host the header compression functions (e.g. ROHC).

- Because in-sequence delivery may not always be guaranteed (mobility events, re-routing), the UP-STF would need to handle the reception of packets out-of-order.

- Because ciphering protects IP headers, the radio protocols below the UP-STF would not be aware of the kind of traffic being carried.

The resulting impacts are:

- Increased overhead due to the new termination protocol.

- Increased complexity of the UP-STF function, decreased complexity of PDCP due to new location of ROHC.

- The sequence number of the UP-STF would have to be large enough to avoid a possible wrap around to handle out-of-order packets.

- Optimisations relying on IP header visibility would not be possible (e.g. DRX timer settings, smart RRM and smart scheduling strategies, codec rate adaptation…).

- Potential emerging services such as caching service at the RAN would not be possible.

Therefore, the RAN2 clear preference is to have security in RAN in the PDCP layer.

RAN2 would also like to point out that regardless of the location of user plane security termination, the control plane security termination must remain in the 5G Access Network and the provision of security keys to the gNB needs to be supported anyway.

2. Actions:

To SA3 group.

ACTION: 	Please consider the RAN2 preference for user plane security termination.

3. Date of Next TSG-RAN WG2 Meeting:

TSG-RAN WG2 #97bis		2017.04-03 – 2017.04.07	Spokane