SA WG2 Meeting #122E e-meeting	S2-176767

11 – 15 September 2017, Elbonia	(revision of S2-17xxxx)

Source:	LG Electronics

Title:	TS 23.502: Alignment of N1 SM information/container

Document for:	Approval

Agenda Item:	22

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposed to clarify the use of “N1 SM information” in the SM procedures.

1.	Discussion

In the current TS 23.501, the NAS protocol stack between the UE and the CN NFs is shown as below:

Figure 8.2.2.3-1: Control Plane protocol stack between the UE and the SMF

The SM message is transferred between the UE and the SMF, via the AN and AMF. At some point, NAS-SM message is encapsulated in a single container (e.g. Namf or Nsmf service message) or twice (e.g. NAS-SM message is encapsulated by NAS-MM message and then NAS-MM is also encapsulated by N2 or AN message). In the current SM procedures, the encapsulation is specified as “N1 SM information” or “N1 SM container” in the NAS-MM message and N11 service procedures.

The figure shown below depicts each message type that contains NAS-SM message, including AN message (e.g. RRC), N2 message and N11 (or Namf/Nsmf service) message.

In the SM procedures described in TS 23.502, two kinds of terminologies, “N1 SM information” and “N1 SM container”, are both being used. Since the NAS-SM message is piggybacked in those messages (NAS-MM, N11), “N1 SM Container” seems more suitable terminology. So we would like to update “N1 SM information” to the “N1 SM container”

Also the description of messages in the SM procedures are modified to align with the structure shown above.

2.	Proposal

According to the discussion above, we would like to propose to add the following texts in TS 23.502

* * * * Start of 1st Change * * * *

4.3.2.2.1	Non-roaming and Roaming with Local Breakout

This clause specifies PDU Session establishment in the non-roaming and roaming with local breakout cases. The procedure is used to establish a new PDU session as well as to hand over an existing PDU Session between 3GPP access and non-3GPP access.

In case of roaming, the AMF determines if a PDU session is to be established in LBO or Home Routing. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Figure 4.3.2.2.1-1: UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

The procedure assumes that the UE has already registered on the AMF thus the AMF has already retrieved the user subscription data from the UDM.

1.	From UE to AMF: NAS Message (S-NSSAI, DNN, PDU Session ID, Request type, N1 SM container (PDU Session Establishment Request)).

In order to establish a new PDU session, the UE generates a new PDU Session ID.

The UE initiates the UE Requested PDU Session establishment procedure by the transmission of a NAS message containing a PDU Session Establishment Request within the N1 SM container. The PDU Session Establishment Request may include a PDU Type, SSC mode, Protocol Configuration Options.

The Request Type indicates "Initial request" if the PDU Session Establishment is a request to establish a new PDU Session and indicates "Existing PDU Session" if the request refers to an existing PDU Session between 3GPP access and non-3GPP access. The Request Type indicates "Emergency Request" if the PDU session Establishment is a request to establish a PDU session for emergency bearer services. The Request Type indicates "Existing Emergency PDU Session" if the request refers to an existing PDU Session for emergency services between 3GPP access and non-3GPP access.

The NAS message sent by the UE is encapsulated by the AN in a N2 message towards the AMF that should include User location information and Access Technology Type Information.

The PDU Session Establishment Request message may contain SM PDU DN Request Container containing information for the PDU session authorization by the external DN.

The AMF receives from the AN the NAS SM message (built in step 1) together with User Location Information (e.g. Cell Id in case of the RAN).

The UE shall not trigger a PDU session establishment for a PDU session corresponding to a LADN when the UE is outside the area of availability of the LADN.

2.	The AMF determines that the message corresponds to a request for a new PDU Session based on that Request Type indicates "initial request" and that the PDU Session ID that is not used for any existing PDU Session(s) of the UE. If the NAS message does not contain an S-NSSAI, the AMF may determine a default S-NSSAI for the requested PDU session either according to the UE subscription, if it contains only one default S-NSSAI, or based on the operator policy. The AMF selects an SMF as described in TS 23.501 [2], clause 6.3.2. The AMF stores an association of the PDU session ID and the SMF ID.

The case where the Request Type indicates "Existing PDU Session", and either the AMF does not recognize the PDU Session ID or the subscription context from UDM does not contain an SMF ID corresponding to the DNN constitutes an error case.

If the Request Type indicates "Emergency Request" or "Existing Emergency PDU Session", the AMF selects the SMF as described in TS 23.501 [2], clause 5.16.4.

3.	From AMF to SMF: Nsmf_PDUSession_CreateSMRequest (SUPI, DNN, S-NSSAI, PDU Session ID, AMF ID, Request Type, N1 SM container (PDU Session Establishment Request), User location information, Access Technology Type, PEI).

The AMF ID uniquely identifies the AMF serving the UE. The AMF forwards the PDU Session ID together with the N1 SM container containing the PDU Session Establishment Request received from the UE. The PEI (Permanent Equipment Identifier) is defined in TS 23.501 [2] clause 5.9.3.

4.	If the SMF has not yet registered and subscription data is not available, then the SMF registers with the UDM, retrieves subscription data and subscribes to be notified when subscription data is modified.

Editor's note: Whether registration and subscription data retrieval happen in 1 or 2 steps and the use of corresponding service operations is FFS. Whether indicating "subscriber data retrieval" in the Nudm_UEContext_Management service is possible is FFS.

If the Request Type in step in step 3 indicates "Existing PDU Session" the SMF determines that the request is due to handover between 3GPP access and non-3GPP access. The SMF identifies the existing PDU Session based on the PDU Session ID.

Subscription data includes the authorized PDU type(s), authorized SSC mode(s), default 5QI/ARP, subscribed Session-AMBR.

Editor's note:	Further detailing of SM-related subscription data is FFS.

The SMF checks whether the UE request is compliant with the user subscription and with local policies. If the DNN corresponds to an LADN, the SMF verifies whether the UE is located within the LADN service area based on the UE location reporting from the AMF. If that is not the case the SMF rejects the UE request via NAS SM signalling by responding to the AMF with Nsmf_PDUSession_CreateSMResponse including a relevant SM rejection cause. The SMF indicates to the AMF that the PDU session ID is to be considered as released, it deregisters from UDM and the rest of the procedure is skipped.

5.	SMF to DN via UPF

If the SMF needs perform secondary authorization/authentication during the establishment of the PDU session by a DN-AAA server as described in clause 5.6.6 of TS 23.501 [2], the SMF selects an UPF as described in TS 23.501 [2] clause 6.3.3 and triggers the PDU session establishment authentication/authorization as described in clause 4.3.2.3.

If the PDU session establishment authentication/authorization fails, the SMF terminates the PDU session establishment procedure and indicates a rejection to the UE.

6a.	If dynamic PCC is deployed, the SMF performs PCF selection. If dynamic PCC is not deployed, the SMF may apply local policy.

6b.	The SMF may invoke the Npcf_SMPolicyControl_Get operation (see clause 5.2.5.4.2) to establish a PDU-CAN session with the PCF and get the default PCC Rules for the PDU Session. If the Request Type in step 3 indicates "Existing PDU Session", the SMF may notify an event previously subscribed by the PCF by invoking the Nsmf_EventExposure_Notify operation and the PCF may update policy information in the SMF by invoking Npcf_SMPolicyControl_UpdateNotify operation (see clause 5.2.5.5.3 for details). The PCF may provide authorized Session-AMBR and the authorized 5QI/ARP to SMF.

6c.	The PCF subscribes to the IP allocation/release event in the SMF (and may subscribe other events) by invoking the Nsmf_EventExposure_Subscribe operation.

NOTE 1:	The purpose of step 6 is to receive PCC rules before selecting UPF. If PCC rules are not needed as input for UPF selection, step 6 can be skipped.

7.	If the Request Type in step 3 indicates "Initial request", the SMF selects an SSC mode for the PDU Session. If step 5 is not performed, the SMF also selects an UPF as described in TS 23.501 [2] clause 6.3.3. In case of PDU Type IPv4 or IPv6, the SMF allocates an IP address/prefix for the PDU Session as described in TS 23.501 [2] clause 5.8.1. For Unstructured PDU Type the SMF may allocate an IPv6 prefix for the PDU session and N6 point-to-point tunnelling (based on UDP/IPv6) as described in TS 23.501 [2] clause 5.6.9.4.

If the Request Type indicates "Emergency Request" or "Existing Emergency PDU Session", the SMF selects the UPF as described in TS 23.501 [2], clause 5.16.4.

8a.	SMF may invoke the Nsmf_EventExposure_Notify service operation to report some event to the PCF that has previously subscribed. If Request Type is "initial request" and dynamic PCC is deployed and PDU Type is IPv4 or IPv6, SMF notifies the PCF (that has previously subscribed) with the allocated UE IP address/prefix. The PCF may provide authorized Session-AMBR and the authorized 5QI/ARP to SMF.

8b.	PCF may provide updated policies to the SMF by invoking the Npcf_SMPolicyControl_UpdateNotify service operation (see clause 5.2.5.5.3).

9.	If Request Type indicates "initial request" and step 5 was not performed, the SMF initiates an N4 Session Establishment procedure with the selected UPF, otherwise it initiates an N4 Session Modification procedure with the selected UPF:

9a.	The SMF sends an N4 Session Establishment/Modification Request to the UPF and provides Packet detection, enforcement and reporting rules to be installed on the UPF for this PDU Session. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to UPF in this step.

9b.	The UPF acknowledges by sending an N4 Session Establishment/Modification Response. If CN Tunnel Info is allocated by the UPF, the CN Tunnel Info is provided to SMF in this step.

10.	SMF to AMF: Nsmf_PDUSession_CreateSM Response (Cause, N2 SM information (PDU Session ID, QoS Profile(s), CN Tunnel Info, S-NSSAI, Session-AMBR), N1 SM container (PDU Session Establishment Accept (Authorized QoS Rule, SSC mode, S-NSSAI, allocated IPv4 address, Session-AMBR))).

Editor's note:	It is FFS how the AMF and SMF associate the N11 transactions with a corresponding UE context.

The N2 SM information carries information that the AMF shall forward to the (R)AN which includes:

-	The CN Tunnel Info corresponds to the Core Network address of the N3 tunnel corresponding to the PDU session.

-	The QoS Profile provides the (R)AN with the mapping between QoS parameters and QoS Flow Identifiers. Multiple QoS profiles can be provided to the (R)AN. This is further described in TS 23.501 [2] clause 5.7.

-	The PDU Session ID may be used by AN signalling with the UE to indicate to the UE the association between AN resources and a PDU session for the UE.

S-NSSAI is corresponding to the PDU session.

The N1 SM container contains the PDU Session Establishment Accept that the AMF shall provide to the UE.

Multiple Authorized QoS Rules may be included in the PDU Session Establishment Accept within the N1 SM container and in the N2 SM information.

The SM Response further contains PDU Session ID and information allowing the AMF to know which the target UE as well to determine which access towards the UE to use.

NOTE 2:	The access information is to deal with the case where a UE is simultaneously connected over 3GPP and Non 3GPP access.

The SMF shall subscribe to the UE location reporting in case of successful PDU session establishment providing reporting criteria (e.g. UE location with respect to the area of LADN availability for a LADN).

11.	AMF to (R)AN: N2 PDU Session Request (N2 SM information, NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept))).

The AMF sends the NAS message containing PDU Session ID and PDU Session Establishment Accept targeted to the UE and the N2 SM information received from the SMF within the N2 PDU Session Request to the (R)AN.

12.	(R)AN to UE: The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE establishing the necessary RAN resources related to the Authorized QoS Rules for the PDU Session request received in step 10.

(R)AN also allocates (R)AN N3 tunnel information for the PDU Session. In case of Dual Connectivity, the Master RAN node may assign some (zero or more) QFIs to be setup to a Master RAN node and others to Secondary RAN node. The RAN tunnel information includes a tunnel endpoint for each involved RAN node, and the QFIs assigned to each tunnel endpoint. A QFI can be assigned to either the Master RAN node or the Secondary RAN node and not to both.

(R)AN forwards the NAS message (PDU Session ID, N1 SM container (PDU Session Establishment Accept)) provided in step 10 to the UE. (R)AN shall only provide the NAS message to the UE if the necessary RAN resources are established and the allocation of (R)AN tunnel information are successful.

13.	(R)AN to AMF: N2 PDU Session Response (PDU Session ID, Cause, N2 SM information (PDU Session ID, (R)AN Tunnel Info, List of accepted/rejected QoS profile(s))).

The (R)AN Tunnel Info corresponds to the Access Network address of the N3 tunnel corresponding to the PDU session.

14.	AMF to SMF: Nsmf_PDUSession_ UpdateSMContext Request (N2 SM information).

The AMF forwards the N2 SM information received from (R)AN to the SMF.

Editor's note:	It is FFS if steps are needed to indicate from the UE to the core network that the UE has successfully established the PDU Session, or whether it is sufficient with a successful establishment in (R)AN indicated in step 12. For example, it is FFS if the UE shall send a NAS PDU Session Establishment Complete message to indicate that the UE has successfully established the PDU Session.

15a.	If the N4 session for this PDU Session was not established already, the SMF initiates an N4 Session Establishment procedure with the UPF. Otherwise, the SMF initiates an N4 Session Modification procedure with the UPF. The SMF provides AN Tunnel Info and CN Tunnel Info. The CN Tunnel Info only needs to be provided if the SMF selected CN Tunnel Info in step 8.

NOTE 3:	If the PDU Session Establishment Request was due to mobility between 3GPP and non-3GPP access, the downlink data path is switched towards the target access in this step.

15b.	The UPF provides a N4 Session Establishment/Modification Response to the SMF.

16.	SMF to AMF: Nsmf_PDUSession_ UpdateSMContext Response (Cause).

After this step, the AMF forwards relevant events to the SMF, e.g. at handover where the (R)AN Tunnel Info changes or the AMF is relocated.

Editor's note:	It is FFS if the SMF has to explicitly subscribe to these events or whether the subscription is implicit.

17.	SMF to UE, via UPF: In case of PDU Type IPv6, the SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the UPF.

18.	If the PDU Session Establishment request is due to a handover between 3GPP access and non-3GPP access, i.e. the Request type is set to "existing PDU Session" the SMF executes the steps 2a-9 specified in figure 4.3.4.2-1 in order to release the user-plane over the source access (3GPP or non-3GPP access).

19.	If the SMF identity is not included in the DNN subscription context, either stored already in SMF if it was previously registered or provided in steps 4a-4b by the UDM, the SMF invokes the Nudm_UEContextManagement_Update service operation including the SMF address, the DNN and the PDU Session Id. The UDM stores the SMF identity, SMF address and the associated DNN and PDU Session Id.

If during the procedure the PDU session establishment is not successful, the SMF informs the AMF. Once the AMF associates an SMF with a PDU session ID, the SMF is automatically subscribed to notifications of incoming N1 signalling associated with this PDU session ID. These notifications automatically provide any User Location information and Access Type that the AMF has received from the (R)AN in association with N1 signalling.

* * * * Start of 2nd Change * * * *

4.3.2.2.2	Home-routed Roaming

This procedure is used in case of home-routed roaming scenarios.

Figure 4.3.2.2.2-1: UE-requested PDU Session Establishment for home-routed roaming scenarios

1.	This step is the same as step 1 in clause 4.3.2.2.1.

2.	As in step 2 of clause 4.3.2.2.1 with the addition that the AMF also selects a SMF in HPLMN. The AMF stores an association of the PDU session ID and the SMF ID in VPLMN.

3.	As in step 3 of clause 4.3.2.2.1 with the addition that the AMF also provides the identity of the SMF in HPLMN it has selected in step 2 The AMF ID uniquely identifies the AMF serving the UE. The H-SMF is provided when the PDU Session is home-routed. The N1 SM container contains the PDU Session Establishment Request received from the UE.

Editor's note:	The parameter S-NSSAI may need to be updated to reflect agreements regarding slice selection parameters. This is FFS.

4.	The V-SMF selects a UPF in VPLMN as described in TS 23.501 [2], clause 6.3.3.

5.	The V-SMF initiates an N4 Session Establishment procedure with the selected V-UPF:

5a.	The V-SMF sends an N4 Session Establishment Request to the V-UPF. If CN Tunnel Info is allocated by the SMF, the CN Tunnel Info is provided to V-UPF in this step.

5b.	The V-UPF acknowledges by sending an N4 Session Establishment Response. If CN Tunnel Info is allocated by the V-UPF, the CN Tunnel Info is provided to V-SMF in this step.

6.	V-SMF to H-SMF: Create PDU Session Request (SUPI, DNN, S-NSSAI, PDU Session ID, V-SMF ID, V-CN-Tunnel-Info, PDU type, Protocol Configuration Options, User location information, SM PDU DN Request Container).

7-12.	These steps are the same as steps 4-9 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Home PLMN;

-	The H-SMF stores an association of the PDU Session and V-SMF ID for this PDU Session for this UE.

13.	H-SMF to V-SMF: Create PDU Session Response (Authorized QoS Rule, SSC mode, H-CN Tunnel Info, QoS profile(s), Session-AMBR)

The H-CN Tunnel Info contains the tunnel information for up-link traffic towards H-UPF.

Multiple Authorized QoS Rules may be included in the Create PDU Session Response.

The H-SMF shall send to the V-SMF the indication, if the VPLMN is allowed to insert an UL CL or a Branching Point for a PDU session towards this DNN.

14-18.	These steps are the same as steps 10-14 in clause 4.3.2.2.1 with the following differences:

-	These steps are executed in Visited PLMN;

-	The V-SMF stores an association of the PDU Session and H-SMF ID for this PDU Session for this UE.

19a.	The V-SMF initiates an N4 Session Modification procedure with the V-UPF. The V-SMF provides Packet detection, enforcement and reporting rules to be installed on the V-UPF for this PDU Session, including AN Tunnel Info, H-CN Tunnel Info and V-CN Tunnel Info.

19b.	The V-UPF provides a N4 Session Establishment/Modification Response to the V-SMF.

After this step, the V-UPF delivers any down-link packets to the UE that may have been buffered for this PDU Session.

20.	This step is the same as step 18 in clause 4.3.2.2.1. with the following differences:

-	The SMF is a V-SMF

21.	H-SMF to UE, via H-UPF and V-UPF in VPLMN: In case of PDU Type IPv6, the H-SMF generates an IPv6 Router Advertisement and sends it to the UE via N4 and the H-UPF and V-UPF.

22.	H-SMF may register the address to UDM as Step 19 of 4.3.2.2.1 The UDM stores the H-SMF identity and the associated DNN.

NOTE:	The SMF in HPLMN may initiate step 21 already after step 13.

* * * * Start of 3rd Change * * * *

4.3.3.2	UE or network requested PDU session modification (non-roaming and roaming with local breakout)

The UE or network requested PDU session modification procedure (non-roaming and roaming with local breakout scenario) is depicted in figure 4.3.3.2-1.

Figure 4.3.3.2-1: UE or network requested PDU session modification (for non-roaming and roaming with local breakout)

1.	The procedure is triggered by following steps

1a.	The UE initiates the PDU session modification procedure by the transmission of an NAS message (N1 SM container (PDU Session Modification Request), PDU session ID) message. Depending on the access type, if the UE was in CN-IDLE mode, this SM-NAS message is preceded by the Service Request procedure. The NAS message is forwarded by the (R)AN to the AMF with an indication of User location Information. The AMF invokes Nsmf_PDUSession_UpdateSMContex (PDU session ID, N1 SM container (PDU Session Modification Request)).

Editor's note:	What QoS parameters the UE provides for QoS session modification is FFS.

1b.	The PCF invokes the Npcf_SMPolicyControl_UpdateNotify service operation (see clause 5.2.5.5.3) to notify SMF for initiating the modification of policies procedure upon policy decision or upon AF requests and the modification requires signalling to the UE and/or to the AN.

1c.	The UDM updates the subscription data of SMF by Nudm_SubscriberData_UpdateNotification (SUPI, Subscription Data). The SMF updates the Subscription Data and acknowledges the UDM by returning an Ack with output (SUPI).

1d.	The SMF may decide to modify PDU session. This procedure also may be triggered based on locally configured policy.

If the SMF receives one of the triggers in step 1a ~ 1d, the SMF starts SMF requested PDU session modification procedure.

1e.	If notification control is configured for a GBR Flow, when the (R)AN decides the QoS targets of the flow cannot be fulfilled. (R)AN sends the N2 message (PDU session ID, N2 SM information) to the AMF. The N2 SM information includes the QFI, User location Information and a notification indicating that the QoS targets cannot be fulfilled. The AMF invokes Nsmf_PDUSession_UpdateSMContext (N2 SM information).

2.	The SMF may need to report some subscribed event to the PCF by invoking Nsmf_EventExposure_Notify service operation. The PCF may provide new policy information to the SMF by invoking Npcf_SMPolicyControl_UpdateNotify service operation (see clause 5.2.5.5.3). This step may be skipped if PDU session modification procedure is triggered by step 1b or 1d. If dynamic PCC is not deployed, the SMF may apply local policy to decide whether to change the QoS profile.

Steps 3 to 7 are not invoked when the PDU session modification requires only action at an UPF (e.g. gating).

3a.	For UE initiated modification, the SMF responds to the AMF through Nsmf_PDUSession_UpdateSMContext (N2 SM information (PDU Session ID, QoS Profile, Session-AMBR), N1 SM container (PDU Session Modification Command (PDU session ID, QoS rule, Session-AMBR))). See TS 23.501 [2] clause 5.7 for the QoS Profile and QoS rule.

The N2 SM information carries information that the AMF shall provide to the (R)AN. It includes the QoS profiles that were added, removed or modified.

The N1 SM container carries the PDU Session Modification Command that the AMF shall provide to the UE.

3b. For network initiated modification, the SMF invokes Namf_Communication_N1N2MessageTransfer (N2 SM information (PDU Session ID, QoS Profile, Session-AMBR), N1 SM container (PDU Session Modification Command (PDU session ID, QoS rule, Session-AMBR))).

If the UE is in CM-IDLE state and an ATC is activated, the AMF updates and stores the UE context based on the Namf_Communication_N1N2MessageTransfer and steps 4, 5, 6 and 7 are skipped. When the UE is reachable e.g. when the UE enters CM-CONNECTED state, the AMF forwards the N1 message to synchronize the UE context with the UE.

4.	The AMF may send N2 PDU Session Request (N2 SM information received from SMF, NAS message (PDU Session ID, N1 SM container (PDU Session Modification Command))) Message to the (R)AN.

5.	The (R)AN may issue AN specific signalling exchange with the UE that is related with the information received from SMF. For example, in case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE modifying the necessary RAN resources related to the PDU Session.

The UE acknowledges the PDU Session Modification Command by sending a NAS message (PDU session ID, N1 SM container (PDU Session Modification Command Ack)) message .

6.	The (R)AN may acknowledge N2 PDU Session Request by sending a N2 PDU Session Ack (QFI(s), RAN tunnel information, NAS message, User location Information) Message to the AMF. In case of Dual Connectivity, if one or more QFIs were added to the PDU session, the Master RAN node may assign one of these QFIs to a RAN node which was not involved in the PDU session earlier, then the RAN tunnel information includes a new N3 tunnel endpoint for QFIs assigned to the new RAN node. Correspondingly, if one or more QFIs were removed from the PDU session, a RAN node may not be involved in the PDU session anymore, and the corresponding tunnel endpoint is removed from the RAN tunnel information.

7.	The AMF forwards the N2 SM information and the N1 SM container (PDU Session Modification Command Ack) received from the AN to the SMF via Nsmf_PDUSession_UpdateSMContext service operation.

8.	The SMF may update N4 session of the UPF(s) that are involved by the PDU session modification by sending N4 Session Modification Request (N4 Session ID) message to the UPF.

NOTE:	The UPF that are impacted in the PDU session modification procedure depends on the modified QoS parameters and on the deployment. For example in case of the session AMBR of a PDU session with an UL CL changes, only the UL CL is involved.

9.	If the SMF interacted with the PCF in step 1b or 2, the SMF notifies the PCF whether the PCC decision could be enforced or not by invoking Nsmf_EventExposure_Notify service operation.

SMF notifies any entity that has subscribed to User Location Information related with PDU session change.

* * * * Start of 4th Change * * * *

4.3.4.2	UE or network requested PDU session release for Non-Roaming and Roaming with Local Breakout

Figure 4.3.4.2-1 captures both the UE requested PDU session release procedure and the network requested PDU session release procedure. The procedure allows the UE to request the release of one PDU session. The procedure also allows the SMF or PCF to initiate the release of a PDU session. In the case of LBO, the procedure is as in the case of non-roaming with the difference that the SMF, the UPF and the PCF are located in the visited network.

Editor's note:	Further studies are needed to determine the nature of the information allowing the AMF to know which UE is the target of the SMF request as well to determine which access towards the UE to use

Editor's note:	Alignment of message naming with the procedures for PDU session establishment is needed.

Editor's note:	The names of N11 Message are FFS.

Figure 4.3.4.2-1: UE or network requested PDU session release for non-roaming and roaming with local breakout

1.	The procedure is triggered by one of the following options:

1a.	The UE initiates the UE requested PDU session release procedure by the transmission of an NAS message (N1 SM container (PDU Session Release Request), PDU Session ID) message. The NAS message is forwarded by the (R)AN to the AMF with an indication of User Location Information. This message is relayed to the SMF corresponding to the PDU Session ID via N2 and the AMF. The AMF invokes the Nsmf_PDUSession_UpdateSMContext service operation and provides the N1 SM container to the SMF.

NOTE 1:	Depending on the access type, when the UE is in CM-IDLE state, the UE can trigger a Service Request procedure before being able to release the PDU session.

1b.	The PCF may invoke the Npcf_SMPolicyControl_UpdateNotify service operation to request the release of the PDU session.

1c.	The SMF may decide to release a PDU session e.g. based on a request from the DN (cancelling the UE authorization to access to the DN) or based on a request from the UDM (subscription change) or from the OCS. The release procedure also may be triggered based on locally configured policy (e.g. the release procedure may be related with the UPF relocation for SSC mode 2 / mode 3).

If the SMF receives one of the triggers in step 1a ~ 1c, the SMF starts PDU session release procedure.

2.	The SMF releases the IP address / Prefix(es) that were allocated to the PDU session and releases the corresponding User Plane resources:

2a.	The SMF sends an N4 Session Release Request (N4 Session ID) message to the UPF. The UPF shall drop any remaining packets of the PDU session and release all tunnel resource and contexts associated with the N4 Session.

2b.	The UPF acknowledges the N4 Session Release Request by the transmission of an N4 Session Release Response (N4 Session ID) message to the SMF.

NOTE 2:	If there are multiple UPFs associated with the PDU session, the Session Release Request procedure (steps 2a and 2b) is done for each UPF.

3a.The SMF responds to the AMF with the Nsmf_PDUSession_UpdateSMContext response (N2 SM Resource Release request, N1 SM container (PDU Session Release Command))

The SMF creates an N1 SM container including PDU Session Release Command message (PDU Session ID, Cause). The Cause may indicate a trigger to establish a new PDU session with the same characteristics (e.g. when procedures related with SSC mode 2 are invoked).

NOTE 3:	SSC mode 2 is defined in TS 23.501 [2] clause 5.6.9.

3b.	If the PDU session release is initiated by the SMF, the SMF invokes the Namf_Communication_N1N2MessageTransfer service operation (N1 SM container (PDU Session Release Command), skip indicator).

If the UP connection of the PDU session is active, the SMF shall also include the N2 Resource Release request (PDU Session ID) in the Namf_Communication_N1N2MessageTransfer, to release the (R)AN resources associated with the PDU session.

The "skip indicator" tells the AMF whether it may skip sending the N1 SM container to the UE (e.g. when the UE is in CM-IDLE mode).

NOTE 4:	If the procedure is triggered to relocate PDU session anchor of a PDU session with SSC mode 2, the SMF is assumed not to include the "N1 SM delivery can be skipped" Indication.

Editor's note:	It is FFS whether a Release Indication to explicitly delete the association of PDU Session ID and serving SMF ID within the AMF is added to the SMF request. This would allow skipping step 9 when the UE is in CM-IDLE state and "N1 SM delivery can be skipped" is included in the N11 message.

Editor's note:	It is FFS whether "N1 SM delivery can be skipped" Indication is required.

4.	If the UE is in CM-IDLE state and " skip indicator " is included in the Namf_Communication_N1N2MessageTransfer service operation, the AMF acknowledges the step 4 by sending an N11 Response message ("skip indicator " ) to SMF and steps 5 to 7 are skipped.

NOTE 5:	The UE and the 5GC will get synchronized about the status (released) of the PDU session at the next Service Request or Registration procedure.

Otherwise, if the UE is in CM-IDLE state and "N1 SM delivery can be skipped" is not indicated, the AMF initiates the network triggered Service Request procedure to transmit the NAS message (PDU session ID, N1 SM container) to the UE.

If the UE is in CM-CONNECTED state, then the AMF transfers the SM information received from the SMF in step 4 (N2 SM Resource Release request, N1 SM container) to the (R)AN.

5.	When the (R)AN has received an N2 SM request to release the AN resources associated with the PDU session it issues AN specific signalling exchange(s) with the UE to release the corresponding AN resources.

In case of a 3GPP RAN, an RRC Connection Reconfiguration may take place with the UE releasing the RAN resources related to the PDU session.

During the this procedure, the (R)AN sends any NAS message (N1 SM container (PDU Session Release Command)) received from the AMF in step 5.

The UE acknowledges the PDU Session Release Command by sending a NAS message (PDU session ID, N1 SM container (PDU Session Release Ack)) message over the (R)AN.

6.	[Conditional] If the (R)AN had received a N2 SM request to release the AN resources, the (R)AN acknowledges the N2 SM Resource Release Request by sending an N2 SM Resource Release Ack (NAS message (N1 SM container (PDU Session Release Ack)), User Location Information) Message to the AMF.

Otherwise, the (R)AN just forwards the NAS message (PDU session ID, N1 SM container (PDU Session Release Ack)) from the UE to the AMF.

7a.	The AMF invokes the Nsmf_PDUSession_UpdateSMContext (N1 SM container (PDU Session Release Ack), N2 SM Resource Release Ack) to the SMF.

8.	The SMF responds to the AMF that the PDU session is released with an Nsmf_PDUSession_UpdateSMContext response. The AMF and SMF shall remove all contexts (including the PDU session ID) associated with the PDU session as well as any event subscriptions by the SMF on the AMF.

Editor's note:	It is FFS whether this takes the form of a Nsmf_ PDU Session Event Report Notification (AMF): The SMF sends (N11 Message) PDU Session Event Report Notification as defined in Clause 5.2.7.1 to the AMF; the event trigger is PDU session release.

9.	If Dynamic PCC applied to this session the SMF invokes the Npcf_PolicyControl_PolicyDelete service operation to delete the PDU CAN session.

SMF notifies any entity that has subscribed to User Location Information related with PDU session change

If it is the last PDU session it is handling for the UE the SMF releases the association with the UDM by means of the Nudm_UEContextManagement_Deregistration service operation.

NOTE 6:	The order with which SMF releases the resources is implementation dependant.

* * * * Start of 5th Change * * * *

5.2.8.2.5	Nsmf_PDUSession_Create SM Context service operation

Service operation name: Nsmf_PDUSession_Create SM Context

Description: It allows create SM Context on a certain PDU session.

Known NF Consumers: AMF.

Input, Required: SUPI, DNN, S-NSSAI, PDU Session ID, AMF ID, N1 SM container.

Input, Optional: PEI, UE location information, AN type, H-SMF identifier/address.

Output, Required: Result Indication

Output, Optional: Cause, , PDU Session ID, N2 SM information, N1 SM container.

NOTE:	The information required in Input and Output may be updated when the new system procedures related to session management is identified in clause 4.

See step 3, 10 of clause 4.3.2.2.1 and step 3, 14 clause 4.3.2.2.2 for details on the usage of this service operation.

* * * * Start of 6th Change * * * *

5.2.8.2.6	Nsmf_PDUSession_Update SM Context service operation

Service operation name: Nsmf_PDUSession_Update SM Context

Description: It allows to update the SM Context of a certain PDU session due to new/updated N1/N2 SM information.

Known NF Consumers: AMF.

Input, Required: SUPI, PDU Session ID.

Input, Optional: N1 SM container, N2 SM information, Operation type (e.g. activate, deactivate), UE location information, AN type.

Output, Required: Result Indication,

Output, Optional: Cause, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE.

See step 1a, 1e, 3a, 7 of clause 4.3.3.2 and step 1a, 1e, 4a, 8, 8a of clause 4.3.3.3 for an example usage of this service operation.

* * * * End of Changes * * * *