SA WG2 Meeting #122E e-meeting	S2-176731

11 – 15 September 2017, Elbonia	(was S2-17----)

Source:	CATT

Title:	Revisions to the terminology regarding Registration Area

Document for:	Approval

Agenda Item:	12

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution proposes to revise the text of 3GPP TS 23.501 to align with the definition of Registration Area and TAI List.

- Discussion

There are the description to Registration Area and TAI List in Clause 5.3.2.3, 3GPP TS 23.501 V1.3.0 as follows:

When a UE registers with the network over the 3GPP access, the AMF allocates a set of tracking areas in TAI List to the UE. When the AMF allocates registration area, i.e. the set of tracking areas in TAI List, to the UE it may take into account various information (e.g. Mobility Pattern and Allowed/Non-allowed area (refer to 5.3.4.1)).

To align with the description of Registration Area and TAI List, following changes are proposed to 3GPP TS 23.501.

- Proposal

Add the following to TS 23.501.

*************** First change*********************

5.8.3	UP tunnel management for UE in CM_IDLE state

5.3.4.2	Mobility Pattern

The Mobility Pattern is a concept that may be used by the 5G system core network to characterise and optimise the UE mobility. The 5G system core network determines and updates Mobility Pattern of the UE based on subscription of the UE, statistics of the UE mobility, network local policy, and the UE assisted information, or any combination of them. The statistics of the UE mobility can be historical or expected UE moving trajectory.

UE mobility pattern can be used by the AMF to optimize mobility support provided to the UE, for example, registration area allocation.

*************** Second change*********************

5.16.3.2	IMS voice over PS Session Supported Indication

The serving PLMN AMF shall send an indication toward the UE during the Registration procedure if an IMS voice over PS session is supported or not supported. A UE with "IMS voice over PS" voice capability should take this indication into account when performing voice domain selection, as described in clause 5.16.3.5.

The serving PLMN AMF may only indicate IMS voice over PS session supported in one of the following cases:

-	If the network is able to provide a successful IMS voice over PS session over NR connected to 5GC with a 5G QoS Flow that supports voice as specified in clause 5.7.

-	If the network is not able to provide a successful IMS voice over PS session over NR connected to 5GC, but is able for one of the following:

-	If E-UTRA connected to 5GC supports voice, and the NG RAN is able to trigger handover to E-UTRA connected to 5GC at QoS flow establishment for voice; or

-	If the UE supports HO to EPS, the EPS supports voice, and the NG RAN is able to trigger handover to EPS at QoS flow establishment for voice.

The serving PLMN provides this indication based e.g. on local policy, HPLMN and how extended NG-RAN coverage is. The AMF in serving PLMN shall indicate that IMS voice over PS is not supported if serving PLMN does not have an IMS roaming agreement with HPLMN. This indication is per Registration Area.

*************** End of changes *********************