SA WG2 Meeting #124	S2-179404

27 November - 1 December 2017, Reno, USA	(revision of S2-179347)

Source:	LG Electronics

Title:	TS 23.501: Clarification on LADN location reporting

Document for:	Approval

Agenda Item:	6.5.3 Session management and continuity functions and flows

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This contribution is proposed to clarify LADN location reporting.

Discussion

Based on the discussion during the session, this contribution clarifies that location event subscription is transferred between AMFs.

Proposal

* * * * Start of 1st Change * * * *

5.6.11	UE Location change notification

When a PDU Session is established or modified, or when the user plane path has been changed (e.g. UPF re-allocation), SMF may determine an area of interest, e.g. based on UPF service area, etc., and subscribe to AMF notifications. When the AMF detects that the UE has moved in or out of that area, it need notify SMF of the UE's new location.

For 3GPP access the area of interest is defined by a list of Tracking Areas.

SMF subscribes to "UE mobility event Notification" service provided by AMF. During subscription, SMF provides the area of interest to AMF. The AMF sends UE's new location to SMF when AMF detects that the UE has moved in or out of that area. Upon reception of new UE location notification from AMF, the SMF determines how to deal with the PDU Session, e.g. reallocate UPF.

If the AMF is subscribed to an area of interest by the SMF, and this area of interest is smaller than the Registration Area sent to the UE, the AMF may provide the area of interest information to NG-RAN via the location reporting procedure specified in TS 23.502 [3]. If the NG-RAN detects the UE moves out of or into the area of interest provided by the AMF, NG-RAN notifies the latest UE location to AMF.

In the case of LADN, the SMF only provides the LADN DNN to the AMF to subscribe to notifications when the UE enters or leaves the LADN service area. The AMF notifies the SMF when the AMF detects that the UE has moved in or out of an area where the LADN is available. Upon reception of a notification from the AMF that the UE has moved in or out of an LADN area, the SMF determines how to deal with the PDU Session, e.g. release the PDU Session, deactivate user plane connection for the PDU Session, etc. The AMF may send the UE location to the SMF along with the notification, e.g. for UPF relocation.

The subscription may be maintained during the life of PDU Session, regardless of the UP activation state of PDU Session (e.g. PDU Session UP connection activated or not). When the AMF is changed, the subscription of mobility event is transferred from the old AMF to the new AMF and the new AMF notifies the SMF with the current status related to the subscription of mobility event.

SMF may determine a new area of interest, and send a new subscription to the AMF with the new area of interest.

SMF unsubscribes to "UE mobility event Notification" service when PDU Session is released.

The UE location change notification may also be subscribed by other NF.

* * * End of Changes * * * *