# SA WG2 Meeting #S2-129BIS	S2- 1812503

# 26 - 30 November, 2018, West Palm Beach, USA

Source:	Charter Communications, CableLabs

Title:	Other conclusions for accessing the network via W-5GCAN.

Document for:	Approval

Agenda Item:	6. 7

Work Item / Release:	FS_5WWC

Abstract of the contribution: The proposal concludes the other parts of study for accessing the network via W-5GCAN.

# Proposal

It is proposed to modify the following clause in TR 23.716.

**** Start of Change ****

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions, interim or/and final conclusions, which have been agreed during the course of the work item activities.

…

## 8.2	Conclusions on WWC

### 8.2.1	Conclusions on FWA

The following list summarizes the interim conclusions regarding the support of FWA :

-	The reference architecture is defined in scenario #1 figure 4.2.2.1-3

-	The solution applies only for 5G-RG connected only via NG RAN.

-	solution #19 as described in clause 6.19  is endorsed as a basis for normative specifications.

-	The applicability of roaming scenario and whether and how wholesale model are applicable to FWA is under the scope of BBF. The capability to support or to restrict 5G-RG to connect only to HPLMN is currently support by 3GPP specification and it does not require any modification.

-	No specific cable related aspect identified. Solution can be reused.

### 8.2.2	Conclusions for the scenario that 5G-RG / 5G-CRG connected via W-5GAN / W-5GCAN

Regarding the architecture that 5G-RG connected via W-5GAN shown in Figure 4.2.2.1-1, interim conclusions are listed as below:

NOTE: Unless otherwise stated, the following conclusion applies to 5G-RG/W-5GCAN as well.

NAS transmission after registration:

-	A signalling connection based on protocol FCP between 5G-RG and AGF should be established to transmit the NAS message;

-	Details about FCPthe signalling connection should be specified in BBF.

NOTE: the definition of FCP signalling connection is within BBF scope that from SA2 perspective is capable to transport NAS messages between 5G-RG and AGF.

-	Between 5G-CRG and CFAGF, “Cable Fixed Access Control Protocol” (CFCP) for transporting NAS message will be defined by CableLabs.

Registration procedure:

-	The Registration procedure is based on following principles:

-	Authentication shall be based on NAS exchange encapsulating authentication messages as defined in TS 33.501 where the 5G-RG acts as the UE and AGF act as the (R)AN.

Editor's Note: Whether the transport of NAS message for Registration procedure corresponds to EAP-5G per solution 2 or FCP per solution 3some other signalling connection requires confirmation from BBF.

De-Registration procedure:

-	The De-Registration procedure is based on the De-Registration procedure in solution 3 clause 6.3.6.

Session Management:

-	The PDU Session Establishment Procedure via Wireline Access Network is based on the 5G-RG Requested PDU Session Establishment procedure in solution 3 clause 6.3.3.

-	The PDU Session Modification Procedure via Wireline Access Network is based on the UE or Network Requested PDU Session Modification procedure in solution 3 clause 6.3.4.

-	The PDU Session Release Procedure via Wireline Access Network is based the UE Requested PDU Session Release procedure in solution 3 clause 6.3.5.

Connection Management and Registration Management:

-	The Connection Management is based on solution 5 in clause 6.5.

-	The Registration Management is based on solution 6 in clause 6.6.

Mobility Management:

-	For the scenario of 5G-RG connected only via W-5GAN (as shown in Figure 4.2.2.1-1 in Scenario 1) per solution 4 clause 6.4.2 the handover procedures defined in TS 23.502 [3] clause 4.9 are not applicable, since the 5G-RG is stationary and connected only via a wired network.

** Next Change **

### 8.2.4	Other Conclusions

For cable:

-	Supporting FN-CRG in 5GC based on solution #25 (section 6.25) is endorsed.

-	Collocation of UPF and W-5GCAN User Plane Function based on solution #21 (section 6.21) shall be possible, i.e, standard shall allow this possible deployment option.

NOTE: During normative specification phase, solution to be specified for both BBF and cable based network shall be aligned as much as possible.

Solution #9 "New IPV6 address allocation mechanisms" as described in clause 6.9 is endorsed as a basis for normative specifications.

Editor's note:	This solution would apply to Combo Ethernet PDU Session type only if that PDU Session type is endorsed as part of the study.

**** End of Changes ****