3GPP TSG-SA WG2 Meeting #129BIS 	S2-1812533

26 - 30 November 2018, West Palm Beach, USA

Source:	Huawei, HiSilicon

Title:	Update ATSSS path performance measurement function of solution 3

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS /Rel-16

Abstract of the contribution: This contribution removes some Editor’s Notes and updates ATSSS path performance measurement function of solution 3.

1	Introduction

Change 1: Remove the following Editor’s notes: How the jitter, packet loss ratio and load for each link can be evaluated are FFS.

RTT measurement: UE could calculates the RTT based on the local time when sending the TFCP echo request message and receiving the TFCP echo response message.

Jitter measurement: UL jitter between certain TFCP echo request message and Nth TFCP echo after such certain TFCP echo request message could be measured by the UPF based on the local time when receiving these continuous TFCP echo message.

Packet loss ratio measurement: The UE and UPF the counts of sending and receiving UL packets number between certain TFCP echo request message and the previous TFCP echo request message of this certain TFCP echo request message, and the UE also provides the counting result to the UPF. The UPF calculates the UL packet loss ratio based on the local counting result and the number of UL packets send by UE. The DL packets loss ratio could be measured by the UE using the similar method as UL packet loss ration calculation.

Change 2: Remove the following Editor’s notes: Whether the UPF needs to initiate path performance measurement is FFS.

Considering the scheduling and transmission delay of Uu interface is asymmetric for UL traffic and DL traffic, even for one QoS flow, the UPF may also need to initiate the path performance measurement procedure for calculation of DL jitter, using the same mechanism as UL jitter calculation. So this Editor’s notes is deleted.

2	Proposal

The following changes are proposed to TR 23.793.

********************* start of changes ****************

6.3.1.4	ATSSS path performance measurement function description

The performance of the available path is measured and reported to the Traffic Distribution function by the Path Performance Measurement function. The path performance measurement function is deployed in the UE and the UPF, which means both the UE and the UPF could initiate the path performance measurement, and the measurement result is used for traffic distribution determination.

The performance measurement parameters includes access agnostic parameters and access specific parameters (optional). The access agnostic parameters includes the RTT, jitter, and packet loss ratio parameters, which could be used to justify the path performance of 3GPP access and Non 3GPP access respectively. The access specific parameters includes the parameters which could be used to verify the load or signal strength of each access, e.g. the RSRP/RSRQ for 3GPP access, and the available bandwidth for Non 3GPP access (e.g. WLAN AP). Besides, the performance measurement policies, e.g. access type for measurement, measurement period, report threshold and/or report period, are also included in the performance measurement parameters.

The measurement granularity could be per PDU session or per QoS flow for each access. During the PDU session establishment/modification procedure, the SMF provides path performance measurement parameters for the PDU session or QoS flows, traffic of which is potentially to be distributed, to the UE in the PDU session establishment/ modification Accept via NAS message. Besides, the SMF also provides the PDU session ID or QFI along with path performance measurement parameters to indicate to the UE to bind the path performance measurement parameters with the corresponding PDU session or the QoS flow to be measured.

After receiving the performance measurement parameters for the PDU session or the QoS flow to be measured, the UE initiates path performance measurement for the PDU session or QoS flow based on the path performance measurement policy:

Figure 6.3.1.4-1: Path performance measurement on UE side

-	Access agnostic performance measurement: The UE generates the TFCP echo request message based on the measurement period, and sends the TFCP echo request message via the 3GPP access and/or Non 3GPP access periodically. When the UPF receives the TFCP echo request message from the 3GPP and/or Non 3GPP access node respectively, the UPF generates the TFCP echo response message and sends back via the corresponding access node. The RTT and jitter could be measured via the TFCP echo request/response messages. Besides, the UE and the UPF could exchange the sending and receiving packet data statistics to get the packet loss ratio result. The UPF also could initiate path performance measurement similar as UE.

NOTE 1:	The measurement traffic per QoS flow can be implemented in a way without impact on the link performance for the QoS flow, e.g. the interval of sending TFCP echo request/response message pair for path performance measurement can be set to a couple of minutes. Since the link performance would not deteriorates gradually, interval set to minutes(s) for path performance detection should be enough for ATSSS.

Based on the access agnostic performance measurement, the RTT, jitter and packet loss ratio could be measured as following:

RTT measurement:

- When the UE sends out the TFCP echo request message, the UE records the local time T1.

- When the UE receives the TFCP echo response message, the UE records the local time T2.

The UE could calculate the RTT based on these time stamp: RTT(n) = T2-T1.

NOTE 2: Since the internal process delay of UPF for the TFCP echo request/response message is tiny, it is ignored and not to be eliminated from the RTT.

Jitter measurement:

- Jitter is regarded as the reflection of transfer quality stability within certain time interval. The UL jitter could be   calculated by the UPF, and the DL jitter could be calculated by the UE. The following solution is an example method for calculating the UL jitter, and the DL jitter could be calculated using the similar method.

- It is assumed to calculate the jitter between a certain TFCP echo request message and the Nth TFCP echo request message after such certain TFCP echo request message by the UPF. These continuous TFCP echo request message could be numbered from n-N+1 to n. the average expectation value of arriving time interval between any two TFCP echo requests could be represented as Avgn. The value of variance of arriving time interval of N TFCP echo request could be represented asVarn. In this example, the Varn is regarded as the packet transport jitter for the corresponding QoS Flow or PDU Session. It is assumed that tk is the local time of UPF when receiving the TFCP echo request message numbered by variable k between n-N+1 and n.

Avgn= n-N+1ntk-tk-1/N, Varn= n-N+1ntk-tk-1-Avgn2N

Packet loss ratio measurement:

UE and UPF exchange the packet counting information in certain period to calculate the packet loss ratio during the path performance measurement procedure.

- The UE counts the number of UL packets between certain TFCP echo request message and the previous TFCP echo request message of this certain TFCP echo request message, and provides the result to the UPF via this certain TFCP echo request message.

- UPF also counts the number of received UL packets between certain TFCP echo request message and the previous TFCP echo request message of this certain TFCP echo request message. UPF calculates the UL packet loss ratio based on the local counting result and the number of UL packets send by UE.

- UPF sends the UL packet loss ratio result and the counting number of DL packets between certain TFCP echo response message and the previous TFCP echo response message of this certain TFCP echo response message to the UE via this certain TFCP echo response message.

- UE also counts the number of received DL between certain TFCP echo response message and the previous TFCP echo response message of this certain TFCP echo response message. UE calculates the DL packet loss ratio based on the local counting result and the number of DL packets send by UPF.

-	Access specific performance measurement (optional): The UE can measure the radio signal strength for the 3GPP access and the available bandwidth of Non 3GPP access.

Based on the report threshold or period of performance measurement parameters, the UE could provide the measurement result to the UPF as described in the step 3 of Figure 6.3.1.4-1, via user plane directly or via control plane to SMF and then from SMF to the UPF.

Considering the scheduling and transmission delay of Uu interface is asymmetric for UL traffic and DL traffic even for one QoS flow, the UPF may also need to initiate the path performance measurement procedure for calculation of DL jitter, using the same mechanism as for UL jitter calculation, i.e. the UPF sends the TFCP echo request message to the UE periodically, and the UE sends the TFCP echo response message to the UPF. The UE could calculate the DL jitter of the QoS flow using the same method as UPF calculating UL jitter. The UE could provide the DL jitter result to the UPF via TFCP echo response message.

When the “Smallest Delay” steering mode is selected for a service data flow, the UE and UPF could select the available access for the UL and DL SDF based on above several combined measured parameters. The following provides some examples to illustrate how to use combined measured parameters to select the available access by the UE and UPF:

- RTT and Packet loss ratio: The AN node may drop a number of packets to reduce the load condition in order to achieve the better RTT. In such case, even the RTT is better, the packet loss ratio may be terrible, so the UE and UPF could take both RTT and packet loss ratio as the factors when selecting the available access for the SDF.

- RTT and jitter: The measured RTT may change frequently when the quality of radio interface is not stability. The measured RTT may be small in certain measurement, while may be large in the next measurement. If the UE and UPF selects the available access only based on RTT, the SDF may be switched frequently between two access paths. This may bring bad impaction to the application. Since jitter could reflect the transfer quality stability of the access within certain time interval, the UE and UPF could take both RTT and packet loss ratio as the factors when selecting the available access for the SDF.

NOTE 3: If all detected QoS parameters of each access fulfil the service requirement, the access with smallest RTT is suggested to be selected. If one or more of the detected QoS parameters of each access cannot fulfil the service requirement, the UE and the UPF could select the available access based on local policy, e.g. the potential vMOS of the service could be calculated by the UE and the UPF using the detected QoS parameters as input, and the access with better vMOS can be selected.

****************** End of changes**********************