# SA WG2 Meeting #S2-129bis	S2-1813213

26 – 30 October, 2018, West Palm Beach, Florida, USA      (revision of S2-1812836/2757/2127)

Source: China Mobile, Alibaba, CATR, China Unicom, Huawei, Tencent, KDDI, China Telecom, NEC

Title:    Solution for Key Issue 14: How to ensure that slice SLA is guaranteed

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA/Rel-16

Abstract of the contribution: This contribution proposes a solution about how to guarantee slice SLA from control plane.

Proposal

This paper propose a solution about Slice SLA guarantee from control plane.

* * * Start of Change (new text) * * * *

6.X	Solution X: Ensure slice SLA is guaranteed within certain region area

6.X.1	Description

This solution is for Key Issue 14: How to ensure that slice SLA is guaranteed.

What slice customer care is, how many subscribers (e.g. 1 millions) can be served by the slice and the Service Experience statistics per application (e.g. average Service MOS 4.0 for Application X for the slice customer) and how many percent (e.g. 90% for Application X) UEs’ service experience satisfy per application ID.

A new slice may be created with dedicated 5GC resource e.g., dedicated SMF/UPF/PCF. Usually 5GC resource is enough while NG RAN is the bottleneck, therefore NG-RAN could be initially configured (e.g., by OAM) with reserved/prioritised resources for the new slice, for example give the slice customer 's devices guaranteed access up to X% (e.g. 20%) of the resources of NG-RAN, in particular area. Other existing slices can access to the reserved/prioritised X% resources of NG-RAN when the new slice is not using them.

However, the initially configured X% of the resources of NG-RAN could overfit or underfit the slice customer requirement and may need to be modified based on the slice SLA fulfilment Info from 5GC.

6.X.1.1	Information for slice SLA guarantee

The information for slice SLA guarantee is similar as the information for the support of QoS Profile Provisioning, see clause 6.3.1.1 for details.

6.X.1.2	Procedure to support slice SLA guarantee with certain region area

NSSF should control subscribers to access the new slice gradually in order to avoid/minimize the negative impact on the other existing slice SLA fulfilment (with signed SLA).

Figure 6.X.1.2-1: NWDAF-Assisted Slice SLA guarantee with certain region area

0a. OAM creates a new slice and configures initial resources in RAN (e.g. 20% for the new slice) and in CN allocated to the slice.

0b: OAM notify the initial number of users and Slice SLA requirement e.g. the average Service MOS requirement and/or the requested number of subscribers and/or the requirement of percentage of the user satisfied to NSSF.

1. For the newly created slice, NSSF only allow partial number of subscribers to access the slice.

Note: NSSF initially allows partial (e.g. 10%) users number to access the slices assuming that users number do not impact the other existing slice SLA, allow NWDAF to train Service MOS Model and then calculate QoE information according to Solution 3: QoS profile Provisioning.

2. The NSSF sends an Analytics request/subscribe (Event ID = Slice statistics Info, Event Filter information = (S-NSSAIs, Tracking Area, Time Window)) to NWDAF by invoking a Nnwdaf_EventSubscription_Subscribe.

3. NWDAF monitor QoE per user or group of users and provides the QoE data analytics (e.g. the estimated average Service MOS and/or the number of registered subscribers and/or percentage of the user satisfied) to the NSSF and/or OAM.

4. Based on QoE data analytics provided by NWDAF, NSSF determines whether slice SLA fulfilment (including the new and existing slices with SLA requirement) is overfitting or underfitting or fitting.

5. NSSF determines the allowed number of the user for the slice and notifies slice SLA fulfilment to RAN via AMF or OAM. RAN could take into account slice SLA fulfilment to schedule Resource per slice within the RAN resource configured by OAM.

Note: NSSF informs RAN is dependent on RAN3 and SA5 feedback.

6. OAM trains slice resource model with the KPIs from OAM and QoE from NWDAF and then adjust slice resource until slice is stable e.g. RAN resource from 20% (initially configured by OAM when creating slice) to 15%.

7. When slice is stable i.e. the RAN and CN resource configure is fixed, OAM notifies NSSF to update the allowed number of the user for the slice.

6.X.2	Impacts on Existing Nodes and Functionality

NWDAF: Provides QoE data analytics to the NSSF.

NSSF:

- Get the QoE data analytics from NWDAF;

- Determines the per slice SLA fulfilment information (per TA) based on the QoE from NWDAF;

- Controls partial number of subscribers to access the slice to avoid negative impact on the existing slice SLA;

- Provides the SLA fulfilment information to RAN via AMF or via OAM.

OAM:

- Notifies NWDAF to do QoE for a slice.

- May obtain information on SLA fulfilment from NSSF and determines the resource usage for RAN and notifies NSSF to update the allowed number of the user for the slice.

AMF:

- Transfers the per slice SLA fulfilment information per TA from NSSF to RAN.

RAN:

- Receives slice SLA fulfilment information from NSSF via AMF or via OAM, which may be taken into account when scheduling radio resource;

6.X.3	Solution Evaluation

This solution allows NSSF to determine per slice SLA fulfilment information based on QoE from NWDAF.

With per slice SLA fulfilment information, NSSF could control subscribers to access a new slice gradually in order to avoid/minimize the negative impact on the other existing slice SLA fulfilment..

Optionally, this solution also allows RAN to take into account the per slice SLA fulfilment information to schedule radio resource.

The solution applies effectively in some region areas (may also in certain time period), in which the slice SLA is concerned.

* * * End of Change * * * *

Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x	Key Issue 14: How to ensure that slice SLA is guaranteed

Solution X supports the control plane/OAM contribute to slice SLA with assistance information i.e. QoE information from NWDAF. This is targeting slice SLA being guaranteed within certain region area

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

8.x	Key Issue 14: How to ensure that slice SLA is guaranteed

Solution X is selected as the basis for the normative work.

NOTE: The notification part in the solution from control plan (NSSF) to RAN will be further checked with RAN in the normative phase.

* * * End of Changes * * * *