3GPP TSG-SA WG2 Meeting #129 	S2-1810185

# 15-19 October 2018, Dongguan, China												was	S2-18xxxx

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0740 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover from EPC/ePDG to 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following logic is implemented |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of the registration procedure that the UE must perform for ePDG to 5GS mobility based on the UE's mode of operation, single registration or dual-registration mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect UE implementation of ePDG to 5GS mobility. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change* * *

#### 4.11.4.1	Handover from EPC/ePDG to 5GS

Figure 4.11.4.1-1: Handover from EPC/ePDG to 5GS

0.	Initial status: one or more PDU Sessions have been established between the UE and the EPC/ePDG via untrusted non-3GPP access. The UE has indicated its 5G NAS capability in order for the ePDG to select a combined PGW+SMF, and has provided the PDU Session ID to the combined PGW/SMF.

1.	 For the UE to move PDU session(s) from EPC/ePDG to 5GS, the UE behaviour is as follows:

-	If the UE is operating in "single-regisration" mode (please see clause 5.17.2.1 in TS 23.501 [2]) and

-	if the UE's EMM state is EMM-REGISTERED in EPC, the UE performs Registration procedure of type mobility regisration in 5GS as specified in clause 4.11.1. The UE and network behaviour for PDU sessions supported in EPC and NG-RAN/5GC is as specified in clauses 4.11.0 through 4.11.3.

-	otherwise (i.e. UE's EMM state is EMM-DEREGISTERED in EPC) and

-	if the UE is in 3GPP RM-REGISTERED state in 5GS, the UE performs Step 2.

-	otherwise (i.e UE is in 3GPP RM-REGISTERED state in 5GS), the performs Registration procedure of type initial regisration in 5GS.

-	otherwise (i.e. UE is operating in "dual-registration" mode), and

-	if the UE is in 3GPP RM-REGISTERED state in 5GS, the UE performs Step 2.

-	otherwise (i.e UE is in 3GPP RM-DEREGISTERED state in 5GS), the performs Registration procedure of type initial regisration in 5GS.

2.	The UE initiates a UE requested PDU Session Establishment via 3GPP Access acording to clause 4.3.2.2 and includes the "Existing PDU Session" indication and the PDU Session ID.

3.	The combined PGW+SMF/UPF initiates a PDN GW initiated Resource Allocation Deactivation with GTP on S2b as described in TS 23.402 [26] clause 7.9.2 to release the EPC and ePDG resources.

* * * End of Changes* * *