3GPP TSG-SA WG2 Meeting #129	S2-1810533

October 15 - 19, 2018, Dongguan, P. R. China

Source:	CATT

Title:	Solution for optimizing connection management based on NWDAF output

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA / Rel-16

Abstract of the contribution: This paper would like to propose a solution on optimizing connection management based on NWDAF output.

1.	Discussion

This document proposes to a new solution to address the Key Issue 3: Customizing mobility management based on NWDAF output in TR 23.791.

* * * Start of Changes * * *

6.x	Solution for Key Issue #9: <Optimizing connection management based on NWDAF output>

6.x.1	Description

Editor's note:	Describe the solutions. Sub-clause(s) may be added to capture details, procedural flow, etc.

6.x.1.1	General

This solution would like to utilize NWDAF to predict the time window in which the UE has or doesn’t have UL/DL data transmission, so that the AMF can optimize connection management. Such prediction can be specified as the UE communication pattern. In order to obtain UE communication pattern, it is necessary to let the NWDAF know the communications that UE has.

In the Rel 15, communication pattern is defined as per UE level and the 5GC has been able to understand communication pattern (called “Expected UE Behaviour parameters” in TS 23.501) which is provisioned by external party via the NEF. However, in the case of the UE simultaneously communicating with multiple application servers, it may be not easy for application layer to know that per UE communication pattern.

In this solution, the NWDAF is expected to discover UE communication pattern to optimize connection management. The NWDAF collects the UE communication history from the OAM system and/or 3rd party Application Servers, such as the time that the UE started each communication (identified by the application server IP and UE IP), service type, and the duration of the communication etc..

By applying data mining, the NWDAF may be able to model the UE communication pattern, and then provide the analytical result to the AMF (maybe via PCF). The analytical result for a specific UE or a group of UE may be traditional communication pattern defined in TS 23.682 with following additions:

-	The time and day that the UE might start a low latency communication, e.g. Time: 8:00-9:00, Day: Monday-Friday; and/or

-	The time and day that the UE might not have any communication, e.g. Time: 23:00-7:00, Day: Sunday-Friday;

The 5GC can utilize the UE mobility information and UE communication pattern to optimize connection management for a UE:

-	If the UE communication pattern indicates that the UE has a low latency communication for a time period, the AMF may keep the UE in CM-CONNECTED mode to reduce the latency of UE obtaining the service;

-	If the UE communication pattern indicates that the UE has no communication in a time period, the AMF may advise the UE to adopt power saving method, e.g. PSM or MICO.

-	If the UE communication pattern indicates that the UE has frequent communications in a time period, and the UE mobility information shows that the UE stay in a same place in that time period, the AMF may keep the UE in CM-CONNECTED mode to reduce the signalling on CM state transitions but without bring frequent handover.

6.x.1.2	Procedures

The provision of UE communication pattern and UE mobility prediction from the NWDAF to the 5GC is shown in the Figure below.

Figure 6.x.1.2-1 procedure of using NWDAF out to optimize connection management

1.	A UE registers to the AMF.

2.	The AMF contact PCF to retrieve Access and Mobility policies which may contains UE state transition policy derived from analytic information provided by the NWDAF. The analytic information includes UE mobility prediction and UE communication pattern.

3a.	The PCF may subscribe UE communication pattern from the NWDAF if it needs to provide UE state transition policy.

3b. The AMF may subscribe analytic result on UE communication if the PCF needn’t provide UE state transition policy or UE communication pattern.

4.	The AMF responds the UE. If the AMF has received UE communication pattern, and the UE communication pattern indicates that the UE has no communication in a time period, the AMF suggests the UE adopting power saving and includes the time period that the power saving can be activated in the Registration Accept message.

NOTE:	Whether to adopt power saving and which power saving method is adopted are up to UE decision.

5a-7.	The NWDAF provide the UE communication pattern and UE mobility prediction to the PCF, so that the PCF can determine the UE state transition policy, e.g.:

-	If the UE communication pattern indicates that the UE has a low latency communication for a time period, the AMF keeps the UE in CM-CONNECTED mode; or

-	If the UE communication pattern indicates the UE has frequent communications in a time period and the UE mobility information shows that the UE stay in a same place in that time period, the AMF keeps the UE in CM-CONNECTED mode.

The PCF update the AMF with new generated UE state transition policy.

5b.	The NWDAF provide the UE communication pattern and UE mobility prediction to the AMF.

8.	The AMF stores the received policies or analytic results, and then optimizes connection management for the UE as described in the 6.x.1.1.

6.x.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.x.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * Start of Changes * * *