3GPP TSG-SA2 Meeting #129	S2-1811450

15 - 19 October 2018, Dongguan, China	(was S2-1811129, S2-1810613, S2-188050)

Source:	Huawei, HiSilicon, NTT DOCOMO

Title:	KI#15: Update solution 42 and solution 43 for redirection from one CN type to another

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract of the contribution: This paper proposes to update solution 42 and solution 43 to remove EN and also some issue consideration, e.g. how to solve the case without N26 and load condition are added.

1	Introduction

Key Issue 15: Core Network selection for Cellular IoT aims to study how to inform the UE whether and which 5GS CIoT features are available in 5GC, and how to steer the UE to EPC or 5GC based on operator policies or load condition if the CIoT features required by the UE are supported in both EPC and 5GC. The requirements regarding this KI include:

- “It shall be possible for a UE supporting CIoT features to discover whether 5G CIoT feature(s) are supported in 5GC via NB-IoT/WB-E-UTRA access, and to perform core network selection between EPC and 5GC based on such discovery.”

- “It shall be possible for the RAN to take into account CIoT feature indication(s) from the UE to perform AMF selection.”

- “Upon UE selecting 5GC, it shall be possible for the 5GC, based on e.g. operator policy or load conditions, to redirect the UE to EPC, and vice versa.”

- “Upon redirecting the UE to EPC, it shall be possible for the EPC to prevent idle and connected mode mobility to 5GC, and vice versa.”

2	Discussion

Two mechanisms have been proposed on how to redirect the UE to the other CN type. Solution 42: NAS based Redirection between Core Networks and Solution 43: RRC based Redirection between Core networks apply to KI 15. In particular they address the requirements c), d) above.

There is the following editor’s notes in Solution 42/43:

Editor's note: How the MME knows the 5GC is available for the UE is FFS.

Editor’s Note: How to re-enable S1 capabilities if disabled is FFS.

Editor's note: How the AMF knows the EPC is available for the UE is FFS.

Editor’s Note: How to re-enable N1 capabilities if disabled is FFS.

2.1 	UE capability for redirection decision

When the AMF or MME determines to redirect UE between 5GC and EPC, it shall take the 5GS CIoT features or EPS CIoT optimisations supported by the UE into account. For example if a UE doesn’t support EPS CIoT optimisations, the AMF can’t redirect the UE to EPC. This information is not same as whether the UE supports S1/N1 mode. The Preferred Network Behaviour includes the CIoT optimisations the UE supports and what it would prefer to use. It is proposed to clarify that the Preferred Network Behaviour can be used by the AMF/MME to judge whether it can redirect the UE to other type of CN, i.e., whether the 5GC/EPC is available for the UE.

2.2	Re-enablement for N1/S1 mode

TS 24.501 provides how to re-enable N1 mode, in clause 4.9.1:

The UE shall re-enable the N1 mode capability when the UE powers off and powers on again or the USIM is removed.

As an implementation option, the UE may start a timer for re-enabling N1 mode capability, after the N1 mode capability was disabled. On the expiry of this timer, the UE should re-enable the N1 mode capability.

And in clause 4.9.2:

When the UE supporting both N1 mode and S1 mode needs to stay in E-UTRA connected to EPC, in order to prevent unwanted handover or cell reselection from E-UTRA connected to EPC to NG-RAN connected to 5GCN, the UE operating in single-registration mode shall disable the N1 mode capability for 3GPP access and:

a)	shall set the N1mode bit to "N1 mode not supported" in the UE network capability IE (see 3GPP TS 24.301) of the ATTACH REQUEST message and the TRACKING AREA UPDATE REQUEST message in EPC; and

b)	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the N1 mode capability for 3GPP access.

c)	if another RAT of the registered PLMN or a PLMN from the list of equivalent PLMNs cannot be found, or the UE does not have a registered PLMN, then perform PLMN selection as specified in 3GPP TS 23.122 [5]; or

d)	if no other allowed PLMN and RAT combinations are available, then the UE may re-enable the N1 mode capability for 3GPP access and remain camped in NG-RAN of the registered PLMN, and may periodically scan for another PLMN and RAT combination which can provide EPS services or non-EPS services (if the UE supports non-EPS services).

The UE shall re-enable the N1 mode capability for 3GPP access when the UE performs PLMN selection.

The UE should memorize the identity of the PLMN where N1 mode capability for 3GPP access was disabled and should not consider this PLMN in subsequent PLMN selections as specified in 3GPP TS 23.122

For the re-enablement of N1 or S1 mode, it is proposed reuse the same mechanism. A clarification is added that the existing procedure in TS 24.501 covers the re-enablement of N1 mode in this case.

2.3	RRC based redirection

2.4	Load condition consideration

2.5	Other Notes and ENs

1) In clause 6.42.2:

Editor's note:	It is FFS how the solution interoperates with the steering of roaming as defined in TS 23.122 [21], e.g. whether the CN redirection or PLMN redirection takes precedence.

It is proposed to resolve the EN by clarifying that the Steering of roaming is not applied in 5GC if the UE is redirected to EPC. This means that the CN redirection takes precedence over Steering of Roaming.

2) In clause 6.43.2

Editor's note:	RFSP/SPID has not been specified for NB-IoT UEs.

It is proposed to replace this by a NOTE.

3) In clauses 6.43.4.1.2 & 6.43.4.1.3

NOTE 2:	As per current procedures the AS layer indicates the available core networks to the NAS layer. If the target CN is not available for the UE, the UE stays registered in the source core network.

This note has been replaced with more descriptive text of the UE behaviour.

3	Proposal

It is proposed to approve the changes to TR23.724 listed below.

*** Start of changes ***

6.42	Solution 42: NAS based Redirection between Core Networks

6.42.1	Introduction

This NAS based solution applies to Key issue 15 of load balancing the UEs between the two systems.

6.42.2	Functional Description

This solution is a procedure to steer the UE during the (re-)registration procedure.

In EPC, when UE sends the Attach Request to the MME and MME determines that the UE needs to move to the 5G system, the MME shall reject the Attach Request and send an appropriate EMM cause indicating the target CN system to the UE. After, the UE shall disable S1 mode and re-enable the N1 mode if it was disabled and perform registration in the target 5G system.

Similar, in case of 5GC, the AMF will send a Registration Reject with a cause indicating to the UE to register in the EPC network. Upon the indication, the UE shall disable the N1 mode and re-enable the S1 mode if was disabled.

6.42.3	Support of EPC interworking

TS 23.501 [5], clause 5.17 applies when a registered/connected UE(s) is moved to 5GC or EPC systems.

6.42.4	Procedures

6.42.4.1	Redirection using NAS indication

6.42.4.1.1	General

When a UE triggered registration either in EPC or 5GC network, network redirects the UE to target system by indication in the NAS layer.

6.42.4.1.2	Redirection from EPC to 5GC network

Figure 6.42.4.1.2-1: Redirection from EPC to 5GC network

1.	The UE sends the Attach or TAU Request as step#1-2 of Figure 5.3.2.1-1 in TS 23.401 [4]. The UE includes an indication that it supports N1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports N1 mode and that Preferred Network Behaviour, the MME knows the UE can be redirected to 5GC.

2.	If the MME decides to redirect the UE to the 5GC network, then the MME rejects the UE's attach request and sends an Attach / TAU Reject with appropriate EMM cause to indicating the UE to be registered in the 5GC network.

3.	Upon target CN type indication in the Attach Reject the UE disables S1 mode and re-enables the N1 mode if disabled, and the UE starts the 5GC Registration procedure, see Figure 4.2.2.2.2-1 in TS 23.502 [7].

If the UE cannot find a cell supporting connectivity to 5GC, the UE re-enables the S1 mode and attempts to Attach to EPC. The UE includes the “Inter-CN Rsedirection” indication in the Attach request indicating that the UE has been redirected from EPC to 5GC. Based on the indication, the MME ceases the redirections to 5GC for this UE.

4.	After, the 5GC will accept the UE registration as described in the Figure 4.2.2.2.2-1 in TS 23.502 [7].

6.42.4.1.3	Redirection from 5GC to EPC network

Figure 6.42.4.1.3-1: Redirection from 5GC to EPC network

1.	The UE sends the Registration Request as step#1-3 of Figure 4.2.2.2.2-1 in TS 23.502 [7]. This can be initial, mobility or periodic registration. The UE includes an indication that it supports S1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports S1 mode and the Preferred Network Behaviour, the AMF knows the UE can be redirected to EPC.

2.	AMF decides to redirect the UE to the EPC, the AMF rejects the UE's Registration Request and sends a Registration Reject with appropriate MM cause to indicating to the UE to attempt a registration in the EPC network.

3.	Upon CN type indication in the Registration Reject, the UE disables the N1 mode and re-enables the S1 mode if disabled, and the UE starts the EPC Attach or TAU procedure, see Figure 5.3.2.1-1 in TS 23.401 [4].

If the UE cannot find a cell supporting connectivity to EPC, the UE re-enables the N1 mode and attempts to Attach to 5GC. The UE includes the “Inter-CN Redirection” indication in the Registration Request indicating that the UE has been redirected from 5GC to EPC. Based on the indication, the AMF ceases the redirections to EPC for this UE.

4.	After, the EPC will accept the UE Attach/TAU as described in the Figure 5.3.2.1-1 in TS 23.401 [4].

6.42.4.1.4	Interaction with Steering of Roaming

If the AMF rejects the registration request and redirects the UE to EPC, the steering of roaming as described in Annex C in TS 23.122 [21] cannot be applied. In this case the UE may be redirected to EPC as described in this solution.

6.42.5	Impacts on existing entities and interfaces

EPC impacts:

-	If a MME determines that the UE needs to be redirected to other system, then MME needs to use an appropriate EMM cause and notify to the UE in NAS layer.

5GC impacts:

-	AMF uses an appropriate EMM cause and notifies to the UE over NAS.

UE impacts:

-	UE needs to understand the NAS rejection value and performs actions accordingly, i.e. disable the S1 or N1 mode.

-	The UE needs to indicate the “Inter-CN Redirection” indicator in Attach/Tracking Area Update and Registration Requests, when it failed to connect to the target CN and returns back to the source CN.

6.42.6	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

6.43	Solution 43: RRC based Redirection between Core networks

6.43.1	Introduction

This RRC based solution applies to Key issue 15 of load balancing the UEs between the two systems.

6.43.2	Functional Description

This solution provides a procedure to redirect the UE between the 5GC or EPC by the network over a RRC notification.

Proposed to re-use the "target CN indication" in the N2 release message by the AMF (in Rel-15 specified for Emergency Services) for the load balancing purpose. In this case, when the AMF is notified (e.g. by O&M) or determines based on local policy, and UE’s Preferred Network Behaviour, then the AMF triggers N2 release with an indication to redirect the UE to the target CN. After, the UE performs Attach/TAU procedure in EPC. Based on the received UE context the MME sets a 5GC forbidden indicator in the local MM Context.

“5GC forbidden” is a new indication set locally in the serving MME/AMF, and not carried via N26 or S10 in the UE context. If the UE changes the serving node, the “5GC forbidden” needs to be determined again in the target MME/AMF based on the existing information in the UE context, e.g. RFSP Index and handover restrictions.

In addition, similar indication needs to be introduced in the S1-AP interface i.e. MME will indicate the RAN to perform the redirection of a UE to the target 5GC system which will be notified to the UE in the RRC release. In fact, similar mechanism is defined in TS 36.413 [22] (cause IE "Load Balancing TAU Required") for load balancing between MMEs within the EPC. Similar, upon determining (e.g. by O&M notification or by local policy, and UE’s Preferred Network Behaviour) by the MME that the UE(s) needs to move to target network, then MME will trigger UE context release command with a cause IE i.e. target CN information (i.e. 5GC) to the RAN which is notified to the UE in the RRC release, thus UE can perform the registration procedure in the target CN system (i.e. 5GC). In addition, MME may provide the RFSP Index and handover restriction list to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in 5GC when it returns to Idle mode, and to prevent handovers to 5GC in Connected mode. After, the UE performs the registration procedure in the 5G system. Based on received UE context the AMF sets the EPC forbidden indicator in the local MM context.

NOTE: RFSP/SPID has not been specified for NB-IoT UEs.

Editor's note:	If N26 interface doesn’t exist, it is FFS whether a “inter-CN Redirection” indication will be required for the target CN to know the UE is redirected.

6.43.3	Support of EPC interworking

TS 23.501 [5], clause 5.17 applies when a registered/connected UE(s) is moved to 5GC or EPC systems.

6.43.4	Procedures

6.43.4.1	Redirection using RRC indication

6.43.4.1.1	General

The network will redirect the UEs to the target system based on load balancing determination as below.

6.43.4.1.2	Redirection from EPC to 5GC network

Figure 6.43.4.1.2-1: Redirection from EPC to 5GC network

0.	A UE is attached to the EPC network. As part of Attach / TAU procedure, the UE includes an indication that it supports N1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports N1 mode and the Preferred Network Behaviour, the MME knows the UE can be redirected to 5GC.

1.	If the MME has determined that the UE need to be redirected to the 5GC, when the MME releases the S1 context, the MME includes an indication of the target CN information (i.e. 5GC) to the Initial Context Release command.

In addition, MME may provide the RFSP Index to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in 5GC when it returns to Idle mode.

NOTE 1:	Defining the appropriate cause value of indication is up to the Stage 3.

2.	RAN node will release the RRC connection with an indication of the target core network type and idle-mode camping policies.

3.	RAN node sends the Release complete to the MME.

4.	After the RRC release, the UE attempts to perform a registration procedure to the 5G system. 

The AS layer in the UE informs the NAS about the previously received redirection to target CN type. If the UE finds a cell supporting connection to 5GC, NAS layer sends a Registration Request to 5GC based on the target CN type from the AS layer.

The AMF gets the UE context from EPC as described in the clause 4.11 in TS 23.502 [7], if the N26 interface exists.

5.	The AMF sends the Registration Accept to the UE. Afterwards, based on the received UE context via N26,the AMF may provide the handover restriction list to the RAN which prevents the handovers to EPC.

6.43.4.1.3	Redirection from 5GC to EPC network

Figure 6.43.4.1.3-1: Redirection from 5GC to EPC network

0.	A UE is registered in the 5G system. As part of Registration procedure, the UE includes an indication that it supports S1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports S1 mode and that Preferred Network Behaviour, the AMF knows the UE can be redirected to EPC.

1.	If the AMF has determined that the UE need to be redirected to the EPC, when the AMF releases the N2 connection, the AMF includes an indication of the target CN information (i.e. EPC) to the N2 release command. 

In addition, AMF may provide the RFSP Index to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in EPC when it returns to Idle mode.

2.	RAN triggers RRC release procedure includes target CN indicated received in N2 procedure is conveyed to the UE.

3.	RAN sends N2 release acknowledgement to the AMF

4.	Based on the RRC indication, the UE attempts to perform the registration procedure i.e. mobility Attach or TAU procedure in EPC. 

The AS layer in the UE informs the NAS about the previously received redirection to target CN type.  If the UE finds a cell supporting connection to EPC, the NAS layer sends an Attach or TAU Request to EPC based on the target CN type from the AS layer.

The MME retrieves the UE context from 5GC as described in the clause 4.11 in TS 23.502 [7], if the N26 interface exists.

5.	After, the MME sends the Attach/TAU Accept message to the UE. Afterwards, based on the received UE context via N26 the MME sets the 5GC forbidden indicator and may provide the handover restriction list to the RAN which prevents the handovers to the 5GC.

6.43.5	Impacts on existing entities and interfaces

EPC impacts:

-	If a MME determines that the UE needs to be redirected to other system, then the MME needs to create an appropriate cause and notify to the RAN in S1-AP.

-	RAN needs to provide the redirection indication in RRC to the UE.

-	When the MME receives the Attach/TAU request message the MME sets the 5GC forbidden indicator based on the UE context received via N26.

5GC impacts:

-	If an AMF determines that the UE needs to be redirected to the other CN type, then the AMF needs to create an appropriate cause and notify to the RAN in N2.

-	RAN needs to provide the redirection indication in RRC to the UE.

-	When the AMF receives the Registration request message, the AMF sets the EPC forbidden indicator based on the received UE context via N26.

UE Impacts:

-	The UE needs to understand redirection indication and perform actions accordingly. The existing redirection indication in RRC Connection Release (cnType) in WB-E-UTRAN can be reused and a similar indication needs to be added for NB-IoT.

-	The AS layer informs NAS about the redirection.

NOTE:	Defining the appropriate cause value of indication is up to the Stage 3.

.

6.43.6	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*** End of changes ***