3GPP TSG-SA WG2 Meeting #132 	S2-1903422

# 8 April – 12 April, 2019, Xi’an, China

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1135 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction regarding legacy UE and non-NPN UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. Two types of UEs need to be considered: - Legacy UE: unlike CSG which was introduced in the first release of EPS, CAG is not introduced in the first release of 5GS. I.e., legacy R-15 UE does not understand CAG. - Non-NPN-capable UE: UEs not supporing NPN feature is neither required to understand CAG. It is undesirable for Non-NPN UE to implement functions designed for NPN feature. In section 8.1 of TR23.734, the conclusion for key issue 1 and 2 indicates solution 1/2. Solution 1/2 describes the use of “cellReservedForOtherUse” to prevent access from Non-NPN-capable UE. By use of “cellReservedForOtherUse”, legacy UE and Non-NPN-capable UE can be prevented from accessing cells dedicated to NPN For Non-NPN-capable UE, the use of ‘cellReservedForOtherUse” is applicable for both Standalone NPN or PLMN integrated NPN. For NPN-capable UE, SNPN ID or CAG can be used to prevent access to non-authorized NPN. 2 Legacy UE or Non-NPN-capable UE should avoid camping on a NPN-dedicated cells. Thus, the information to prevent these UEs from camping on NPN-dedicated cells should not be optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that: - Access from Legacy UE and Non-NPN-capable UE are prevented by NG-RAN e.g. using CellReservedForOtherUse. And this information is not optional to prevent service interruption to these UE. - CAG ID is used to prevent NPN-capalbe UE’s access to not-authorized NPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non-NPN-capable UEs are unnecessarily required to implement some NPN-related functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.1, 5.30.2.2, 5.30.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changes >>>

## 5.30	Support for non-public networks

### 5.30.1	General

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2]. An NPN may be deployed as:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, or

-	a Public network integrated NPN, i.e. a non-public network deployed with the support of a PLMN.

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3 and the additional functionality covered in clause 5.30.2.

Public network integrated NPNs can be enabled using network slicing (see Annex D).

To prevent UEs not supporting NPN functionality from trying to access cells dedicated to a NPN, NG-RAN, as described in TS 38.331 [28] and in TS 38.304 [50], can broadcast information, e.g. using cellReservedForOtherUse. To prevent UEs supporting a NPN from trying to access a unauthorized NPN, the Closed Access Group (CAG) functionality or the combination of PLMN IN and NID identifying an SNPN can be used in addition.

### 5.30.2	Stand-alone non-public networks

#### 5.30.2.1	Identifiers

The combination of a PLMN ID and Network identifier (NID) identifies an SNPN.

NOTE 1:	The PLMN ID used for SNPNs is not required to be unique. PLMN IDs reserved for use by private networks can be used for non-public networks, e.g. based on mobile country code (MCC) 999 as assigned by ITU [78]).

The NID shall support two assignment models:

-	Locally managed NIDs are assumed to be chosen individually by SNPNs at deployment time (and may therefore not be unique)

-	Universally managed NIDs are assumed to be globally unique.

NOTE 2:	Which legal entities manage the number space is beyond the scope of this specification.

An optional human-readable network name helps to identify an SNPN during manual SNPN selection.

#### 5.30.2.2	Broadcast system information

NG-RAN nodes which provide access to SNPNs broadcast the following information:

-	One or multiple PLMN IDs

NOTE 1:	The PLMN ID used for SNPNs is not required to be globally unique. Non-unique PLMN IDs reserved for private networks can be used, e.g. based on mobile country code 999 as assigned by ITU [78]).

-	List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to

NOTE 2:	It is assumed that an NG-RAN node supports broadcasting a total of twelve NIDs. Further details are defined in TS 38.331 [28].

NOTE°3:	The presence of a list of NIDs for a PLMN ID indicates that the related PLMN ID and NIDs identify SNPNs.

-	Optionally a human-readable network name per NID.

NOTE 4:	The human-readable network name per NID is only used for manual SNPN selection. The mechanism how human-readable network name is provided (i.e. whether it is broadcasted or unicasted) to the UE is specified in TS 38.331 [28].

-	Information, as described in TS 38.331 [28] and in TS 38.304 [50], to prevent UEs not supporting SNPNs from accessing the cell

#### 5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with subscriber identifiers and credentials for one or multiple SNPNs identified by the combination of PLMN ID and NID.

A subscriber of an SNPN is identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.2.2. The realm part of the NAI may include the NID of the SNPN.

An SNPN-enabled UE supports the SNPN access mode. When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.

Emergency services are not supported in SNPN access mode.

NOTE 1:	Voice support with emergency services in SNPN access mode is not specified in this release.

If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in in clause 4.4 of TS 23.122 [17].

NOTE 2:	Details of activation and deactivation of SNPN access mode are up to UE implementation.

#### 5.30.2.4	Network selection in SNPN access mode

When the UE is set to operate in SNPN access mode the UE does not perform normal PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17].

UEs operating in SNPN access mode read the available PLMN IDs and list of available NIDs from the broadcast system information and take them into account during network selection.

For automatic network selection, the UE selects and attempts to register with the available SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials. If multiple SNPNs are available that the UE has SUPI and credentials for, then the priority order for selecting and attempting to register with SNPNs is based on UE implementation.

For manual network selection UEs operating in SNPN access mode provide to the user the list of NIDs and related human-readable names (if available) of the available SNPNs the UE has SUPI and credentials for.

NOTE:	The details of SNPN selection will be defined in TS 23.122 [17].

When a UE performs Initial Registration to an SNPN, the UE shall indicate the selected NID and the corresponding PLMN ID to NG-RAN. NG-RAN shall inform the AMF of the selected PLMN ID and NID.

#### 5.30.2.5	Network access control

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a locally managed NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to temporarily prevent the UE from automatically selecting and registering with the same SNPN.

If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a universally managed NID and there is no subscription for the UE, then the AMF shall reject the UE with an appropriate cause code to permanently prevent the UE from automatically selecting and registering with the same SNPN.

NOTE:	The details of rejection and cause codes will be defined in TS 24.501 [47].

In order to prevent access to SNPNs for authorized UE(s) in case of network congestion/overload, unified access control information is configured per non-public network (i.e. as part of the subscription information that the UE has for a given non-public network).

#### 5.30.2.6	Cell (re-)selection in SNPN access mode

UEs operating in SNPN access mode only select cells and networks broadcasting both PLMN ID and NID of the selected SNPN.

NOTE:	Further details on the NR idle mode procedures for SNPN cell selection will be defined in TS 38.331 [28] and in TS 38.304 [50].

#### 5.30.2.7	Access to PLMN services via stand-alone non-public networks

To access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN with a PLMN (using the credentials of that PLMN) following the same architectural principles as specified in clause 4.2.8 and the SNPN taking the role of "Untrusted non-3GPP access".

Editor's note:	The need to support QoS differentiation for access to PLMN services via non-public networks is FFS.

#### 5.30.2.8	Access to stand-alone non-public network services via PLMN

To access SNPN services, a UE that has successfully registered with a PLMN may perform another registration via the PLMN with an SNPN (using the credentials of that SNPN) following the same architectural principles as specified in clause 4.2.8 and the PLMN taking the role of "Untrusted non-3GPP access".

Editor's note:	The need to support QoS differentiation for access to SNPN services via PLMN is FFS.

### 5.30.3	Public network integrated NPN

#### 5.30.3.1	General

Public network integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. The existing network slicing functionalities apply as described in clause 5.15. When an NPN is made available via a PLMN, then the UE has a subscription for the PLMN.

NOTE 1:	Annex D provides additional consideration to consider when supporting Non-Public Network as a Network Slice of a PLMN.

As network slicing does not enable the possibility to prevent UEs from trying to access the network in areas which the UE is not allowed to use the Network Slice allocated for the NPN, Closed Access Groups may optionally be used in addition to network slicing to apply access control.

NOTE 2:	To prevent UEs not supporting NPN functionality from trying to access cells dedicated to a NPN, NG-RAN, as described in TS 38.331 [28] and in TS 38.304 [50], can broadcast information, e.g. indicating cellReservedForOtherUse.

A Closed Access Group identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG.

CAG is used for the Public network integrated NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated cell(s).

The following clauses describes the functionality needed for supporting CAGs.

Editor's note:	It is FFS whether Emergency services are to be supported in a CAG cell.

#### 5.30.3.2	Identifiers

The following is required for identification:

-	A CAG is identified by a CAG Identifier which is unique within the scope of a PLMN ID;

-	A CAG cell broadcasts one or multiple CAG Identifiers per PLMN;

NOTE 1:	It is assumed that an NG-RAN node supports broadcasting a total of twelve CAG Identifiers. Further details are defined in TS 38.331 [28].

-	A CAG cell may in addition broadcast a human-readable network name per CAG Identifier:

NOTE 2:	The human-readable network name per CAG Identifier is only used for presentation to user when user requests a manual CAG selection.

#### 5.30.3.3	UE configuration and subscription aspects

To support CAG, the UE is configured, using the UE Configuration Update procedure for access and mobility management related parameters described in TS 23.502 [3] in clause 4.2.4.2, with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	optionally, an indication whether the UE is only allowed to access 5GS via CAG cells (see TS 38.304 [50] for how the UE identifies whether a cell is a CAG cell);

#### 5.30.3.4	Network and cell (re-)selection, and access control

The following is assumed for network and cell selection, and access control:

-	The CAG cell shall broadcast information such that only UEs supporting CAG are accessing the cell;

NOTE:	The above also implies that cells are either CAG cells or normal PLMN cells.

-	For aspects of automatic and manual network selection in relation to CAG, see TS 23.122 [17];

-	For aspects related to cell (re-)selection, see TS 38.304 [50];

-	The Mobility Restrictions shall be able to restrict the UE's mobility according to the Allowed CAG list (if configured in the subscription) and include an indication whether the UE is only allowed to access CAG cells (if configured in the subscription);

-	During transition from CM-IDLE to CM-CONNECTED, if the UE is accessing the 5GS via a CAG cell, the NG-RAN shall provide the CAG Identifier to the AMF:

-	The AMF shall verify whether UE access is allowed by Mobility Restrictions:

-	If the CAG Identifier received from the NG-RAN is part of the UE's Allowed CAG list, then the AMF accepts the NAS request;

-	If the CAG Identifier received from the NG-RAN is not part of the UE's Allowed CAG list, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE removes that CAG Identifier, if it exists, from its Allowed CAG list, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the network via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request with an appropriate cause code, whereas the UE updates its local configuration, as defined in TS 24.501 [47]. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

-	During connected mode mobility procedures:

-	Based on the Mobility Restrictions received from the AMF:

-	Source NG-RAN shall not handover the UE to a target NG-RAN node if the target is a CAG cell and the related CAG Identifier is not part of the UE's Allowed CAG list;

-	Source NG-RAN shall not handover the UE to a non-CAG cell if the UE is only allowed to access CAG cells;

-	Update of Mobility Restrictions:

-	When the AMF receives the Nudm_SDM_Notification from the UDM and the AMF determines that the Allowed CAG list or the indication whether the UE is only allowed to access CAG cells have changed;

-	The AMF shall update the Mobility Restrictions in the UE and NG-RAN accordingly; and

-	If the UE is currently accessing a CAG cell and the related CAG Identifier has been removed from the Allowed CAG list or if the UE is currently accessing a non-CAG cell and the indication that the UE is only allowed to access CAG cells has been set in the subscription, then the AMF shall release the NAS signalling connection for the UE by triggering the AN release procedure.

<<< End of changes >>>