SA WG2 Meeting #133 	S2-1905956

# 13 - 17 May 2019, Reno, Nevada, USA					(revision of S2-1905832, S2-1905447, S2-1903319)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1102 | rev | 3 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to access network performance measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2019-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The details for the addresses of the PMF in the UPF, which are sent to the UE, are not specified. There is an editor's note related to whether a PMF session between the UE and the UPF should be established. This note can be removed as there is no need for any PMF session as such. For Access Availability/Unavailability Report, the UPF shall acknowledge the PMF-Access Report received from the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The details for the addresses of the PMF in the UPF, which are sent to the UE, are specified. The editor's note related to whether a PMF session between the UE and the UPF should be established is removed. For Access Availability/Unavailability Report, the UPF shall acknowledge the PMF-Access Report received from the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PMF functionality will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.5.1, 5.32.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.32.5	Access Network Performance Measurements

#### 5.32.5.1	General principles

When the UE requests an MA PDU Session Establishment and indicates it is capable to support ATSSS, the UE may receive Measurement Assistance Information when the MA PDU session is accepted by the network. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether measurement reports need to be sent to the network.

Measurement Assistance Information shall include the addressing information of a Performance Measurement Function (PMF) in the UPF, with which the UE can send PMF protocol messages:

-	For IP PDU session type, Measurement Assistance Information contains one IP address for the PMF, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access;

-	For Ethernet PDU session type, Measurement Assistance Information contains one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages to allow Round Trip Time (RTT) measurements, i.e. when the "Smallest Delay" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF.

When the UE requests a MA PDU session and indicates it is capable to support the MPTCP functionality only, the network should not send Measurement Assistance Information as in this case the UE can use measurements available at the MPTCP layer.

#### 5.32.5.3	Access Availability/Unavailability Report

If required by the network in the Measurement Assistance Information, the detection of the unavailability and of the availability of an access, as well as the decision of reporting it, can be performed by the UE, based on implementation. When the UE has decided, based on implementation, to report the unavailability/availability of an access, it shall:

-	build a PMF-Access Report containing the access type and an indication of availability/unavailability of this access;

-	send the PMF-Access Report to the UPF via the user plane.

The UPF shall acknowledge the PMF-Access Report received from the UE.

END OF CHANGES