3GPP TSG-SA2 Meeting #136-AH	S2-2000224

# Incheon, Korea (Republic Of), 13th Jan 2020 - 17th Jan 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1961 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Scope parameter for binding indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has already been agreed that an NF service consumer may subscribe via another network function for notifications. And that two binding indications are then required. For example, NF_A may subscribe to NF_B on behalf of NF_C. NF_A additionally subscribe to subscription related events. Further, the possibility to provide NF service instance as part of the binding has been removed at the previous meeting because it was unclear to which service the instance ID would relate to. The possibility to allow for a service level binding seems desirable to comply with the overall principles of binding. At the previous meeting, the possibility to indicate the service a binding relates to was already introduced. But this is restricted to notifications. As the binding for notifications and the binding for other services could be described in the same request message, and even binding for several different other services could be provided, those cases need to be kept apart for unambiguous signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The binding indication is extended with a scope parameter that can indicate for a binding indication provided by the service consumer whether that binding indication relates to notifications to subscription events, to notification to other events, or to other services that the NF service consumer produces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The semantics of binding indications provided by the NF service consumer are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.17.12.3, 4.17.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 4.17.12.3	Binding created as part of service request

If the NF service consumer can also be as a NF service producer for later communication from the contacted producer, a service request sent to the producer may include binding indication.

NOTE:	This clause only applies to the AMF as NF service consumer sending requests to the SMF unless further usage has been defined in stage 3. Implicit subscriptions are not described in this clause.

Figure 4.17.12.3-1: Binding created as part of service request

1.	Instance A, as an NF service consumer sends a service request using either Direct Communication or Indirect communication via SCP and Instance B is selected as NF service producer. If Instance A can also be NF service producer for later communication for the concerned data context, it may include binding indication referring to NF service instance, NF service set, NF instance or NF Set as specified in Table 6.3.1.0-1 of TS 23.501 [2] in the request sent to the NF service producer; if NF service instance or NF service set are provided, the binding indication shall indicate "other service" as scope and include the service name. In this case, if indirect communication is used, the SCP sends to the Instance B the service request including the binding indication.

2.	Instance B as the NF service producer sends a response to the NF service consumer.

3.	When Instance B as NF service consumer needs to invoke the service provided by Instance A, Instance B sends a request using the binding indication received in step 1 as described in Steps 3-4 in Figure 4.17.12.2-1 with the following difference:

-	Based on the received binding indication, if delegated discovery is not used, the Instance B may need to discover the corresponding endpoint address of the Instance A.

2nd change

#### 4.17.12.4	Binding for subscription requests

Binding for notifications can be created as part of an explicit or implicit subscription request. In this case, illustrated in Figure 4.17.12.4-1, the subscription request may include a Binding Indication 1 referring to NF service instance, NF service Set, NF instance or NF Set and a service name of the NF service consumer as specified in Table 6.3.1.0-1 of TS 23.501 [2]. The NF Service Set ID, NF service instance ID, and service name relate to the service of a NF service consumer that will handle the notification.

For direct communication, the NF service producer selects the target for the related notifications using the received Binding Indication as specified in Table 6.3.1.0-1 of TS 23.501 [2].

For indirect communication, the NF service producer includes a Routing Binding Indication with the same contents as the received Binding Indication and the SCP selects the target for the related notifications using the received Routing Binding Indication as specified in Table 6.3.1.0-1 of TS 23.501 [2].

If the Binding Indication for Notifications needs to be updated, the NF service consumer may initiate a new Subsription request to the NF service producer with an updated Binding Indication or may include the Binding Indication in the acknowledment of a Notification. A Subsription request may also contain updated Notification Correlation ID and Notification Target Address.

Binding for the subscription resource at the NF service producer can also be created: the Subscription Response message may contain an a Binding Indication 2 referring to NF service instance, NF instance or NF Set of the NF service producer.

For direct communication, the NF service consumer selects the target for the related request to the producer, such as the request to update the subscription shown in Figure 4.17.12.4-1, using the received Binding Indication 2 as specified in Table 6.3.1.0-1 of TS 23.501 [2].

For indirect communication, the NF service consumer includes a Routing Binding Indication with the same contents as the received Binding Indication 2 and the SCP selects the target for the related request using the received Routing Binding Indication 2 as specified in Table 6.3.1.0-1 of TS 23.501 [2].

Figure 4.17.12.4-1: Binding in a subscription request

Figure 4.17.12.4-2: Binding during subscription via another network function

An NF service consumer may subscribe via another network function. For example, NF_A may subscribe to NF_B on behalf of NF_C. NF_A additionally subscribe to subscription related events. In this case, both the binding indication from NF_C and NF_A are provided to the NF service producer NF_B. The Binding Indication for notifications to subscription related events shall include a scope parameter indicating "subscription events".

The NF_C's binding indication is used for reselection of a notification endpoint, which is used for event notification. The NF_A's binding indication is used for reselection of a notification endpoint, which is used for subscription change event notification.

End of changes