SA WG2 Meeting #136AH 	S2-2000912

# 13-17 Janurary 2020, Incheon, KR

Source:	InterDigital

Title:	New solutions for Key Issue #5: Providing network configuration recommendations

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution:

# Discussion

Current observed service experience related network data analytics mechanisms do not include a procedure to easily leverage NWDAF statistics and predictions to improve the service experience. While a NF can obtain information about the service experience, it cannot request

- a recommendation of a QoS profile or any network configuration parameters to achieve a target service experience;

- an estimate of the service experience that would result from a modification of the QoS profile or any network configuration parameters.

# Proposal

This document proposes to add into TR 23.700-91 a new Solution for Key Issue #5: New types of outputs provided by NWDAF.

FIRST CHANGE – ALL NEW TEXT

## 6.0 	Mapping Solutions to Key Issues

## 6.X 	Solution for Key Issue #5: Providing network configuration recommendations

### 6.X.1 	Description

### 6.X.1.1 	Services Principles

This solution is for Key Issue #5: New types of outputs provided by NWDAF.

In the first mechanism, the NWDAF is queried with a target service experience that the NF would like to achieve. The NWDAF answers with a suggested set of configuration parameters (such as QoS profiles, RAT and/or frequency parameters) that the NF may apply to reach the target service experience.

In the second mechanism the NWDAF is queried by a NF with a potential modification of a set of configuration parameters (such as QoS profiles, RAT and/or frequency parameters). The NWDAF answers with an estimation of the service experience that would result from this modification.

### 6.X.1.2 	Services Procedures

### 6.X.1.2.1 Requesting network configuration parameters for target service experience

The NF provides a target service experience for an application and the NWDAF answers with a set of network configuration parameters. This is detailed below:

- Consumer NF sends a Recommendation request by means of a Nnwdaf_ConfigurationRecommendation_Subscribe or Nnwdaf_ConfigurationRecommendation_Request to NWDAF, specifying the target of the requests in terms of network configuration parameters and UEs/ application ID.

- The NWDAF suggests values for the requested parameters that is estimated to achieve the requested Service Experience to the consumer NF by a Nnwdaf_ ConfigurationRecommendation_Notify or Nnwdaf_ ConfigurationRecommendation_Response. This answer also indicates how well the proposed parameters are estimated to satisfy the request.

Table: request input

| Information | Description |
| --- | --- |
| Request filter information | Application ID, list of UEs, time window, S-NSSAI, DNN, Area of Interest etc |
| Target service experience | Desired QoE for the application |
| Configuration parameters | List of network parameters for which a recommendation is requested |

Table: request output

| Information | Description |
| --- | --- |
| Estimated service experience | Service experience that would result from following the recommendation |
| Spatial validity condition | Where the estimated Service Experience applies |
| Time validity condition | When the estimated Service Experience applies |
| Recommendation | Recommended values for each parameter, UE etc. |

### 6.X.1.2.2 Requesting service experience estimate that would result from a new network configuration

The NF requests a service experience estimate by providing a set of network configuration parameters. Then, the NWDAF answers with a service experience estimate for the configuration sent by the NF. This is detailed and illustrated below:

- Consumer NF sends a configuration evaluation request by means of a Nnwdaf_ ConfigurationEvaluation_Subscribe or Nnwdaf_ ConfigurationEvaluation_Request to NWDAF.

- The NWDAF provides the estimated Service Experience (that would result from applying the input configuration) to the consumer NF by means of Nnwdaf_ ConfigurationEvaluation_Notify or Nnwdaf_ ConfigurationEvaluation_Response.

Table: request input

| Information | Description |
| --- | --- |
| Request filter information | Application ID, list of UEs, time window, S-NSSAI, DNN, Area of Interest etc |
| Configuration | Values for each parameter, UE etc that constitute the configuration that must be evaluated. |

Table: request output

| Information | Description |
| --- | --- |
| Estimated service experience | Estimated service experience that would result from applying the recommendation. This can be a range. |
| Spatial validity condition | Where the estimated Service Experience estimation applies |
| Time validity condition | When the estimated Service Experience estimation applies |

### 6.X.2 	Impacts on Existing Nodes and Functionality

This solution adds a new interface to service experience NWDAF.

END OF CHANGES