SA WG2 Meeting #136AH 	S2-2001500

13 - 17 January 2020, Incheon, South Korea	 (resubmission of S2-2001443)

Source:	CATT

Title:	Solution for ProSe QoS

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes a solution for ProSe QoS to address KI#2 QoS aspect of TR 23.752.

1	Introduction

It was agreed at SA2#135 meeting that the 5G ProSe QoS shall take Rel-16 V2X QoS as baseline:

-	QoS handling for V2X communication over NR PC5 reference point defined in clauses 5.4.1 to 5.4.4 of TS 23.287 [5] is used as basis for supporting PC5 direct communication.

-	The standardized PQI values defined for NR PC5 in TS 23.287 [5] can be used to support performance requirements defined in clause 7.6.2 of TS 22.261 [3] for interactive services if applicable.

and furthermore in KI#2:

-	For QoS support differences with what is documented in TS 23.287 [5] clause 5.4 will be documented.

In this contribution, the ProSe QoS solution is proposed in which the differences with TS 23.287 clause 5.4 are highlighted.

2	Proposal

It is proposed to capture the following solution into TR 23.752.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  | X |
| 3 | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  |  |  |  |  |
| 5 |  | X |  |  |  |  |  |  |
| 6 |  |  | X |  |  |  |  |  |
| 7 |  |  | X |  |  |  |  | X |
| X |  | x |  |  |  |  |  |  |

Second CHANGE

6.X	Solution #X: QoS handling for ProSe communication

6.X.1	General Description

In order to support QoS handling for ProSe communication, the mechanism defined in TS 23.287 [y] clause 5.4 is reused with the following differences:

-	V2X Packet Filter Set is replaced by ProSe Packet Filter Set.

Editor's note: It is FFS which filter combination the ProSe Packet Filter Set is based on, e.g. Source/Destination Layer-2 ID, ProSe Application ID, etc.

For public safety services, considering the higher priority and higher reliability, new PQI values needs to be defined. The one-to-one mapping of standardized PQI values to PC5 QoS characteristics are defined as followings:

Table 6.X.1-1: Proposed Standardized PQIs to QoS characteristics mapping (changes with respect to TS 23.287 [5] Table 5.4.4-1 in Bold-Italic text)

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | GBR | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| New value#2 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| New value#3 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| New value#4 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| New value#5 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |

Editor's note: Whether the new defined PQI values can be supported by RAN needs to be checked with RAN WGs.

6.X.2	Procedures

The QoS handling defined in clauses 5.4.1.2 to 5.4.1.4 of TS 23.287 [5] is used.

6.X.3	Impacts on Existing Nodes and Functionality

UE impact:

-	UE is enhanced to support QoS handling for ProSe communication over NR based PC5 reference point.

END of CHANGES