3GPP TSG SA WG2 Meeting #136-AH	S2-2000270

13th – 17th January 2020, Incheon, KR

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1974 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exclusion of DNN in I-SMF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cisco Systems |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  |  | Date: |  | 2020-01-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For I-SMF, AMF selects and inserts an I-SMF which can serve the UE location and the S-NSSAI. DNN is not applicable for I-SMF selection (See Clause 5.34.3 in TS 23.501). When AMF selects an I-SMF, it needs to exclude DNN as a selection criteria in NRF discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that DNN is not used for NRF Discovery in I-SMF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDU Session may fail if no SMF can be found during I-SMF selection that supports the specified DNN (NRF may return empty I-SMF list based on DNN). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * * *

### 4.23.2	I-SMF selection

For non roaming or LBO roaming case, the AMF selects an SMF serving the PDU Session as described in clause 4.3.2.2.3. If the service area of the selected SMF does not control UPF that can serve the UE location, the AMF selects an I-SMF as described in clause 4.3.2.2.3.2 with UE location information as addtional input and without Data Network name (DNN) as input to NRF in step 3.

For home routed roaming case, the AMF selects V-SMF as described in clause 4.3.2.2.3.2 with UE location information as additional input and without Data Network name (DNN) as input to NRF in step 3.

When the delegated discovery is used, the SCP selects the SMF as described in Annex E.

* * * End Of Change * * * *